***Sub Engineer***

1. Systematic errors are those errors

a. which cannot be recognised

b. whose character is understood

c. whose effects are cumulative and can be eliminated

d. none of these.

2. The representation of general topography of a very flat terrain is possible only

a. by drawing contours at large interval

b. by drawing contours at small interval

c. by giving spot levels at large interval

d. by giving spot levels to salient features at close interval.

3. If deflection angles are measured in a closed traverse, the difference between the sum of the right-hand and that of the left hand angles should be equal to

a. 0°

b. 90°

c. 180°

d. 360°

4. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

5. The rocks having alumina or clay as their major constituents, are known as

a. argillaceous rocks

b. calcareous rocks

c. sedimentary rocks

d. igneous rocks.

6. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

7. Vanadium steel is generally used for

a. railway switches and crossing

b. bearing balls

c. magnets

d. axles and springs.

8. PVC stands for

a. plastic very compact

b. polythene vinyl chloride

c. polythene vinyl carbon

d. polythene vanadium carbide.

9. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

10. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

11. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

12. A simply supported beam (l + 2a) with equal overhangs (a) carries a uniformly distributed load over the whole length, the B.M. changes sign if

a. l > 2a

b. l < 2a

c. l = 2a

d. l = 4a

13. Pick up the correct statement from the following :

a. In incompressible flow the density of a fluid remains constant

b. In compressible flow, the density of a fluid changes from point to point

c. In uniform flow, the velocity of a fluid does not change with respect to length of flow direction

d. All the above.

14. For most economical rectangular section of a channel, the depth is kept

a. three times the hydraulic radius

b. half the width

c. hydraulic mean depth

d. none of these.

15. Mercury is generally used in barometers because

a. the height of the barometer will be less

b. it is a best liquid

c. both (a) and (b) above

d. both (b) and (c) above.

16. The ratio of inertia force of a flowing fluid and the viscous force of the liquid is called :

a. Renold's number

b. Froude's number

c. Euler's number

d. Weber's number.

17. A soil has bulk density 2.30 g/cm3 and water content 15 per cent, the dry density of the sample, is

a. 1.0 g/cm2

b. 1.5 g/cm3

c. 2.0 g/cm3

d. 2.5 g/cm3

18. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

19. The earth pressure of a soil at rest, is proportional to (where φ is the angle of internal friction of the soil)

a. tan (45° + φ)

b. tan2 (45° - φ)

c. tan2 (45° + φ)

d. none of these

20. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

21. A foundation rests on

a. base of the foundation

b. subgrade

c. foundation soil

d. both (b) and (c)

22. The allowable tensile stress in mild steel stirrups, reinforced cement concrete, is

a. 1400 kg/cm2

b. 190 kg/cm2

c. 260 kg/cm2

d. 230 kg/cm2

23. The breadth of a ribbed slab containing two bars must be between

a. 8 cm to 10 cm

b. 10 cm to 12 cm

c. 12 cm to 15 cm

d. none of these.

24. As per I.S. 456 - 1978, the pH value of water shall be

a. less than 6

b. equal to 6

c. not less than 6

d. equal to 7

25. Pile foundation is generally provided if soil is

a. compressible

b. water logged

c. made up

d. all the above.

26. The Auger borings are not common

a. in soils that require lateral support

b. in cohesive soils

c. in soft soils

d. none of the above.

27. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

28. Pick up the correct statement from the following:

a. Angular and rough aggregates reduce the workability of the concrete

b. Large size aggregates increase the workability due to lesser surface area

c. The slump of the concrete mix decreases due to an increase in temperature

d. All the above.

29. The intake opening is generally covered by a screen to prevent entry of debris etc. and its level is kept

a. at the level of water of the source

b. at the bottom of water of the source

c. at about 2.5 m above the bottom

d. none of these.

30. From the surface of reservoir, evaporation may be minimised by sprinkling

a. Hydrochloric acid

b. Cetyl alcohol

c. Methane

d. None of these.

31. In sewage having fully oxidised organic matter, the nitrogen exists in the form of

a. nitrates

b. free ammonia

c. aluminoid nitrogen

d. none of these.

32. Pick up the correct statement from the following :

a. Maximum daily flow = 2 times the average daily flow

b. Maximum daily flow =  x average daily flow

c. Sewers are designed for minimum permissible velocity at minimum flow

d. All the above.

33. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

34. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

35. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

36. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

37. Traffic engineering only includes

a. design and application of control devices

b. analysis of traffic characteristics

c. traffic opeation

d. all the above.

38. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

39. The first stage of deciding the alignment of a hill road, is

a. reconnaissance

b. detailed survey

c. trace-out

d. preliminary survey.

40. Pick up the incorrect statement from the following:

a. Tresguet did not provide the top camber for the drainage of surface water

b. Tresguet provided the top camber for the drainage of surface water

c. Telford provided two layers of stones in the central 5.4 m width and one layer was provided on the sides

d. Macadam provided a camber to the formation at the dug-up state, to drain percolated water.

41. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

42. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

43. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

44. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

45. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

46. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

47. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

48. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

49. Total correction for elevation, temperature and gradient for a runway should not be more than

a. 20%

b. 25%

c. 30%

d. 35%

50. The runway orientation is made so that landing and take off are

a. against the wind direction

b. along the wind direction

c. perpendicular to wind direction

d. none of these.