***Sub Engineer***

1. The power of a lens

a. is reciprocal of its focal length

b. is positive if it is a convex lens

c. is negative if it is a concave lens

d. is measured in diopter.

2. In a theodolite

a. the telescope axis is perpendicular to transit axis

b. the axis of rotation is perpendicular to transit axis

c. the telescope axis, the transit axis and the rotation axis pass through the centre of theodolite

d. all the above.

3. Magnetic bearing of a survey line at any place

a. remains constant

b. changes systematically

c. varies differently in different months of the year

d. is always greater than true bearing.

4. The least count of a vernier scale is

a. sum of the smallest divisions of main and vernier scales

b. value of one division of the primary scale divided by total number of divisions of vernier scale

c. value of one division of vernier scale divided by total number of divisions of primary scale

d. none of these.

5. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

6. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

7. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

8. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

9. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

10. Pick up the correct statement from the following:

a. For a uniformly distributed load, B.M. curve is a parabola

b. For a load varying linearly, the shear force curve is a parabola

c. For a load varying linearly, the B.M. curve is a cubic parabola

d. All the above.

11. A cylinder is said to be thin if the ratio of its thickness and diameter, is less than

a. 1/20

b. 1/15

c. 1/10

d. 1/5

12. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

13. The momentum correction factor (β) for the viscous flow through a circular pipe is

a. 1.33

b. 1.50

c. 1.66

d. 2.00

14. Total energy line is

a. pressure head

b. datum head

c. kinetic head

d. All the above.

15. For most economical rectangular section of a channel, the depth is kept

a. three times the hydraulic radius

b. half the width

c. hydraulic mean depth

d. none of these.

16. A spherical load 900 kg is rolled through 9.8 m across the deck of a ship weighing 10, 000 kg. If the metacentric height of the ship is 5 metres, the angle of heel, is

a. 10° 5'

b. 10° 10'

c. 10° 15'

d. 10° 20'

17. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

18. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

19. A failure wedge develops if a retaining wall

a. moves away from the backfill

b. moves towards the backfill

c. sinks downwards

d. stresses equally by vertical and horizontal forces.

20. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

21. If the ratio of the span to the overall depth does not exceed 10, the stiffness of the beam will ordinarily be satisfactory in case of a

a. simply supported beam

b. continuous beam

c. cantilever beam

d. none of these.

22. In a combined footing if shear stress does not exceed 5 kg/cm2, the nominal stirrups provided are

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

23. If the width of the foundation for two equal columns is restricted, the shape of the footing generally adopted, is

a. square

b. rectangular

c. trapezoidal

d. triangular.

24. If the size of a column is reduced above the floor, the main bars of the columns, are

a. continued up

b. bent inward at the floor level

c. stopped just below the floor level and separate lap bars provided

d. all the above.

25. In high mountaneous region, the type of roof generally recommended for buildings, is

a. shed type

b. gable type

c. gambrel type

d. mansord type.

26. A stair should not have pitch more than

a. 30°

b. 40°

c. 50°

d. 60°.

27. Grading of sand causes great variation in

a. strength of concrete

b. durability of concrete

c. handing and placing of concrete

d. all the above.

28. The entrained air in concrete

a. increases workability

b. decreases workability

c. decreases resistance to weathering

d. increases strength

29. According to IS : 1172-1963, a minimum of 135 litres of water capita per day, is required for

a. Boarding schools

b. Nurses home and medical quarters

c. hostels

d. all the above.

30. The ensure proper growth of children's teeth, the quantity of flouride used in water mains, is

a. 1 mg/litre

b. 2 mg/litre

c. 3 mg/litre

d. 4 mg/litre

31. The design period in years for pumping plants, is

a. 1

b. 2 to 3

c. 3 to 5

d. 5 to 10

32. The gas evolved in sewers is

a. Carbondioxide

b. Hydrogen sulphide

c. Methane

d. All of these.

33. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

34. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

35. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

36. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

37. The desirable camber for straight roads with thin bituminous surfacing, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

38. The steepest gradient permitted on roads which, in ordinary conditions, does not exceed, is known

a. maximum gradient

b. exceptional gradient

c. floating gradient

d. all the above.

39. While designing hair pin bends on higways, the minimum

a. gradient is 1 in 40

b. gradient is 1 in 200

c. superelevations is 1 in 10

d. All the above.

40. Deviation of the alignment of a trace cut may be permitted in areas involving

a. land slides

b. sand dunes

c. dens

d. none of these.

41. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

42. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

43. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

44. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

45. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

46. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

47. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

48. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

49. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.