***Sub Engineer***

1. The Random errors tend to accumulate proportionally to

a. numbers of operations involved

b. reciprocal of operations involved

c. square root of the number of operation involved

d. cube root of the number of operation involved.

2. While measuring with a metallic tape of 30 m length pull should be applied

a. 1 kg

b. 2 kg

c. 3 kg

d. 4 kg

3. Geodetic surveying is undertaken

a. for developing the science of geodesy

b. making use of most accurate instruments and methods of observation

c. for determination of accurate positions on the earth's surface of system of control points

d. all the above.

4. For taking offsets with an optical square on the right hand side of the chain line, it is held

a. by right hand upside down

b. by left hand upright

c. by right hand upright

d. by left hand up side down.

5. The cast iron when heated to red heat with powdered red haemetite in an oven for increasing its toughness, is converted to

a. grey cast iron

b. white cast iron

c. mottled cast iron

d. toughed cast iron.

6. The clay to be used for manufacturing bricks for a large project, is dugout and allowed to weather throughout

a. the monsoon

b. the winter

c. the summer

d. none of these.

7. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

8. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

9. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

10. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

11. The property of a material by which it can be drawn to a smaller section, due to tension, is called

a. plasticity

b. ductility

c. elasticity

d. malleability.

12. n and j are numbers of members and joints in a frame. It contains redundant members if

a. n = 3j - 2

b. n < 2j - 3

c. n < j - 2

d. n > 2j - 3

13. Pick up the correct statement from the following :

a. When the length of the tube is equal to diameter of the internal mouth piece, the jet of liquid comes out without touching the sides of the tube

b. When the length of the tube is three times the diameter of the internal mouth piece, the jet diameter is equal to diameter of the tube.

c. both (a) and (b)

d. Neither (a) nor (b).

14. Atmospheric pressure varies with

a. altitude

b. temperature

c. weather conditions

d. all of the above.

15. Back water curve is caused if

a. friction head loss is more than the bed slope

b. pressure is due to weir in the channel

c. there is an increase in width of the channel

d. none of these.

16. Which one of the following statements is true ?

a. The value of kinetic energy correction factor for turbulent flow lies between 1.03 to 1.06

b. The value of kinetic energy correction factor for laminar flow is 2

c. The practical value of kinetic energy correction factor for turbulent flow is unity

d. all the above.

17. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

18. If the failure of a finite slope occurs through the toe, it is known as

a. slope failure

b. face failure

c. base failure

d. toe failure.

19. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

20. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

21. The live load to be considered for an accessible roof, is

a. Nil

b. 75 kg/m3

c. 150 kg/m2

d. 200 kg/cm2

22. The maximum ratio of span to depth of a cantilever slab, is

a. 10

b. 12

c. 14

d. 16

23. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

24. If W is weight of a retaining wall and P is the horizontal earth pressure, the factor of safety against sliding, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

25. Raft foundations are used for :

a. providing increased area of foundation over poor bearing capacity of soil

b. spanning over small soft or loose pockets

c. counter acting the hydrostatic effect

d. all the above.

26. The line of intersection of the surfaces of a sloping roof forming an external angle exceeding 180°, is

a. ridge

b. hip

c. valley

d. none of these.

27. If 1500 g of water is required to have a cement paste 1875 g of normal consistency, the percentage of water is,

a. 25%

b. 30%

c. 35%

d. 40%

28. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

29. Pick up the correct statement from the following regarding the pressure conduits :

a. Pressure conduits are permitted to run  th full

b. Pressure conduits are always laid along down grades

c. The hydraulic gradient line always coincides the invert of the conduit

d. None of these.

30. If Q is discharge is cubicmetres per sec and D is the economical diameter of the pipe. According to Lea

a. D = 0.67 to 0.87 Q

b. D = 0.77 to 0.97 Q

c. D = 0.97 to 1.22 Q

d. D = 1.22 to 1.33 Q.

31. Aerobic activity is maximum

a. in freshly produced sewage

b. at sewer pipes

c. in sewer treatment plants

d. none of these.

32. Pick up the correct statement from the following :

a. The larger the sewer in size, more will be velocity

b. The smaller the sewer in size, less will be velocity

c. The larger the sewer in size, no deposition will take place

d. The larger the sewer in size, deposition will take place.

33. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

34. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

35. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

36. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

37. Intermediate catch water drains are provided only, if

a. catchment area of the watershed above road is large

b. intensity of rainfall is heavy

c. single catch water drain is inadequate

d. all the above.

38. Design of horizontal curves on highways, is based on

a. permissible friction on the road surface

b. permissible centrifugal ratio

c. permissible maximum super-elevation

d. all the above.

39. If cross slope of a country is 10% to 25%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

40. The ratio of maximum deviation angle and maximum polar deflection angle of a Lemniscate curve, is

a. 3

b. 4

c. 5

d. 6

41. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

42. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

43. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

44. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

45. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

46. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

47. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

48. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

49. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.

50. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m