***Sub Engineer***

1. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum perpendicular offset for the transition curve, is

a. 1.70 m

b. 2.70 m

c. 3.70 m

d. 4.70 m

2. If L is in kilometres, the curvature correction is

a. 58.2 L2 mm

b. 64.8 L2 mm

c. 74.8 L2 mm

d. 78.4 L2 mm.

3. If the whole circle bearing of a line is 180°, its reduced bearing is

a. S 0° E

b. S 0° W

c. S

d. N.

4. Pick up the correct statement from the following :

a. it is difficult to eliminate an error completely at first trial

b. instability of the instrument makes it almost impossible to adjust it satisfactorily

c. adjustment screws must be left bearing firmly but should never be forced

d. all the above.

5. Bitumen felt

a. is used as water proofing material

b. is used as damp proofing material

c. is made from bitumen and hessian fibres

d. all the above.

6. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

7. The variety of pig iron used for the manufacture of steel by Bessemer process, is

a. Bessemer pig

b. Grey pig

c. White forge pig

d. Mottled pig.

8. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

9. The locus of reaction of a two hinged semi-circular arch, is

a. straight line

b. parabola

c. circle

d. hyperbola.

10. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

11. The property of a material by which it can be drawn to a smaller section, due to tension, is called

a. plasticity

b. ductility

c. elasticity

d. malleability.

12. While testing a cast iron beam (2.5 cm x 2.5 cm) in section and a metre long simply supported at the ends failed when a 100 kg weight is applied at the centre. The maximum stress induced is :

a. 960 kg/cm2

b. 980 kg/cm2

c. 1000 kg/cm2

d. 1200 kg/cm2.

13. The ratio of inertia force of a flowing fluid and the viscous force of the liquid is called :

a. Renold's number

b. Froude's number

c. Euler's number

d. Weber's number.

14. Pick up the incorrect statement from to following regarding triangular notch :

a. For measuring low discharge, it gives more accurate result

b. Only one reading (i.e. H) is required for computation of discharge

c. Ventilation is necessary

d. None of these.

15. Capillary rise of water is

a. directly proportional to surface tension

b. inversely proportional to water density

c. inversely proportional to diameter of the tube

d. All of these.

16. In a venturimeter, the divergent cone is kept

a. shorter than convergent cone

b. equal to covergent cone

c. longer than convergent cone

d. none of these.

17. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

18. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

19. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

20. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

21. The advantage of reinforced concrete, is due to

a. fire-resisting and durability

b. economy because of less maintenance cost

c. moulding in any desired shape

d. All the above.

22. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

23. The maximum permissible size of aggregates to be used in casting the ribs of a slab, is

a. 7.5 mm

b. 10 mm

c. 15 mm

d. 20 mm

24. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

25. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

26. Pile foundation is generally provided if soil is

a. compressible

b. water logged

c. made up

d. all the above.

27. Slump test of concrete is a measure ofits

a. consistency

b. compressive strength

c. tensile strength

d. impact value.

28. The factor which affects workability, is

a. shape and size of the aggregates

b. grading and surface textures of the aggregates

c. air entraining agents

d. all the above.

29. For the prediction of future population of a city, the factor to be considered, is

a. births

b. deaths

c. migrants

d. all the above.

30. Specific yield of a well is

a. quantity of water than can be drawn from the well

b. flow of water per unit time

c. total quantity of water available in the well

d. quantity of water per unit time per unit draw-down.

31. A safety lamp when inserted in the upper portion of a manhole causes flames. It indicates the presence of

a. Carbondioxide gas

b. Hydrogen sulphide gas

c. Methane gas

d. Petrol vapours.

32. Pick up the in-correct statement from the following :

a. Manholes are provided in sewer pipes at suitable intervals

b. Catch basins are generally provided in sewers for carrying drainage discharge

c. Inlets are generally provided in all sewers

d. None of these.

33. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

34. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

35. An outlet is said to be proportional if its flexibility, is

a. zero

b. less than one

c. more than one

d. one

36. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

37. Non-passing sight distance along a road is the longest distance at which the driver of a moving vehicle, may see an obstacle on the pavement

a. 10 cm high

b. 25 cm high

c. 50 cm high

d. 100 cm high

38. To prevent movement of moisture from subgrade to road pavement at the same level as that of water-table, thickness of a cut off layer of coarse sand, is

a. 15 cm

b. 20 cm

c. 30 cm

d. 45 cm

39. Pick up the incorrect statement from the following. On highways circular curves may be

a. compound curves

b. reverse curves

c. vertical curves

d. all the above.

40. In a braking test, a vehicle travelling at 36 km ph was stopped at a braking distance of 8.0 m. The average value of the vehicle's skid resistance (friction coefficient) is

a. 0.64

b. 6.25

c. 0.16

d. none of these

41. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

42. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

43. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

44. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

45. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

46. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

47. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

48. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

49. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°

50. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50