***Sub Engineer***

1. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

2. Removal of parallax, may be achieved by focussing

a. the objective

b. the eye-piece

c. the objective and the eye-piece

d. none of these.

3. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

4. Pick up the correct statement from the following :

a. the system of a series of lines which forms a circuit which ends at the starting point, is called a closed traverse

b. the traverse that starts from a point already fixed in some survey system and ends on another such point, is called a controlled traverse

c. the traverse that is not controlled is called a fly traverse

d. all the above.

5. Mastic asphalt is generally used for

a. damp proof course

b. water proof layer

c. partition walls

d. both (a) and (b).

6. Pick up the correct statement from the following:

a. The polymer consists of thousands of monomers joined together

b. The polymer molecule is called macro-molecule

c. A polymetric material consists of a large number of long-chain molecules

d. All the above.

7. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

8. Non acid-resistant asbestos is :

a. tremolite asbestos

b. chrysolite abestos

c. amosite absestos

d. none of these.

9. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

10. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

11. The slenderness ratio of a vertical column of a square cross-section of 2.5 cm sides and 300 cm length, is

a. 240

b. 360

c. 416

d. 500

12. A three hinged arch is loaded with an isolated load 1000 kg at a horizontal distance of 2.5 m from the crown, 1 m above the level of hinges at the supports 10 metres apart. The horizontal thrust is

a. 1250 kg

b. 125 kg

c. 750 kg

d. 2500 kg

13. Barometres are used to measure

a. difference in pressure at two points

b. atmospheric pressure

c. very low pressure

d. very high pressure.

14. The continuity equation

a. expresses the relationship between work and energy

b. relates the momentum per unit volume between two points on a stream line

c. relates mass rate of flow along a stream line

d. requires that Newton's second law of motion be satisfied at every point in fluid.

15. Unit of kinematic viscosity is

a. m2/sec

b. Newton sec/m2

c. Newton sec/m3

d. Kg sec/m2.

16. If the Mach number for a fluid flow is less than 1, the flow is

a. sonic

b. supersonic

c. sub-sonic

d. none of these.

17. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

18. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

19. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

20. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

21. The maximum ratio of span to depth of a slab simply supported and spanning in one direction, is

a. 25

b. 30

c. 20

d. 15

22. The neutral axis of a T-beam exists

a. within the flange

b. at the bottom edge of the slab

c. below the slab

d. all the above.

23. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

24. The steel generally used in R.C.C. work, is

a. stainless

b. mildsteel

c. high carbon steel

d. high tension steel.

25. The thickness of a reinforced brick partition wall, is generally kept

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm.

26. Pick up the correct statement from the following :

a. D.P.C. should be continuous

b. D.P.C.   should  be  of good  impervious material

c. D.P.C. may be horizontal or vertical

d. All the above.

27. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

28. If the engineer-in-charge approves, the 10 cm cubes may be used for the work test of concrete provided maximum nominal size of aggregate, does not exceed

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

29. The fire demand of a city may be worked out by

a. Freeman formula

b. Under Writers formula

c. Bustan's formula

d. All the above.

30. Disinfection of drinking water is done to remove

a. odour

b. colour

c. bacterias

d. all the above.

31. Self-cleansing velocity is

a. velocity at dry weather flow

b. velocity of water at flushing

c. velocity at which no accumulation remains in the drains

d. velocity of water in a pressure filter.

32. A circular sewer section is preferred to because

a. it is cheaper in construction

b. it provides maximum area for a given perimeter

c. it provides maximum hydraulic mean depth

d. all the above.

33. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

34. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

35. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

36. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

37. Pick up the correct statement from the following:

a. During reconnaissance, the general route of the alignment is selected

b. After reconnaissance, a trace is cut for the alignment

c. Last stage is the detailed surveys for desired geometries'of the highway

d. All the above.

38. A district road with a bituminous pavement has a horizontal curve of 1000 m for a design speed of 75 km ph. The super-elevation is

a. 1 in 40

b. 1 in 50

c. 1 in 60

d. 1 in 70

39. Customers prefer parking of their vehicles at

a. 85° to aisles

b. 80° to aisles

c. 75° to aisles

d. 60° to aisles

40. Depth of reinforcement below the surface of a concrete pavement, is generally kept

a. 5 cm

b. 6 cm

c. 7 cm

d. 8 cm

41. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

42. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

43. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

44. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

45. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

46. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

47. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

48. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

49. Pick up the correct statement from the following:

a. The speed of the aircraft relative to the ground, is called cruising speed

b. The speed of the aircraft relative to wind, is called air speed

c. When wind is blowing the direction of the flight, air speed is less than cruising speed

d. All the above.

50. Conical surface of the approach area rises outwards

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 25