***Sub Engineer***

1. The minimum range for sliding the focusing lens in the internal focusing telescope for focusing at all distances beyond 4 m is

a. 5 mm

b. 10 mm

c. 15 mm

d. 20 mm.

2. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

3. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

4. It is more difficult to obtain good results while measuringhorizontal distance by stepping

a. up-hill

b. down-hill

c. in low undulations

d. in plane areas.

5. Clay and silt content in a good brick earth must be at least

a. 50%

b. 40%

c. 30%

d. 25%

6. Duco is one of the patent forms of

a. plastic paints

b. bituminous paints

c. aluminium paints

d. cellulose paints.

7. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

8. In the cement the compound quickest to react with water, is

a. Tricalcium aluminate

b. Tetra-calcium alumino-ferrite

c. Tricalcium silicate

d. Dicalcium silicate.

9. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

10.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. A steel rod of 2 cm diameter and 5 metres long is subjected to an axial pull of 3000 kg. If E = 2.1 x 106, the elongation of the rod will be

a. 2.275 mm

b. 0.2275 mm

c. 0.02275 mm

d. 2.02275 mm.

12. The cross sections of the beams of equal length are a circle and a square whose permissible bending stress are same under same maximum bending. The ratio of their flexural weights is,

a. 1.118

b. 1.338

c. 1.228

d. 1.108

13. A tank 4m x 3m x 2m containing an oil of specific gravity 0.83 is moved with an acceleration g/2 m sec2. The ratio of the pressures at its bottom when it is moving vertically up and down, is

a. 2

b. 3

c. 1/2

d. 1/3

14. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

15. If the forces are due to inertia and gravity, and frictional resistance plays only a minor role, the design of the channels is made by comparing

a. Reynold number

b. Froude number

c. Weber number

d. Mach number.

16. Mercury is generally used in barometers because

a. the height of the barometer will be less

b. it is a best liquid

c. both (a) and (b) above

d. both (b) and (c) above.

17. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

18. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

19. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

20. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

21. If the diameter of the main reinforcement in a slab is 16 mm, the concrete cover to main bars is

a. 13 mm

b. 14 mm

c. 15 mm

d. 16 mm

22. The width of the flange of a T-beam, which may be considered to act effectively with the rib depends upon

a. overall thickness of the rib

b. centre to centre distance between T-beams

c. span of the T-beam

d. all the above.

23. The pitch of the main bars in a simply supported slab, should not exceed its effective depth by

a. four times

b. five times

c. six times

d. seven times.

24. If P kg/m2 is the upward pressure on the slab of a plain concrete footing whose projection on either side of the wall is a cm, the depth of foundation D is given by

a. D = 0.00775 aP

b. D = 0.0775 aP

c. D = 0.07775 aP

d. D = 0.775 Pa

25. The type of bond in which every course contains both headers and stretchers, is called

a. English bond

b. Flemish bond

c. Russian band

d. Mixed bond.

26. Exposed portions of vertical surface at right angles to the door or window frame, are known as

a. jambs

b. lintels

c. reveals

d. soffits.

27. Pick up the correct statement from the following:

a. Consistency does not affect the workability of concrete

b. If the slump increases, workability decreases

c. If the concrete mix is dry, the slump is maximum

d. None of these.

28. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

29. Acidity in water is caused due to

a. Free CO2

b. Iron sulphate

c. Aluminium sulphate

d. All the above.

30. Manholes along the mains from the source to a city are provided at 500 m intervals in

a. steel pipes

b. R.C.C. pipes

c. hume steel pipes

d. all the above.

31. When drainage to sewage ratio is 20, the peak dry weather flow is

a. 20% of the design discharge

b. slightly less than 5% of the design discharge

c. slightly more than 5% of the design discharge

d. none of these.

32. R.M.O. expenses include

a. running expenses

b. maintenance expenses

c. operation expenses

d. all the above.

33. Solution of Laplacian equation in three dimensions  of water in a syphon, is done by

a. analytical method

b. Khosla's method

c. method of relaxation

d. Unwin's method.

34. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

35. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

36. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

37. The absolute minimum radius of horizontal curve for a design speed 60 km ph is

a. 131 m

b. 210 m

c. 360 m

d. none of these.

38. If R is the radius of a main curve and L is the length of the transition curve, the shift of the curve, is

a. L2/24 R

b. L3/24 R

c. L4/24 R

d. L/12 R

39. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

40. Minimum radius of a simple circular curve deflecting through 5°, is

a. 1618.9 m

b. 1816.9 m

c. 1718.9 m

d. 1817.9 m

41. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

42. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

43. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

44. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

45. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

46. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

47. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

48. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

49. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.

50. Pick up the correct abbreviation from the following:

a. V.H.F, (very high frequency)

b. L.O.M. (low powered outer marker)

c. L.M.M. (low powered middle marker)

d. All the above