***Sub Engineer***

1. The sensitiveness of a level tube decreases if

a. radius of curvature of its inner surface is increased

b. diameter of the tube is increased

c. length of the vapour bubble is increased

d. both viscosity and surface tension are increased.

2. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

3. While measuring with a metallic tape of 30 m length pull should be applied

a. 1 kg

b. 2 kg

c. 3 kg

d. 4 kg

4. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

5. Pick up the hypabyssal rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

6. Plywood is obtained by gluing wooden sheets at

a. 100 to 150 N/cm2

b. 100 to 130°C

c. both (a) and (b)

d. Neither (a) nor (b).

7. A volatile substance added to a paint to make its application easy and smooth, is known as

a. base

b. solvent

c. vehicle

d. none to these.

8. Which one of the following polymers is obtained from condensation polymerization?

a. phenol formaldehyde

b. carbamide

c. melamine-formaldehyde

d. all of these.

9. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

10. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

11. Every material obeys the Hooke's law within its

a. elastic limit

b. plastic point

c. limit of proportionality

d. none of these.

12. The equivalent length of a column fixed at one end and free at the other end, is

a. 0.7 l

b. l

c. 2 l

d. 1.5 l.

13. The dimensionless parameter not applicable to flowing liquids, is

a. Weber number

b. Pressure coefficient

c. Kinematic viscosity

d. Friction factor.

14. In a two dimensional flow if the components of the velocity are u = ax ; v = by, the point where no motion occurs, is known as

a. neutral point

b. stagnation point

c. stationary point

d. none of these.

15. In steady flow, which one of the following changes with time

a. velocity

b. pressure

c. density

d. none of these.

16. Pick up the correct statement from the following :

a. Total energy gradient is the graphical representation of the total head at any section of a pipe line

b. Vertical distance between the total energy line and hydraulic grade line is equal to the velocity head

c. Vertical distance between the total energy line and total energy gradient represents loss of head

d. all the above.

17. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

18. The soil moisture driven off by heat, is called

a. free water

b. hydroscopic water

c. gravity water

d. none of these.

19. The angle of internal friction, is least for

a. angular -grained dense sand

b. round-grained loose sand

c. round-grained loose sand

d. clays.

20. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

21. The pitch of the main bars in a simply supported slab, should not exceed its effective depth by

a. four times

b. five times

c. six times

d. seven times.

22. The minimum head room over a stair must be

a. 205 cm

b. 210 cm

c. 200 cm

d. 230 cm

23. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

24. A part of the slab may be considered as the flange of the T-beam if

a. flange has adequate reinforcement transverse to beam

b. it is built integrally with the beam

c. it is effectively bonded together with the beam

d. all the above.

25. The art of bringing the floor to a true level surface by means of screads, is called

a. topping

b. bedding

c. screading

d. none of these.

26. A wall constructed to resist the pressure of an earth filling, is called

a. retaining wall

b. breast wall

c. buttress

d. parapet wall.

27. Curing

a. reduces the shrinkage of concrete

b. preserves the properties of concrete

c. prevents the loss of water by evaporation

d. all of the above.

28. A concrete using an air entrained cement

a. has more resistance to weathering

b. is more plastic and workable

c. is free from segregation and bleeding

d. all the above.

29. Derivation of Thiem's formula  is based on the assumption

a. the well is sunk through the full depth of the aquifer

b. the flow lines are radial and horizontal, and the flow is laminar

c. for hydraulic gradient in Darcy's equation, tangent value is used for sine value

d. all the above.

30. The maximum pressure to which a pipe is subjected to during its operation, is known

a. working pressure

b. design pressure

c. test pressure

d. pipe pressure

31. The asbestos cement sewers are

a. light in weight

b. not structurally strong to bear large compressive stress

c. susceptible to corrosion by sulphuric acid

d. All the above.

32. Imhoff cone is used to determine

a. settlable solids

b. suspended solids

c. dissolved solids

d. none of these.

33. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

34. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

35. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

36. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

37. For maximum strength and durability minimum percentage of cement, by weight is

a. 20%

b. 25%

c. 30%

d. 33%

38. The steepest gradient permitted on roads which, in ordinary conditions, does not exceed, is known

a. maximum gradient

b. exceptional gradient

c. floating gradient

d. all the above.

39. First operation during the detailed survey of a hill road, is

a. adjustment of alignment along with curves

b. derivation of longitudinal and cross-sections

c. fixation of Bench Marks

d. staking the ground.

40. Side drains on both sides of a hill road, are essential when the road is

a. along the spur curves

b. along the re-entrant curves

c. in cutting

d. none of these.

41. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

42. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

43. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

44. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

45. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

46. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

47. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

48. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

49. Pick up the correct statement from the following:

a. Approach areas are measured in horizontal surfaces

b. The imaginary inclined plane which is directly above the approach area is called approach surface

c. Obstruction clearance surface and approach surface are same

d. All the above.

50. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.