***Sub Engineer***

1. The most reliable method of plotting a theodolite traverse, is

a. by consecutive co-ordinates of each station

b. by independent co-ordinates of each station

c. by plotting included angles and scaling off each traverse leg

d. by the tangent method of plotting.

2. The correction to be applied to each 30 metre chain length along θ° slope, is

a. 30 (sec  θ - 1) m

b. 30 (sin  θ - 1) m

c. 30 (cos  θ - 1) m

d. 30 (tan  θ - 1) m

3. If a linear traverse follows a sharp curve round a large lake where it is difficult to have long legs, the accuracy of the traverse may be improved by

a. taking short legs

b. making repeated observations of angular and linear measurements

c. making a subsidiary traverse to determine the length of a long leg

d. all the above.

4. If the radius of a simple curve is 600 m, the maximum length of the chord for calculating offsets, is taken

a. 15 m

b. 20 m

c. 25 m

d. 30 m.

5. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

6. The stones obtained by blasting are used as

a. ballast in railways

b. aggregates for concrete

c. roadmetal

d. All the above.

7. Pick up the incorrect statement from the following:

a. Plastics are chemical resistant

b. Plastics are durable

c. Plastics are ductile

d. Plastics are excellent electric insulators.

8. Wrought iron contains carbon about

a. 1.5% to 5.5%

b. 0.5% to 1.75%

c. 0.1% to 0.25%

d. none to these.

9. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

10. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

11. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

12. The tensile force required to cause an elongation of 0.045 mm in a steel rod of 1000 mm length and 12 mm diameter, is (where E = 2 x 106 kg/cm2)

a. 166 kg

b. 102 kg

c. 204 kg

d. 74 kg

13. Equation of continuity of fluids is applicable only if

a. flow is compressive

b. flow is one dimensional

c. velocity is uniform over the cross-sections

d. all the above.

14. Frictional loss of head includes the loss of energy due to

a. viscosity

b. turbulence

c. both (a) and (b)

d. none of these.

15. In fluids, steady flow occurs when

a. conditions of flow change steadily with time

b. conditions of flow do not change with time at a point

c. conditions of flow remain the same at adjacent point

d. velocity vector remains constant at a point.

16. A syphon is used

a. to connect water reservoirs at different levels intervened by a hill

b. to supply water to a town from higher level to lower level

c. to fill up a tank with water at higher level from a lower level

d. none of these.

17. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

18. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

19. If the cohesive force, (c), is 1.5 t/m2, the density (γ) of the soil is 2.0 t/m3, factor of safety (F) is 1.5 and stability factor (Sn) is 0.05, the safe height of the §lope, is

a. 5 metres

b. 8 metres

c. 10 metres

d. 12 metres

20. An unsaturated 100 cm3 sample of soil weighs 190 g. If its dried weight is 160 g, water content of the soil, is

a. 0.188

b. 0.288

c. 0.388

d. 0.488

21. If the effective length of a 32 cm diameter R.C.C. column is 4.40 m, its slenderness ratio, is

a. 45

b. 50

c. 55

d. 60

22. If the maximum shear stress at the end of a simply supported R.C.C. beam of 6 m effective span is 10 kg/cm2, the share stirrups are provided for a distance x from either end where x is

a. 50 cm

b. 100 cm

c. 150 cm

d. 200 cm

23. If the sides of a slab simply supported on edges and spanning in two directions are equal, the maximum bending moment is multiplied by

a. 0.3

b. 0.4

c. 0.5

d. 0.7

24. To ensure uniform pressure distribution, the thickness of the foundation, is

a. kept uniform throughout

b. increased gradually towards the edge

c. decreased gradually towards the edge

d. kept zero at the edge.

25. Auger boring

a. is generally employed in cohesive and other self soils above water table

b. is most economical upto a depth of 5 metres

c. is done by portable power driven helical augers those diameters range from 7.5 to 30 cm

d. all the above.

26. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

27. Pick up the correct statement from the following:

a. Construction joints in columns are provided at the bottom haunching

b. Construction joints in beams and slabs are provided within middel third

c. Constructionjoints are generally provided in positions subjected to least shear force

d. All the above.

28. To obtain a very high strength concrete, use very fine grained

a. Granite

b. Magnetite

c. Barite

d. Volcanic scoria.

29. The depth of the water table at a place is 45 m below the general ground level. To lift water from a deep tube well in such a locality, the type of pump to be installed is

a. Centrifugal pump

b. Reciprocating pump

c. Deep well turbine pump

d. None of these.

30. The strainer type tube well, is unsuitable for

a. coarse gravels

b. fine sandy strata

c. clean gravels

d. none of these.

31. The disintegrating pump in which solid matter is broken up before passing out, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

32. To maintain aerobic biological activity, the moisture content of the compost mass should be about

a. 45%

b. 50%

c. 55%

d. 60%.

33. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

34. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

35. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

36. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

37. If a Lemniscate curve of transition throughout is introduced to connect two parallel roads, the maximum polar angle of the curve, is

a. 15°

b. 20°

c. 30°

d. 45°

38. Reduction of load capacity in a ruling gradient of

a. 1 in 15, is 15%

b. 1 in 20, is 10%

c. 1 in 25, is 25%

d. none of these.

39. If present A.D.T. is 5000 vehicles and annual increase is 10%, the average future flow after 5 years will be

a. 6050 vehicles

b. 7050 vehicles

c. 8050 vehicles

d. 9050 vehicles

40. A gradient along which the vehicle does not require any tractive effort to maintain a specified speed, is known as

a. ruling gradient

b. pushing gradient

c. floating gradient

d. minimum gradient

41. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

42. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

43. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

44. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

45. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

46. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

47. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

48. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

49. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.

50. For night landing, the thresholds are lighted

a. green

b. red

c. white

d. yellow