***Sub Engineer***

1. In geodetic surveys higher accuracy is achieved, if

a. curvature of the earth surface is ignored

b. curvature of the earth surface is taken into account

c. angles  between the curved lines are treated as plane angles

d. none of these.

2. Keeping the instrument height as 1  m, length of staff 4 m, the up gradient of the ground 1 in 10, the sight on the up slope must be less than

a. 25 cm

b. 20 m

c. 45 m

d. 10 m

3. An imaginary line lying throughout on the surface of the earth and preserving a constant inclination to the horizontal, is called

a. contour line

b. contour gradient

c. level line

d. line of gentle scope.

4. Determining the difference in elevation between two points on the surface of the earth, is known as

a. levelling

b. simple levelling

c. differential levelling

d. longitudinal levelling.

5. Pick up the correct statement from the following:

a. The phenol is either extracted from coal-tar or prepared from benzene

b. Phenol reacts with formaldehyde, to form phenol formaldehyde resin

c. The plastics prepared from phenol-formaldehyde are used for paints, varnishes, w.c. seats

d. All the above.

6. Geologically, marble is known as

a. sedimentary rock

b. igneous rock

c. metamorphic rock

d. stratified rock.

7. For making fly-ash building bricks, the following mix of fly-ash, sand and lime, is

a. 80 : 13 : 7

b. 70 : 20 : 10

c. 60 : 35 : 5

d. none of these.

8. Which one of the following is used for preparing porcelain

a. feldspar

b. quartz

c. minerals

d. All of these.

9. The equivalent length of a column of length L, having both the ends hinged, is

a. 2L

b. L

c.

d.

10. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

11. A column is said to be of medium size if its slenderness ratio is between

a. 32 and 120

b. 120 and 160

c. 160 and 180

d. 180 and 200

12. A cast iron T section beam is subjected to pure bending. For maximum compressive stress to be three times the maximum tensile stress, centre of gravity of the section from flange side is

a. h/4

b. h/3

c. h/2

d. 2/3 h.

13. Water belongs to

a. Newtonian fluids

b. non-Newtonian fluids

c. compressible fluid

d. none of these.

14. Fluids change the volume under external presssure due to

a. plasticity

b. viscosity

c. compressibility

d. none of these.

15. The best side slope for most economical trapezoidal section, is

a. 30°

b. 45°

c. 60°

d. None of these.

16. Manometers are used to measure

a. pressure in water channels, pipes etc.

b. difference in pressure at two points

c. atmospheric pressure

d. very low pressure.

17. Cohesive soils are generally

a. plastic and also compressible

b. elastic and also compressible

c. plastic but incompressible

d. none of these.

18. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

19. The ratio of the volume of voids to the volume of soil solids in a given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

20. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

21. Thickened part of a flat slab over its supporting column, is technically known as

a. drop panel

b. capital

c. column head

d. none of these.

22. An R.C.C. column is treated as long if its slenderness ratio is greater than

a. 35

b. 40

c. 50

d. 60

23. According to I.S. : 456 specifications, the safe diagonal tensile stress for M 150 grade concrete, is

a. 5 kg/cm2

b. 10 kg/cm2

c. 15 kg/cm2

d. 20 kg/cm2

24. Design of a two way slab simply supported on edges and having no provision to prevent the corners from lifting, is made by

a. Marcus formula

b. Rankine Grashoff formula

c. Grashoff formula

d. Rankine-Marcus formula.

25. A pre-stressed concrete pile is

a. lighter in weight

b. extremely durable

c. suitable for heavy load

d. all the above.

26. The depth of excavation of foundations, is generally measured with a

a. ranging rod

b. steel tape

c. levelling staff

d. bonning rod.

27. Pick up the correct statement from the following:

a. Fine sand bulks more than coarse sand

b. With 10% moisture content by weight, the bulking of sand is increased by 50%.

c. The volume of fully saturated sand, is equal to the volume of dry and loose sand

d. All the above.

28. Setting time of cement increases by adding

a. gypsum

b. hydrogen peroxide

c. calcium chloride

d. sodium oxide

29. Well treated water is generally supplied for

a. commercial use

b. public use

c. waste water

d. all the above.

30. Standard process of chlorination of water, is done by

a. plain chlorination

b. pre-chlorination

c. post-chlorination

d. double chlorination.

31. The intensity of rain is expressed in

a. cm/minute

b. cm/hour

c. cm/day

d. none of these.

32. Stoneware sewers are available in size

a. 15 cm

b. 20 cm

c. 25 cm

d. all the above.

33. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

34. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

35. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

36. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

37. A district road with a bituminous pavement has a horizontal curve of 1000 m for a design speed of 75 km ph. The super-elevation is

a. 1 in 40

b. 1 in 50

c. 1 in 60

d. 1 in 70

38. If no super elevation is provided on a road along curves, pot holes may develop at

a. inner edge of the road

b. outer edge of the road

c. centre of the road

d. no where on the road.

39. Width of the shoulders of carriage way is generally kept

a. 125 cm

b. 150 cm

c. 200 cm

d. 250 cm

40. In a right angle bend of a road provided with a transition throughout, the maximum polar angle will be

a. 15°

b. 20°

c. 30°

d. 45°

41. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

42. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

43. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

44. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

45. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

46. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

47. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

48. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

49. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.