***Sub Engineer***

1. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

2. Whole circle bearing of a line is preferred to a quadrantal bearing merely because

a. bearing is not completely specified by an angle

b. bearing is completely specified by an angle

c. Sign of the correction of magnetic declination is different in different quadrants

d. its trigonometrical values may be extracted from ordinary tables easily.

3. Pick up the method of surveying in which field observations and plotting proceed simultaneously from the following

a. chain surveying

b. compass surveying

c. plan table surveying

d. tacheometric surveying.

4. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

5. The lime which contains high percentage of calcium oxide, is generally called

a. fat lime

b. rich lime

c. white lime

d. All of these.

6. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

7. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

8. If water required for 1 bag of cement is 30 litres, the water cement ratio is :

a. 0.40

b. 0.50

c. 0.60

d. None of these.

9. The ratio of moments of inertia of a triangular section about its base and about a centroidal axis parallel to its base, is

a. 1.5

b. 2.0

c. 2.5

d. 3.0

10.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. Ties are load carrying members of a frame, which are subjected to

a. transverse loads

b. axial tension loads

c. axial compressive loads

d. torsional loads.

12. For structural analysis, Maxwell's reciprocal theorem can be applied to :

a. plastic structures

b. elastic structures

c. symmetrical structures

d. all the above.

13. Though angle of deviation of liquid is more in internal mouth piece, the contraction of the jet, is

a. more in the internal mouth piece

b. less in the internal mouth piece

c. equal to external mouth piece

d. none of these.

14. The value of momentum correction factor (β) for a laminar flow through a circular pipe, is

a. 2/3

b. 3/4

c. 4/3

d. 3/2

15. Hydrostatic pressure on a dam depends upon its

a. depth

b. shape

c. material

d. both (b) and (c).

16. For critical flow, the Froude number is :

a. 1.0

b. less than 1.0

c. more than 1.0

d. 2

17. The water content of soil is defined as the ratio of

a. volume of water to volume of given soil

b. volume of water to volume of voids in soil

c. weight of water to weight of air in voids

d. weight of water to weight of solids of given mass of soil.

18. The fundamental equation of air content (ac), degree of saturation (Sr) and void ratio (e), is

a. ac =

b. ac =

c. ac =

d. ac = 1 + e/Sr

19. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

20. The ratio of the volume of voids to the volume of soil solids in a given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

21. The amount of reinformcement for main bars in a slab, is based upon

a. minimum bending moment

b. maximum bending moment

c. maximum shear force

d. minimum shear force.

22. A foundation rests on

a. base of the foundation

b. subgrade

c. foundation soil

d. both (b) and (c)

23. An R.C.C. beam of 25 cm width and 50 cm effective depth has a clear span of 6 metres and carries a U.D.L. of 3000 kg/m inclusive of its self weight. If the lever arm constant for the section is 0.865, the maximum intensity of shear stress, is

a. 8.3 kg/cm2

b. 7.6 kg/cm2

c. 21.5 kg/cm2

d. 11.4 kg/cm2

24. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

25. The least bearing capacity of soil is that of

a. moist clay

b. soft rock

c. laminated

d. coarse sandy soil.

26. The stone whose crushing strength is maximum, is

a. granite

b. chalk

c. slate

d. sand stone

27. Allowable shear strength of concrete, depends upon

a. shear strength

b. tensile strength

c. compressive strength

d. none of these.

28. If aggregates completely pass through a sieve of size 75 mm and are retained on a sieve of size 60 mm, the particular aggregate will be flaky if its minimum dimension is less than

a. 30.5 mm

b. 40.5 mm

c. 50.5 mm

d. none of these.

29. Pick up the correct statement from the following :

a. Water with hardness more than 200 ppm is considered hard

b. Water needed for laundaries should contain hardness less than 75 ppm

c. To provide taste during drinking, water should possess hardness between 75 to 120

d. All the above.

30. A high velocity of wash water is required for

a. rapid gravity filter without strainers

b. slow sand filter with strainers

c. slow sand filter without strainers

d. none of these.

31. Which one of the following gases is most significant as air pollutant

a. Carbondioxide

b. Oxygen

c. Nitrogen

d. Sulphurdioxide.

32. A nuisance is experienced in diluting water if dilution factor is less than

a. 60

b. 40

c. 20

d. 10

33. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

34. The field capacity of a soil is 25%, its permanent wilting point is 15% and specific dry unity weight is 1.5. If the depth of root zone of a crop, is 80 cm, the storage capacity of the soil, is

a. 10 cm

b. 12 cm

c. 14 cm

d. 16 cm

35. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

36. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

37. If cross slope of a country is 10% to 25%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

38. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

39. Maximum super-elevation on hill roads should not exceed

a. 7%

b. 8%

c. 10%

d. 15%

40. The inventor of road making as a building science, was

a. Sully

b. Tresguet

c. Telford

d. Macadam.

41. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

42. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

43. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

44. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

45. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

46. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

47. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

48. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

49. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.

50. Pick up the correct statement from the following:

a. F.I.C. stands for flight information centre

b. Radius of control area is 160 km

c. Radius of control zone is 40 km

d. All the above.