***Sub Engineer***

1. A lemniscate curve will not be transitional throughout, if its deflection angle, is

a. 45°

b. 60°

c. 90°

d. 120°

2. If the declination of the needle is 10° W

a. each of the whole circle reckoning has to be micros by 10°

b. in the quadrantal method, the correction is positive in the 1st and 3rd quadrants

c. in the quadrantal method, the corrections is negative in 2nd and 4th quadrants

d. all the above.

3. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

4. Measuring with a 30 m chain, 0.01 m too short, introduces

a. positive compensating error

b. negative compensating error

c. positive cumulative error

d. negative cumulative error.

5. Teak wood is suitable for

a. sports articles

b. furnitures

c. railway sleepers

d. all the above.

6. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

7. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

8. A good quality stone absorbs water less than

a. 5%

b. 10%

c. 15%

d. 20%

9. If a solid shaft (diameter 20 cm, length 400 cm, N = 0.8 x 105 N/mm2) when subjected to a twisting moment, produces maximum shear stress of 50 N/mm2, the angle of twist in radians, is

a. 0.002

b. 0.0025

c. 0.003

d. 0.005

10. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

11. A member which does not regain its original shape after removed of load producing deformation is said

a. plastic

b. elastic

c. rigid

d. none of these.

12. As the elastic limit reaches, tensile strain

a. increases more rapidly

b. decreases more rapidly

c. increases in proportion to the stress

d. decreases in proportion to the stress.

13. Pick up the correct statement from the following :

a. For maximum velocity of flow, the depth of water in the circular channel must be 0.81 times the diameter of the channel

b. For maximum velocity, the hydraulic mean depth must be 0.3 times the diameter of circular channel

c. For maximum discharge the depth of flow must be 0.95 times the diameter of circular channel

d. All the above.

14. In flow, the liquid particles may possess

a. potential energy

b. kinetic energy

c. pressure energy

d. all the above.

15. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

16. An ideal fluid is

a. incompressible

b. compressible

c. compressible and non-viscous

d. slightly affected by surface torque.

17. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

18. A soil not fully consolidated under the existing over-burden pressure, is called

a. pre-consolidated

b. normally consolidated

c. under-consolidated

d. none of these.

19. If water content of a soil is 40%, G is 2.70 and void ratio is 1.35, the degree of saturation is

a. 75%

b. 80%

c. 85%

d. 90%

20. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

21. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

22. If the depth of actual neutral axis of a doubly reinforced beam

a. is greater than the depth of critical neutral axis, the concrete attains its maximum stress earlier

b. is less than the depth of critical neutral axis, the steel in the tensile zone attains its maximum stress earlier

c. is equal to the depth of critical neutral axis, the concrete and steel attain their maximum stresses simultanesouly

d. all the above.

23. To ensure uniform pressure distribution, the thickness of the foundation, is

a. kept uniform throughout

b. increased gradually towards the edge

c. decreased gradually towards the edge

d. kept zero at the edge.

24. The design of heel slab of a retaining wall, is based on the maximum bending moment due to:

a. weight of the soil above it,

b. load of the surcharge, if any

c. upward soil reaction

d. all the above.

25. The arrangement made to support an unsafe structure temporarily, is known as

a. shoring

b. scaffolding

c. underpinning

d. jacking

26. Cast iron piles

a. are suitable for works under sea water

b. resist shocks or vibrations

c. are suitable for use as batter piles

d. are useful for heavy vertical loads.

27. Grading of sand causes great variation in

a. strength of concrete

b. durability of concrete

c. handing and placing of concrete

d. all the above.

28. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

29. Cast iron pipes are generally preferred to, because

a. of ease to join

b. of durability

c. of longer life

d. all the above.

30. Pick up the incorrect statement from the following. The underground sources of water, is from

a. springs

b. infiltration wells

c. storage reservoirs

d. none of these.

31. The coagulant which is generally not used for treating the sewage, is

a. alum

b. ferric chloride

c. ferric sulphate

d. chlorinated copperas.

32. Drop manholes at the junctions of sewer lines, are provided if

a. invert level of a branch sewer is more than 60 cm that of the main sewer

b. sewer line runs along a main road

c. ordinary manhole cannot be built

d. two sewer lines intersect.

33. The main function of a diversion head works of a canal from a river, is

a. to control floods

b. to store water

c. to raise water level

d. all the above.

34. If water table is comparatively high, the irrigation canal becomes useless, due to

a. water logging of the cultivated areas

b. uncertain water demand

c. spread of malaria

d. all the above.

35. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

36. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

37. The usual width of parapet walls along Highways in hilly region, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 100 cm

38. The gradients of trace cuts for hilly roads, are kept

a. equal to ruling gradient

b. 10 tO 20 per cent easier than ruling gradients

c. 10 to 20 per cent steeper than ruling gradients

d. none of these.

39. Roughness index of roads, is expressed as

a. size of the stone on the pavement

b. number of patches on the pavement

c. cumulative deformation of surface per horizontal distance

d. type of the road surface.

40. Reconnaissance is best done with the help of

a. aerial photographic survey

b. condastral surveys

c. topographical surveys

d. triangulation surveys

41. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

42. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

43. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

44. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

45. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

46. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

47. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

48. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

49. The bearing of the longest line of a wind rose is S 45° E, the bearing of the runway will be numbered

a. 315°

b. 13

c. 31

d. both (c) and (d)

50. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.