***Sub Engineer***

1. In geodetic surveys higher accuracy is achieved, if

a. curvature of the earth surface is ignored

b. curvature of the earth surface is taken into account

c. angles  between the curved lines are treated as plane angles

d. none of these.

2. For preparation of a contour plan for a route survey

a. method of squares is used

b. method of trace contour is used

c. method of cross profile is used

d. indirect method of contouring is used.

3. Total latitude of a point is positive if it lies

a. north of the reference parallel

b. south of the reference parallel

c. east of the reference parallel

d. west of the reference parallel.

4. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

5. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

6. For filling cracks in masonry structures, the type of bitumen used, is

a. cut-back bitumen

b. bitumen-emulsion

c. blown bitumen

d. plastic bitumen.

7. Asbestos cement

a. is brittle

b. warps due to changes in humidity

c. strength is lowered when saturated by water

d. all the above.

8. Duco paints are

a. cellulose paints

b. emulsion paints

c. bituminous paints

d. oil paints.

9. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

10. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

11. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

12. If the width of a simply supported beam carrying an isolated load at its centre is doubled, the deflection of the beam at the centre is changed by

a. 1/2

b. 1/8

c. 2

d. 8

13. The ratio of the percentage error in the discharge and percentage error in the measurement of head, over rectangular notch, is

a. 1/2

b. 2/3

c. 3/2

d. 3/4

14. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

15. The velocity distribution of viscous fluid through a circular/pipe is :

a. hyperbolic

b. circular

c. parabolic

d. elliptical.

16. Most economical section of a circular channel for maximum discharge

a. hydraulic mean depth = 0.286 diameter of circular section

b. wetted perimeter = 2.6 diameter of circular section

c. wetted perimeter = 2.83 depth of water

d. all the above.

17. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

18. Stoke's law does not hold good if the size of particle is smaller than

a. 0.0002 mm

b. 0.002 mm

c. 0.02 mm

d. 0.2 mm

19. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

20. The ratio of the difference between the void ratio of the soil in its loosest state and its natural void ratio (e) to the difference between the void ratios in the loosest and fully dense state, is generally termed as

a. degree of density

b. relativity

c. density index

d. all the above.

21. For a ribbed slab

a. clear spacing between ribs shall not be greater than 4.5 cm

b. width of the rib shall not be less than 7.5 cm

c. overall depth of the slab shall not exceed four times the breadth of the rib

d. all the above.

22. In a singly reinforced beam

a. steel possesses initial stresses when em-beded in concrete

b. plane sections transverse to the centre line of the beam before bending remain plane after bending

c. elastic modulii for concrete and steel have different values within the limits of deformation of the beam

d. none of these.

23. In favourable circumstances a 15 cm concrete cube after 28 days, attains a maximum crushing strength

a. 200 kg/cm2

b. 300 kg/cm2

c. 400 kg/cm2

d. 500 kg/cm2

24. On an absolutely rigid foundation base, the pressure will

a. be more at the edges of the foundation

b. be uniform

c. not be uniform

d. be zero at the centre of the foundation.

25. The triangular portion between any two adjacent arches and the tangent to their crowns, is

a. haunch

b. spandril

c. soffit

d. rise.

26. Which one of the following factors is considered for the orientation of buildings :

a. the direction of the prevailing winds in the area

b. the exposure of the walls and roof of the buildings to the rays of sun

c. the extent up to which the sunrays penetrate with the verandah.

d. all the above.

27. If the average compressive strength is 4000 kg/cm2 and standard deviation is 500, the co-efficient of variation is

a. 12.5%

b. 15%

c. 18.5 %

d. 20%

28. Addition of pozzolana to cement causes

a. loss of heat of hydration

b. reduction in bleeding

c. increase in curing time

d. all the above.

29. Biochemical Oxygen Demand (B.O.D.) of safe drinking water must be

a. nil

b. 5

c. 10

d. 15

30. For maximum alkalinity of water, pH value should be

a. less than 7

b. more than 7

c. 14

d. 21

31. The most widely used pump for lifting sewage is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. air pressure pump.

32. A manhole is classified as shallow if its depth is between

a. 0.5 to 0.7 m

b. 0.7 to 0.9 m

c. 0.9 to 1.20 m

d. 1.2 to 1.50 m.

33. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

34. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

35. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

36. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

37. Pick up the correct statement from the following:

a. The corss fall of the shoulder should be at least 0.5% steeper than camber

b. On superelevated sections, the shoulders should be provided a cross fall equal to camber

c. Earthern roads in general are provided steepest crossfall

d. All the above.

38. If brakes of vehicles are effective, the vehicle-running at 30 km/hour comes to a stop in

a. 10 metres

b. 12 metres

c. 15 metres

d. 18 metres

39. A single lane carriage way whenever changes to two-lane carriage way, is affected through a taper of

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 15 to 1 in 20

40. When a number of hair pin bends are introduced, a minimum intervening distance in between is kept

a. 20 m

b. 40 m

c. 60 m

d. 100 m

41. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

42. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

43. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

44. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

45. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

46. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

47. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

48. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

49. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.

50. For the proposed air port, the survey project provides

a. topograhic plan

b. grading plan

c. drainage plan

d. all the above.