***Sub Engineer***

1. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

2. The bearings of the lines AB and BC are 146° 30' and 68° 30'. The included angle ABC is

a. 102°

b. 78°

c. 45°

d. none of these.

3. The boundary of water of a still lake, represents

a. level surface

b. horizontal surface

c. contour line

d. a concave surface.

4. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

5. Pick up the correct statement from the following: Method of sawing timber

a. tangentially to annual rings, is known as tangential method.

b. in four quarters such that each board cuts annual rings at angles not less than 45°, is known as quarter sawing method.

c. cut out of quarter logs, parallel to the medullary rays and perpendicular to annual rings, is known as radial sawing.

d. all the above.

6. Pick up the correct statement from the following:

a. Alexander Parkes, a Scottish chemist prepared a hard material by mixing camphor and alcohol with nitro cellulose and called it, as Parkesite

b. Dr. L. Bakeland, a Belgian scientist prepared a product known as Bakelite

c. Pollark, an Austrian scientist prepared a substance from urea and formaldehyde and called it Plastic

d. All the above.

7. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

8. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

9. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

10. A masonry dam (density = 20, 000 N/m3) 6 m high, one metre wide at the top and 4 m wide at the base, has vertical water face. The minimum stress at the base of the dam when the reservoir is full, will be

a. 75 N/m2

b. 750 N/m2

c. 7500 N/m2

d. 75000 N/m2

11. If a constant section beam is subjected to a uniform bending moment throughout, its length bends to

a. a circular arc

b. a parabolic arc

c. a catenary

d. none of these.

12. A short masonry pillar is 60 cm x 60 cm in cross-section, the core of the pillar is a square whose side is

a. 14.14 cm

b. 20.00 cm

c. 22.36 cm

d. 25.22 cm.

13. The width of a weir with end contraction, is

a. equal to the width of the channel

b. less than the width of the channel

c. half the width of the channel

d. none of these.

14. A cylindrical vessel 40 cm high is revolved about its vertical axis so that the water touches the bottom when it just spills out. If the radius of the cylinder is 5 cm, the angular velocity of rotation, is

a. 2 rad/sec

b. 3 rad/sec

c. 4 rad/sec

d. 5 rad/sec

15. In a fluid flow a particle may posses

a. kinetic energy

b. pressure energy

c. initial energy

d. all the above.

16. For critical flow, the Froude number is :

a. 1.0

b. less than 1.0

c. more than 1.0

d. 2

17. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

18. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

19. A soil has bulk density 2.30 g/cm3 and water content 15 per cent, the dry density of the sample, is

a. 1.0 g/cm2

b. 1.5 g/cm3

c. 2.0 g/cm3

d. 2.5 g/cm3

20. The critical exist gradient of seepage water in soils, increases with

a. a decrease in specific gravity

b. a decrease in void ratio

c. both (a) and (c)

d. none of these.

21. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

22. A part of the slab may be considered as the flange of the T-beam if

a. flange has adequate reinforcement transverse to beam

b. it is built integrally with the beam

c. it is effectively bonded together with the beam

d. all the above.

23. The minimum clear cover for R.C.C. columns shall be

a. greater of 40 mm or diameter

b. smaller of 40 mm or diameter

c. greater of 25 mm or diameter

d. smaller of 25 mm or diameter

24. The live load to be considered for an inaccessible roof, is

a. Nil

b. 75 kg/m2

c. 150 kg/cm2

d. 200 kg/m2

25. Which of the following metal sheets is most effective in preventing dampness ?

a. Copper sheets

b. lead sheets

c. aluminium sheets

d. all the above.

26. The rock formed from the solidification of molten matter (magma) is called :

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. none of the above.

27. ISI has specified full strength of concrete after

a. 14 days

b. 21 days

c. 28 days

d. none of these.

28. Shrinkage in concrete can be reduced by using

a. less cement in the concrete

b. proper concrete mix

c. presaturated aggregates

d. all the above.

29. Asbestos pipes are

a. highly resistant to corrosion

b. high flexible to accommodate deflection upto 12°

c. very much smooth and hydraulically efficient

d. all the above.

30. In a rapid gravity filter

a. raw water from the source is supplied

b. disinfected raw water is supplied

c. raw water passed through coagulation tank is supplied

d. none of these.

31. In sewage having fully oxidised organic matter, the nitrogen exists in the form of

a. nitrates

b. free ammonia

c. aluminoid nitrogen

d. none of these.

32. Removal of oil and grease from sewage, is known

a. screening

b. skimming

c. filtration

d. none of these.

33. The width of a meander belt is the transverse distance between

a. apex point of one curve and apex point of the reserve curve

b. apex point and the crossing

c. two banks of meandering river

d. none of these.

34. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

35. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

36. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

37. The most commonly adopted method to provide super-elevation on roads, is by pivoting the road surface about

a. outer edge so that the inner edge is lowered

b. crown so that outer edge is raised and inner edge is lowered

c. inner edge so that outer edge is raised

d. none of these.

38. Reduction of load capacity in a ruling gradient of

a. 1 in 15, is 15%

b. 1 in 20, is 10%

c. 1 in 25, is 25%

d. none of these.

39. Pick up the correct statement from the following:

a. Seasonal cycle of traffic volume during April and November, is usually near the annual average

b. Mid-winter seasonal cycle of traffic is least

c. Mid-summer seasonal cycle of traffic is highest

d. All the above.

40. The pavement width of a road depends upon

a. terrain

b. type of traffic

c. number of lanes

d. all the above.

41. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

42. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

43. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

44. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

45. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

46. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

47. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

48. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

49. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots

50. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°