***Sub Engineer***

1. The longitudinal section of the surface of bubble tube is

a. straight

b. circular

c. parabolic

d. elliptic.

2. In levelling operation

a. if first reading is more than second, it represents a rise

b. if  first reading is less than second, it represents a fall

c. if  second reading is less than first, it represents a fall

d. both (b) and (c).

3. Pick up the method of surveying in which field observations and plotting proceed simultaneously from the following

a. chain surveying

b. compass surveying

c. plan table surveying

d. tacheometric surveying.

4. Pick up the correct statement from the following :

a. the power of a lens is the reciprocal of its focal length

b. the unit of power of the lens is diopter

c. the power of two or more thin lenses in contact is the power of the combination of the lenses

d. all the above.

5. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

6. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

7. For one cubic metre of brick masonry, number of bricks required, is

a. 425

b. 450

c. 500

d. 550.

8. Pick up the synthetic resin from the following:

a. Phenolic resin

b. Resorcinol resin

c. Urea resin

d. All of these.

9. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse.

10. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

11. In a loaded beam, the point of con-traflexture occurs at a section where

a. bending moment is zero or changes sign

b. bending moment is maximum

c. shearing force is maximum

d. shearing force is minimum.

12. The reaction at support A of the beam shown in below figure, is

a. zero

b. 5 T

c. 10 T

d. 1 T

13. An error of 1% in measuring the head of water over the crest of a triangular notch, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 2.0%.

d. 2.5%

14. For an irrotational flow, the equation  is given by

a. Cauchy-Riemann

b. Reynold

c. Laplaces

d. Bernoulli.

15. For critical depth of flow of water in open channels, fc the specific energy must be :

a. minimum

b. maximum

c. average of maximum and minimum

d. None of these.

16. Flow of water in pipes of diameter more than 3 metres, can be measured by

a. pitot tube

b. venturimeter

c. orifice plate

d. rotameter.

17. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

18. If voids ratio is 0.67, water content is 0.188 and specific gravity is 2.68, the degree of saturation of the soil, is

a. 25%

b. 40%

c. 60%

d. 75%

19. The seepage force in a soil, is

a. perpendicular to the equipotential lines

b. proportional to the exit graident

c. proportional to the head loss

d. all the above.

20. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

21. According to I.S. : 456, slabs which span in two directions with corners held down, are assumed to be divided in each direction into middle strips and edge strips such that the width of the middle strip, is

a. two-third of the width of the slab

b. three-fourth of the width of the slab

c. four-fifth of the width of the slab

d. three-fifth of the width of the slab.

22. If depth of slab is 10 cm, width of web 30 cm, depth of web 50 cm, centre to centre distance of beams 3 m, effective span of beams 6 m, the effective flange width of the beam, is

a. 200 cm

b. 300 cm

c. 150 cm

d. 100 cm

23. If the maximum shear stress at the end of a simply supported R.C.C. beam of 16 m effective span is 10 kg/cm2, the length of the beam having nominal reinforcement, is

a. 4 cm

b. 6 m

c. 8 m

d. 10 m

24. The minimum clear cover for R.C.C. columns shall be

a. greater of 40 mm or diameter

b. smaller of 40 mm or diameter

c. greater of 25 mm or diameter

d. smaller of 25 mm or diameter

25. Stability of an existing structure may be disturbed by

a. vibrations caused by traffic movements

b. mining in the neighbourhood

c. excavation in the neighbourhood

d. all the above.

26. The inclined support at the ends of treads and rises of a stair, is known as

a. baluster

b. header

c. string

d. beam.

27. The surface where two successive placements of concrete meet, is known as

a. Expansion joint

b. Construction joint

c. both (a) and (b)

d. both (6) and (c).

28. A flaky aggregate is said to be elongated if its length is

a. twice the mean size

b. thrice the mean size

c. four times the mean size

d. five times the mean size.

29. Asbestos pipes are

a. highly resistant to corrosion

b. high flexible to accommodate deflection upto 12°

c. very much smooth and hydraulically efficient

d. all the above.

30. Air valves are generally provided in pressure pipes of water supply

a. at pipe junctions

b. at summits

c. at low points

d. near service pipes.

31. The dimensions of a rectangular settling tank are : length 24 m, width 6 m and depth 3 m. If 2 hour detention period for tanks is recommended, the rate of flow of sewage per hour, is

a. 208 cu m

b. 212 cu m

c. 216 cu m

d. 220 cu m.

32. In detritus tanks,

a. flow velocity is kept 0.09 m/sec

b. detention period is kept 3 to 4 minutes

c. organic and inorganic materials are separated

d. all the above.

33. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

34. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

35. F.S.L. of a canal at its head with respect to parent channel is kept

a. at the same level

b. 15 cm lower

c. 15 cm higher

d. none of these.

36. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

37. The wall constructed for the stability of a back filling portion of a road on the down hill side, is known as

a. retaining wall

b. breast wall

c. parapet wall

d. all the above.

38. Thickness of broken line markings on multi-lane road for lanes is generally kept

a. 10 cm

b. 12 cm

c. 15 cm

d. 18 cm

39. The width of road pavements, depends upon

a. width of traffic lane

b. number of traffic

c. width of median strip

d. all the above.

40. Horizontal curves on highways are provided

a. to break the monotony of driving

b. to discourage the tendency to increase speed

c. to decrease the mental strain on drivers

d. all the above.

41. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

42. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

45. Pick up the correct statement from the following with regards to C.P.M. network analysis of projects

a. Earliest occurrence time of the event from which the activity arrow originates plus the duration of the activity, is called earliest finish time of the activity

b. The latest occurrence time of the node of which the activity arrow terminates minus the duration of the activity, is called latest start time

c. The latest occurrence time for the node at which the activity arrow terminates, is called latest finish time

d. All the above.

46. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

47. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

48. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

49. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.

50. The length of runway is increased per 300 m rise above M.S.L.

a. 4%

b. 5%

c. 6%

d. 7%