***Sub Engineer***

1. The included angles of a theodolite traverse, are generally measured

a. clockwise from the forward station

b. anti-clockwise from the back station

c. anti-clockwise from the forward station

d. clockwise from the back station.

2. From any point on the surface with a given inclination

a. only one contour gradient is possible

b. two contour gradients are possible

c. indefinite contour gradients are possible

d. all the above.

3. While rotating the theodolite in the horizontal plane, the bubble of the bubble tube takes up the same position in its tube, it indicates

a. the rotation axis is vertical

b. the trummion axis is horizontal

c. the line of collimation is perpendicular to vertical axis

d. none of the above

4. The surface of zero elevation around the earth, which is slightly irregular and curved, is known as

a. mean sea level

b. geoid surface

c. level surface

d. horizontal surface.

5. A rock contains only one mineral. It is called

a. homogeneous

b. non-homogeneous

c. monomineralic

d. polymineralic.

6. The silica is used for preparing

a. silica bricks

b. coke oven

c. lining for glass furnaces

d. all of these.

7. Pick up the correct statement from the following:

a. During rusting, first ferrous bicarbonates are formed

b. On further oxidation ferrous bicarbonates get converted to ferric bicarbonates

c. Ultimately hydrated ferric oxide is formed during rusting and carbon dioxide gets liberated

d. All the above.

8. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

9. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

10. The ratio of shear stress and shear strain of an elastic material, is

a. Shear Modulus

b. Young's Modulus

c. Modulus of Elasticity

d. both (a) and (b).

11. The under mentioned type is simple strain

a. compressive strain

b. shear strain

c. volumetric strain

d. all the above.

12. If the stress in each cross-section of a pillar is equal to its working stress, it is called

a. body of equal

b. body of equal section

c. body of equal strength

d. none of these.

13. Equation of continuity of fluids is applicable only if

a. flow is compressive

b. flow is one dimensional

c. velocity is uniform over the cross-sections

d. all the above.

14. The unit of the viscosity is

a. kg sec/m2

b. Newton sec per m2

c. Newton-sec2/m3

d. m2 per sec.

15. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

16. A jet projected at an angle of 45θ, 40 m from the foot of a vertical column, just reaches the top of the column. The height of the column is

a. 20 m

b. 30 m

c. 40 m

d. 60 m.

17. Pick up the correct statement from the following:

a. Failure plane carries maximum shear stress

b. Failure plane does not carry maximum shear stress

c. Failure plane carries shear stress equal to maximum shear stress

d. None of these.

18. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

19. Pick up the correct statement from the following:

a. At critical hydraulic gradient, the saturated sand becomes quick

b. The critical gradient depends on the void ratio and the specific gravity

c. The quick sand occurs more in fine sand and silt than coarse material

d. All the above.

20. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

21. The thickness of base slab of a retaining wall generally provided, is

a. one-third of the width of the stem at the bottom

b. one fourth of the width of the steam at the bottom

c. width of the stem at the bottom

d. twice the width of the steam at the bottom.

22. If the depth of actual neutral axis of a doubly reinforced beam

a. is greater than the depth of critical neutral axis, the concrete attains its maximum stress earlier

b. is less than the depth of critical neutral axis, the steel in the tensile zone attains its maximum stress earlier

c. is equal to the depth of critical neutral axis, the concrete and steel attain their maximum stresses simultanesouly

d. all the above.

23. If the permissible compressive stress for a concrete in bending is C kg/m2, the modular ratio is

a. 2800/C

b. 2300/2C

c. 2800/3C

d. 2800/C2

24. On piles, the drop must be at least

a. 100 cm

b. 120 cm

c. 140 cm

d. 150 cm

25. An arch may fail due to

a. uneven settlement of abutments

b. sliding of voussoirs

c. crushing of the material

d. all the above.

26. The local swelling of a finished plaster, is termed

a. cracking

b. dubbing

c. blistering

d. hacking.

27. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

28. Water required per bag of cement, is

a. 14 kg

b. 21 kg

c. 28 kg

d. 35 kg

29. Hard water contains

a. calcium

b. magnesium bicarbonates

c. magnesium sulphate

d. all the above.

30. Pick up the correct statement from the following :

a. Even if ports are closed, the wet intake tower will have water filled up to reservoir level

b. The dry intake towers are generally constructed with heavier construction to resist buoyant forces

c. No buoyant force acts on wet intake towers

d. All the above.

31. The algae dies out in the zone of

a. degradation

b. active decomposition

c. recovery

d. cleaner water.

32. Which one of the following resists hydrogen sulphide corrosion

a. R.C.C.

b. Glazed stone wares

c. Asbestos cement

d. Glazed ware.

33. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

34. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

35. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

36. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

37. The total length of a valley formed by two gradients - 3% and + 2% curve between the two tangent points to provide a rate of change of centrifugal acceleration 0.6 m/sec2, for a design speed 100 km ph, is

a. 16.0 m

b. 42.3 m

c. 84.6 m

d. none of these.

38. Pick up the correct statement from the following:

a. Borrow pits are located out side the right of way

b. Borrow pits may be located on either side of the right of way

c. Spoil bank is located on one side of the right of way

d. All the above.

39. On concrete roads, the camber generally provided, is

a. 1 in 30 to 1 in 48

b. 1 in 36 to 1 in 48

c. 1 in 48 to 1 in 60

d. 1 in 60 to 1 in 72

40. For a properly designed vehicle, the resistance generally ignored, is

a. rolling resistance

b. grade resistance

c. axle resistance

d. none of these.

41. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

42. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

43. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

44. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

45. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

46. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

47. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

48. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

49. According to I.C.A.O. the recommended length of air ports is decided on

a. sea level elevation

b. standard sea level temperature (15°C)

c. effective gradient percentage

d. all the above.

50. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.