***Sub Engineer***

1. If Δ is the angle of deflection of a simple curve of radius R, the length of its long chord, is

a. R cos

b. 2R cos

c. R sin

d. 2R sin .

2. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

3. Pick up the correct statement from the following :

a. the system of a series of lines which forms a circuit which ends at the starting point, is called a closed traverse

b. the traverse that starts from a point already fixed in some survey system and ends on another such point, is called a controlled traverse

c. the traverse that is not controlled is called a fly traverse

d. all the above.

4. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

5. The rocks which are formed due to cooling of magma at a relatively shallow depth from the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Ignoeous rocks.

6. A good quality stone absorbs water less than

a. 5%

b. 10%

c. 15%

d. 20%

7. With storage, strength of cement

a. increases

b. decreases

c. remains the same

d. none to these.

8. Minimum required water cement ratio for a workable concrete, is

a. 0.40

b. 0.50

c. 0.60

d. 1.0.

9. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

10. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

11. A 8 metre long simply supported rectangular beam which carries a distributed load 45 kg/m. experiences a maximum fibre stress 160 kg/cm2. If the moment of inertia of the beam is 640 cm4, the overall depth of the beam is

a. 10 cm

b. 12 cm

c. 15 cm

d. 16 cm

12. If the shear force along a section of a beam is zero, the bending moment at the section is

a. maximum

b. minimum

c. average of maximum-minimum

d. none of these.

13. Hydraulic ram is a device

a. for lifting water without an electric motor

b. for accelerating water flow

c. for lifting heavy loads

d. none of these.

14. For the most economical trapezoidal open channel,

a. half of the top width must be equal to one of the sloping sides

b. the hydraulic mean depth must be equal to half the depth of flow

c. the semicircle drawn with top width as diameter must touch the three sides of the channel

d. All of these.

15. A pipe of 0.1 m2 cross sectional area suddenly enlarges to 0.3 m2 cross-sectional area. If the discharge of the pipe is 0.3 m3 /sec, the head loss is

a. 2/g m of water

b. g/2 m of water

c. 1g m of water

d. g m of water.

16. Total energy line is

a. pressure head

b. datum head

c. kinetic head

d. All the above.

17. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

18. Which one of the following statements is true for Mohr-Coulomb envelope ?

a. The generalised Mohr theory suggests that, though the shear stress depends on the normal stress, the relation is not linear

b. Coulomb and Mohr suggest that a definite relationship exists among the principal stress and the angle of internal friction

c. For an ideal pure friction material, the straight line passes through the origin.

d. All the above.

19. 'Talus' is the soil transported by

a. wind

b. water

c. glacier

d. gravitational force.

20. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

21. In a prestressed beam carrying an external load W with a bent tendon is having angle of inclination θ and prestressed load P. The net downward load at the centre is

a. W - P cos θ

b. W - P sin θ

c.  W - 2P sin θ

d. W + 2P sin θ

22. On piles, the drop must be at least

a. 100 cm

b. 120 cm

c. 140 cm

d. 150 cm

23. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

24. A continuous beam shall be deemed to be a deep beam if the ratio of effective span to overall depth, is

a. 2.5

b. 2.0

c. less than 2

d. less than 2.5

25. The platform at the end of a series of steps, is known as

a. relief

b. rest

c. landing

d. stop.

26. The mortar in which both cement and lime are used as binding materials, is called

a. lime mortar

b. fire resistant mortar

c. gauged mortar

d. light weight mortar.

27. In slump test, each layer of concrete is compacted by a steel rod 60 cm long and of 16 mm diameter for

a. 25 times

b. 30 times

c. 40 times

d. 50 times

28. Pick up the incorrect statement from the following:

a. A rich mix of concrete possesses higher strength than that a lean mix of desired workability with excessive equantity of water

b. The strength of concrete decreases as the water cement ratio increases

c. If the water cement ratio is less than 0.45, the concrete is not workable and causes honey-combed structure

d. Good compaction by mechanical vibrtions, increases the strength of concrete

29. The percentage of chlorine in fresh bleaching powder is roughly

a. 50 to 60

b. 30 to 35

c. 40 to 50

d. 20 to 25

30. The best process of disinfection of public water supply, is by

a. boiling

b. chlorination

c. adding lime

d. adding zone.

31. The reduced levels of the string at the consecutive sight rails A and B are 203.575 m, 203.475 m respectively. If the difference of their R.D.s is 10 m, the gradient of the sewer line is

a. 1 in 100 upward

b. 1 in 500 upward

c. 1 in 100 downward

d. 1 in 503 upward.

32. The pH value of sewage is determined with the help of

a. Imhoff Cone

b. turbidimeter

c. potentiometer

d. none of these.

33. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

34. The ratio of the rate of change of discharge of an outlet to the rate of change in level of water surface in a distributary at its normal depth, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

35. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

36. Canals taken off from ice-fed perennial rivers, are known

a. ridge canals

b. perennial canals

c. inundation canals

d. ice canals.

37. If R is the radius of a main curve and L is the length of the transition curve, the shift of the curve, is

a. L2/24 R

b. L3/24 R

c. L4/24 R

d. L/12 R

38. The maximum radial distance of a Lemniscate curve, having maximum polar angle a, is

a. 3 R sin2α

b. 3 R sin 2α

c. 3 R sin α/2

d. none of these.

39. Volume of traffic which would immediately use a new road or an improved one when opened to traffic, is known

a. development traffic

b. current traffic

c. general traffic

d. normal traffic growth.

40. The convexity provided to the carriageway between the crown and edge of the pavement, is known as

a. super-elevation

b. camber

c. height of the pavement

d. None of these.

41. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

42. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

43. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

44. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

45. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

46. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

47. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

48. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

49. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m

50. The strength of winds is measured with the help of

a. Benfort scale

b. Wind indicator

c. Barometres

d. None of these.