***Sub Engineer***

1. The operation of making the algebraic sum of latitudes and departures of a closed traverse, each equal to zero, is known

a. balancing the sights

b. balancing the departures

c. balancing the latitudes

d. balancing the traverse.

2. Two contour lines, having the same elevation

a. cannot cross each other

b. can cross each other

c. cannot unite together

d. can unite together.

3. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

4. The imaginary line passing through the intersection of cross hairs and the optical centre of the objective, is known as

a. line of sight

b. line of collimation

c. axis of the telescope

d. none of these.

5. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

6. The lime which contains high percentage of calcium oxide, is generally called

a. fat lime

b. rich lime

c. white lime

d. All of these.

7. Pick up the correct statement from the following:

a. Quick lime contains calcium oxide

b. Slaked lime may be obtained from quick lime

c. Slaked lime is obtained by adding water to quick lime

d. All the above.

8. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

9. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3 M

d. depth d ∝

10. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

11. For a stable frame structure, number of members required, is

a. twice the number of joints minus three

b. twice the number of joints minus two

c. twice the number of joints minus one

d. none of these.

12. The bending moment is maximum on a section where shearing force

a. is maximum

b. is minimum

c. is equal

d. changes sign.

13. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

14. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

15. Pick up the incorrect statement from the following :

a. In radial flow, fluid flows such that pressure and velocity at any point change with respect to its distance from the central axis

b. In radial flow, velocity of flow is in a radial direction

c. In radial flow, flow may take place radially inward to or outward from the centre

d. In radial flow, flow is one dimensional with stream lines parallel.

16. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

17. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

18. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

19. The angle between the directions of the failure and the major principal plane, is equal to

a. 90° + effective angle of shearing resistance

b. 90° + half of the angle of shearing resistance

c. 45° - half of the angle of shearing resistance

d. 45° + half of the angle of shearing resistance.

20. The shear resistance of a soil is con-tituted basically of the following component.

a. The frictional resistance to translocation between the individual soil particles at their contact point

b. To the structural relation to displacement of the soil because of the interlocking of the particles

c. Cohesion and adhesion between the surfaces of the soil particles

d. All the above.

21. For M 150 mix concrete, according to I.S. specifications, local bond stress, is

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

22. According to the steel beam theory of doubly reinforced beams

a. compression is resisted by compression steel

b. stress in tension steel equals the stress in compression steel

c. no stress is developed in compression concrete as well as in tension concrete

d. all the above.

23. If R and T are rise and tread of a stair spanning horizontally, the steps are supported by a wall on one side and by a stringer beam on the other side, the steps are designed as beams of width

a. R + T

b. T-R

c. R2 + T2

d. R-T

24. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

25. For providing a raft foundation, the following activities are involved ramming the foundation bedexcavation of the soil upto required depthlaying the reinforcement over the foundation bedcuring the cement concrete placed over reinforcementpouring the cement concrete over the reinforcement. The correct sequence is

a. 1, 2, 3, 4, 5

b. 5, 4, 3, 2, 1

c. 2, 1, 3, 5, 4

d. 3, 2, 5, 1, 4.

26. Dampness causes

a. bleaching of paints

b. crumbling of plaster

c. growth of termites

d. none of these.

27. To hydrate 500kg of cement full water needed, is

a. 110 kg

b. 120 kg

c. 130 kg

d. 140 kg

28. The grade of concrete M 150 means that compressive strength of a 15 cm cube after 28 days, is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

29. The nitrate concentration in domestic water supplies, is generally limited to

a. 15 ppm

b. 25 ppm

c. 30 ppm

d. 45 ppm.

30. The joint used for joining the plain ends of cast iron pipes, is

a. socket and spigot joint .

b. dresser coupling joint

c. flexible joint

d. expansion joint.

31. The design period of sewage treatment works in normally

a. 5 - 10 years

b. 15 - 20 years

c. 30 - 40 years

d. 40 - 50 years.

32. Drop manholes at the junctions of sewer lines, are provided if

a. invert level of a branch sewer is more than 60 cm that of the main sewer

b. sewer line runs along a main road

c. ordinary manhole cannot be built

d. two sewer lines intersect.

33. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

34. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

35. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

36. The main function of a diversion head works of a canal from a river, is

a. to control floods

b. to store water

c. to raise water level

d. all the above.

37. If cross slope of a country is upto 10% the terain is classified as

a. plain

b. rolling

c. mountainous

d. steep.

38. Transverse joints are provided at distances varying from

a. 12 m to 18 m

b. 16 m to 24 m

c. 17 m to 27 m

d. 25 m to 35 m

39. Thickness of a pavement may be reduced considerably by

a. compaction of soil

b. stabilisation of soil

c. drainage of soil

d. combination of all the above.

40. The type of curves generally provided on highways, is

a. critical curve

b. transition curve

c. vertical curve

d. all the above.

41. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

42. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

43. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

44. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

45. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

46. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

47. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

48. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

49. The bearing of the runway at threshold is 290°, the runway number is

a. N 70° W

b. 290°

c. 29°

d. W 20° N

50. Pick up the correct statement from the following:

a. F.I.C. stands for flight information centre

b. Radius of control area is 160 km

c. Radius of control zone is 40 km

d. All the above.