***Sub Engineer***

1. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

2. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

3. If deflection angles are measured in a closed traverse, the difference between the sum of the right-hand and that of the left hand angles should be equal to

a. 0°

b. 90°

c. 180°

d. 360°

4. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

5. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

6. Brass is an alloy of

a. copper and zinc

b. zinc and lead

c. tin and silver

d. zinc and nickel

7. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

8. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

9. Flat spiral springs

a. are supported at outer end

b. are wound by applying a torque

c. are used in clock-work mechanism

d. all the above.

10. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

11. The shape of the bending moment diagram over the length of a beam, having no external load, is always

a. linear

b. parabolic

c. cubical

d. circular.

12. A three hinged arch is loaded with an isolated load 1000 kg at a horizontal distance of 2.5 m from the crown, 1 m above the level of hinges at the supports 10 metres apart. The horizontal thrust is

a. 1250 kg

b. 125 kg

c. 750 kg

d. 2500 kg

13. In a fluid flow a particle may posses

a. kinetic energy

b. pressure energy

c. initial energy

d. all the above.

14. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

15. Weber number is the ratio of inertia force to

a. surface tension

b. gravitational force

c. elasticity

d. viscosity.

16. The pressure less than atmospheric , pressure, is known

a. suction pressure

b. vacuum pressure

c. negative gauge pressure

d. all the above.

17. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

18. The phreatic line in an earth dam may be

a. elliptical

b. parabolic

c. a straight line

d. all the above.

19. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

20. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

21. For a continuous floor slab supported on beams, the ratio of end span length and intermediate span length, is

a. 0.6

b. 0.7

c. 0.8

d. 0.9

22. Side face reinforcement shall be provided in the beam when depth of the web in a beam exceeds

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

23. The load stress of a section can be reduced by

a. increasing the total perimeter of bars

b. replacing larger bars by greater number of small bars

c. replacing smaller bars by greater number of greater bars

d. none of these.

24. According to load factor method, the permissible load W on a short column reinforced with longitudinal bars and lateral stirrups, is

a. Stress in concrete x area of concrete

b. Stress in steel x area of steel

c. Stress in concrete x area of concrete + Stress in steel x area of steel

d. None of these.

25. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

26. A concrete structure is set on fire and the temperature raises to 1000°C. The strength of concrete as compared to original strength reduces to

a. 10%

b. 15%

c. 20%

d. 25%.

27. For the construction of cement concrete floor, the maximum permissible size of aggregate, is

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

28. The cement becomes useless if its absorbed moisture content exceeds

a. 2%

b. 3%

c. 4%

d. 5%

29. By adding 1 ml of orthotolidine solution to 100 ml chlorinated water taken after contact period, the residual chlorine makes the colour of solution

a. yellowish

b. greenish

c. bluish

d. reddish

30. Pick up the incorrect statement from the following. In take of water supply should

a. be nearer to the treatment plant

b. receive water from purer zone of the source

c. be located downstream of waste water disposal point

d. remain easily accessible during floods.

31. The asbestos cement sewers are

a. light in weight

b. not structurally strong to bear large compressive stress

c. susceptible to corrosion by sulphuric acid

d. All the above.

32. The sewer which received discharge from two or more main sewers, is known as

a. a trunk sewer

b. an outfall sewer

c. a main sewer

d. an intercepting sewer.

33. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

34. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

35. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

36. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

37. The convexity provided to the carriageway between the crown and edge of the pavement, is known as

a. super-elevation

b. camber

c. height of the pavement

d. None of these.

38. Reference pillars fixed on the centre line of a proposed road, provide the following information:

a. horizontal distance of road from the centre line

b. reduced level at the top of pillar

c. formation level of the road

d. all the above.

39. Passing zones are generally not provided on

a. summit curves

b. horizontal curves

c. two lane highways

d. all the above.

40. The most commonly adopted method to provide super-elevation on roads, is by pivoting the road surface about

a. outer edge so that the inner edge is lowered

b. crown so that outer edge is raised and inner edge is lowered

c. inner edge so that outer edge is raised

d. none of these.

41. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

42. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

43. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

44. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

45. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

46. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

47. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

48. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

49. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.

50. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)