***Sub Engineer***

1. An internal focussing type surveying telescope, may be focussed by the movement of

a. objective glass of the telescope

b. convex-lens in the telescope

c. concave lens in the telescope

d. plano-convex lens in the telescope.

2. Which one of the following mistakes/errors may be cumulative + or - :

a. bad ranging

b. bad straightening

c. erroneous length of chain

d. sag.

3. The sum of the interior angles of a geometrical figure laid on the surface of the earth differs from that of the corresponding plane figure only to the extent of one second for every

a. 100 sq. km of area

b. 150 sq. km of area

c. 200 sq. km of area

d. none of these.

4. For preparation of a contour plan for a route survey

a. method of squares is used

b. method of trace contour is used

c. method of cross profile is used

d. indirect method of contouring is used.

5. Pick up the correct statement from the following:

a. Catalysts are added to assist and accelerate the hardening of resin,

b. The fillers are inert materials and they impart strength and hardness

c. Fibrous fillers increase thermal resistance

d. All the above.

6. Laterite is found in

a. U.P.

b. Punjab

c. West Bengal

d. Kerala.

7. Pick up the correct statement from the following:

a. soft stones are required for carving

b. light stones are required for arches

c. hard stones are required to stand high pressure

d. All the above.

8. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2.

9. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

10. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

11. A three hinged parabolic arch hinged at the crown and springings, has a horizontal span of 4.8 m and a central rise of 1 m. It carries a uniformly distributed load of 0.75 tonne per metre over half left hand span. The horizontal thrust at the support will be

a. 1.08 tonnes

b. 1.8 tonnes

c. 0.8 tonnes

d. none of these.

12. The moment diagram for a cantilever whose free end is subjected to a bending moment, will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

13. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

14. Pick up the correct statement from the following :

a. Total energy gradient is the graphical representation of the total head at any section of a pipe line

b. Vertical distance between the total energy line and hydraulic grade line is equal to the velocity head

c. Vertical distance between the total energy line and total energy gradient represents loss of head

d. all the above.

15. Pick up the correct statement from the following :

a. Dimensional homogeneity means the dimensions of each term in an equation on both sides are equal

b. Dimensionally homogeneous equations are independent of the system of units

c. In dimensionally homogeneous equation, the powers of fundamental dimensions on either side of the equation are identical

d. All the above.

16. Back water curve is caused if

a. friction head loss is more than the bed slope

b. pressure is due to weir in the channel

c. there is an increase in width of the channel

d. none of these.

17. The critical exist gradient of seepage water in soils, increases with

a. a decrease in specific gravity

b. a decrease in void ratio

c. both (a) and (c)

d. none of these.

18. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

19. Chemical weathering of soil is caused due to

a. carbonation

b. hydration

c. leaching

d. all the above.

20. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

21. According to load factor method, the permissible load W on a short column reinforced with longitudinal bars and lateral stirrups, is

a. Stress in concrete x area of concrete

b. Stress in steel x area of steel

c. Stress in concrete x area of concrete + Stress in steel x area of steel

d. None of these.

22. If the maximum shear stress at the end of a simply supported R.C.C. beam of 16 m effective span is 10 kg/cm2, the length of the beam having nominal reinforcement, is

a. 4 cm

b. 6 m

c. 8 m

d. 10 m

23. The zone in which transverse bending is likely to occur may be obtained by drawing a line from the faces of the column making an angle θ° with horizontal where θ° is

a. 30°

b. 45°

c. 60°

d. none of these.

24. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

25. In grillage foundations, distance between flanges of grillage beams, is kept

a. 40 cm

b. equal to flange width

c. twice the flange width

d. maximum of (a), (b) and (c).

26. In clay soil

a. swelling and shrinkage characteristics prevail

b. consolidation continues even after several years of construction.

c. differential settlement is generally prevalent

d. all the above.

27. For construction of structures in sea water, the cement generally preferred to, is

a. Portland-pozzolana cement

b. quick setting cement

c. low heat Portland cement

d. rapid hardening cement

28. Pick up the correct statement from the following:

a. Water and aggregates are measured in litres

b. The finished concrete is measured in cubic metres

c. 20 bags of cement make one tonne

d. All the above.

29. Surface water is obtained from

a. well

b. springs

c. artesian well

d. rain.

30. Pick up the correct statement from the following :

a. Wrought iron pipes are lighter than cast iron pipes

b. Wrought iron pipes corrode quickly and are less durable

c. Wrought iron pipes coated with zinc, are called galvanised iron pipes

d. All the above.

31. For a peak discharge of 0.0157 cumec, with a velocity of 0.9 m/sec, the diameter of the sewer main, is

a. 12 cm

b. 15 cm

c. 18 cm

d. 20 cm.

32. If D is the diameter of a circular sewer and D' is the top horizontal diameter of an equivalent egg shaped section, the relationship which holds good, is

a. D' = 0.64 D

b. D' = 0.74 D

c. D' = 0.84 D

d. D' = 0.94 D.

33. Borrow pits should preferably be located in

a. field on the left side of the canal

b. field on the right side of the canal

c. fields on both sides of the canal

d. central half width of the section of the canal.

34. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

35. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

36. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

37. Stability of hill slopes depends upon

a. angle of the slope

b. geological conditions

c. ground water conditions

d. all the above.

38. The distance travelled by revolving the wheel of a vehicle more than its circumferential movement, is known as

a. slip

b. skid

c. neither (a) nor (b)

d. both (a) and (b)

39. Thickness of a pavement may be reduced considerably by

a. compaction of soil

b. stabilisation of soil

c. drainage of soil

d. combination of all the above.

40. Concrete pavement is provided if daily traffic per lane exceeds

a. 750 tonnes

b. 1000 tonnes

c. 1250 tonnes

d. 1300 tonnes

41. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

42. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

43. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

44. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

45. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

46. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

47. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

48. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

49. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.

50. Two single runways may be arranged so as to have

a. L-shape

b. T-shape

c. X-shape

d. All the above.