***Sub Engineer***

1. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

2. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

3. Setting out a curve by two theodolite method, involves

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. none of these.

4. Which one of the following procedures for getting accurate orientation is the most distinctive feature of the art of plane tabling

a. radiation

b. intersection

c. traversing

d. resection.

5. Lime concrete is generally used for

a. wall foundations

b. flooring at ground level

c. both (a) and (b)

d. neither (a) nor (b).

6. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

7. Rapid hardening cement contains

a. Tri-calcium silicate

b. Tri-calcium aluminate

c. Tetra-calcium alumino-ferrite

d. Dicalcium silicate.

8. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

9. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

10. Pick up the correct statement from the following:

a. The structural member subjected to compression and whose dimensions are small as compared to its length, is called a stmt

b. The vertical compression members, are generally known as columns or stanchions

c. Deflection in lateral direction of a long column, is generally known as buckling

d. All the above.

11. An arch with three hinges, is a structure

a. statically determinate

b. statically indeterminate

c. geometrically unstable

d. structurally sound but indeterminate

12. The ratio of the effective length of a column and minimum radius of gyration of its cross-sectional area, is known

a. buckling factor

b. slenderness ratio

c. crippling factor

d. none of these.

13. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

14. For critical depth of flow of water in open channels, fc the specific energy must be :

a. minimum

b. maximum

c. average of maximum and minimum

d. None of these.

15. In case of laminar flow through a circular pipe,

a. momentum correction factor is 1.33

b. energy correction factor is 2.00

c. both (a) and (b)

d. Neither (a) nor (b).

16. In Newton's viscosity equation , the coefficient (μ) is known as coefficient of

a. absolute viscosity

b. dynamic viscosity

c. simple viscosity

d. all the above.

17. The length/diameter ratio of cylindrical specimens used in triaxial test, is generally

a. 1.5

b. 2

c. 2.5

d. 3

18. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

19. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

20. The ratio of the difference between the void ratio of the soil in its loosest state and its natural void ratio (e) to the difference between the void ratios in the loosest and fully dense state, is generally termed as

a. degree of density

b. relativity

c. density index

d. all the above.

21. The system in which high tensile alloy steel bars (silica manganese steel) are used as prestressing tendons, is known as

a. Freyssinet system

b. Magnel-Blaton system

c. C.C.L. standard system

d. Lee-McCall system.

22. If the maximum shear stress at the end of a simply supported R.C.C. beam of 16 m effective span is 10 kg/cm2, the length of the beam having nominal reinforcement, is

a. 4 cm

b. 6 m

c. 8 m

d. 10 m

23. Lapped splices in tensile reinforcement are generally not used for bars of size larger than

a. 24 mm diameter

b. 30 mm diameter

c. 36 mm diameter

d. 32 mm diameter

24. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

25. The method of moving each brick through a small horizontal distance before it is finally laid in any brick wall and pressing it by means of brick hammer, is known as

a. trowelling

b. laying

c. grouting

d. placing.

26. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

27. Pick up the correct statement from the following:

a. Sand obtained from flooded pits, need not be washed before use

b. Sea shore sand contains chlorides which cause effloresance

c. The chloride in sea shore sand and shingle may cause corrosion of reinforcement if the concrete is porous

d. All the above.

28. Pick up the correct statement from the following:

a. Angular and rough aggregates reduce the workability of the concrete

b. Large size aggregates increase the workability due to lesser surface area

c. The slump of the concrete mix decreases due to an increase in temperature

d. All the above.

29. The maximum hourly consumption, is generally taken as

a. 120%

b. 130%

c. 140%

d. 150%.

30. Normal values of overflow rate for plain sedimentation tanks in litres/hr/m2, generally range between

a. 100 to 250

b. 250 to 500

c. 500 to 750

d. 750 to 1000

31. Rate of flow of sewage is generally assumed

a. more than the rate of water supply

b. equal to the rate of water supply

c. less than the rate of water supply

d. at 150 litres per capita.

32. For estimating the peak run off the rational formula Q = 0.0278 KpA was evolved by

a. Kinchling

b. Lloyd Davis

c. Frubling

d. all the above.

33. The sinuosity of a meander is the ratio of

a. meander length and the width of meander

b. meander length and half width of the river

c. curved length and the straight distance

d. none of these.

34. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

35. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

36. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

37. If a Lemniscate curve having an angle of deflection Δ, is transitional throughout, the maximum polar angle is

a. Δ/3

b. Δ/4

c. Δ/5

d. Δ/6

38. Minimum number of 50 kg cement bags per cubic metre of concrete for a mix corresponding to crushing strength 280 kg/cm2 at 28 days, are

a. 6.5

b. 7.0

c. 7.5

d. 8.0

39. For maximum strength and durability minimum percentage of cement, by weight is

a. 20%

b. 25%

c. 30%

d. 33%

40. If brakes of vehicles are effective, the vehicle-running at 30 km/hour comes to a stop in

a. 10 metres

b. 12 metres

c. 15 metres

d. 18 metres

41. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

42. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

43. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

44. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

45. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

46. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

47. Optimistic time, most likely time and pessimistic times for the activities of a network in the given figure are written above their arrows. If the contractual obligation time for the project is 75, the latest occurrence time for the event 2, is

a. 20

b. 25

c. 35

d. 15

48. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

49. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.