***Sub Engineer***

1. The branch of surveying in which both horizontal and vertical positions of a point, are determined by making instrumental observations, is known

a. tacheometry

b. tachemetry

c. telemetry

d. all the above.

2. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

3. From any point on the surface with a given inclination

a. only one contour gradient is possible

b. two contour gradients are possible

c. indefinite contour gradients are possible

d. all the above.

4. The chaining on sloping ground is

a. easier along the falling gradient

b. easier along the up gradient

c. equally convenient along falling as well as up gradient

d. all the above.

5. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

6. The lime which contains mainly calcium oxide and slacks with water, is

a. quick lime

b. hydraulic lime

c. poor lime

d. none of these.

7. Gniess is obtained from

a. igneous rocks

b. metamorphic rocks

c. sedimentary rocks

d. sedimentary metamorphic rocks.

8. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

9. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

10.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. In a beam, the neutral plane

a. may be its centre

b. passes through the C.G. of the area of cross-section

c. does not change during deformation

d. none of these.

12. A beam is said to be of uniform strength, if

a. deflection is same throughout the beam

b. bending stress is same throughout the beam

c. shear stress is same throughout the beam

d. none of these.

13. The flow in open channel is laminar if the Reynold number is

a. less than 500

b. more than 500

c. 1000

d. none of these.

14. If jet of water coming out from a nozzle with a velocity 9.81 m/s, the angle of elevation being 30°, the time to reach the highest point is

a. 0.25 s

b. 0.50 s

c. 1.0 s

d. 1.5 s.

15. In a two dimensional flow if the components of the velocity are u = ax ; v = by, the point where no motion occurs, is known as

a. neutral point

b. stagnation point

c. stationary point

d. none of these.

16. The instrument used for measuring the velocity of flow, is known as

a. venturimeter

b. orifice meter

c. pitot tube

d. none of these.

17. Hydrometer readings are corrected for:

a. meniscus correction

b. dispersing agent correction

c. meniscus and dispersing agent corrections

d. temperature, meniscus and dispersing agent corrections.

18. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

19. Degree of saturation of a natural soil deposit having water content 15%, specific gravity 2.50 and void ratio 0.5, is

a. 50%

b. 60%

c. 75%

d. 80%

20. The coefficient of compressibility of soil, is the ratio of

a. stress to strain

b. strain to stress

c. stress to settlement

d. rate of loading to that of settlement.

21. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

22. The minimum head room over a stair must be

a. 205 cm

b. 210 cm

c. 200 cm

d. 230 cm

23. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

24. If R and T are rise and tread of a stair spanning horizontally, the steps are supported by a wall on one side and by a stringer beam on the other side, the steps are designed as beams of width

a. R + T

b. T-R

c. R2 + T2

d. R-T

25. According to National Building Code, the hydrants in water mains is provided at minimum interval of

a. 50 m

b. 60 m

c. 75 m

d. 90 m.

26. In horizontal D.P.C, thickness of cement concrete (1 : 2 : 4) is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

27. Pick up the correct statement from the following:

a. Construction joints in columns are provided at the bottom haunching

b. Construction joints in beams and slabs are provided within middel third

c. Constructionjoints are generally provided in positions subjected to least shear force

d. All the above.

28. Pick up the correct statement from the following:

a. High percentage of C3S and low percentage of C2S make the cement less resistive to chemical attack

b. Low percentage of C3S and high percentage of C2S contribute to slow hardening

c. Low percentage of C3S and high percentage of C2S provide greater resistance to chemical attack

d. All the above.

29. The maximum depth of sedimentation tanks is limited to

a. 3 m

b. 4 m

c. 5 m

d. 6 m.

30. Low turbidity of water is detected by

a. Jackson turbidity meter

b. Baylis turbidimeter

c. Hellipe turbidimeter

d. None of these.

31. Clogging of sewers, is caused due to

a. low discharge

b. greasy and oily matters

c. domestic wastes thrown in manholes

d. all the above.

32. Large diameter sewers subjected to external pressure alone, are reinforced

a. near the outer surface of the pipe

b. both (a) and (b)

c. with elliptical cage

d. none of these.

33. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

34. When a canal is carried over a natural drainage, the structure provided, is known as

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

35. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

36. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

37. Selection of the routes, of highways depends upon

a. avoidance of cutting hard rocks

b. minimum number of bridges

c. minimum possible length of alignment

d. all the above.

38. In a right angle bend of a road provided with a transition throughout, the maximum polar angle will be

a. 15°

b. 20°

c. 30°

d. 45°

39. Intermediate catch water drains are provided only, if

a. catchment area of the watershed above road is large

b. intensity of rainfall is heavy

c. single catch water drain is inadequate

d. all the above.

40. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

41. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

42. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

43. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

44. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

45. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

46. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

47. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

48. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

49. Pick up the correct statement from the following:

a. Approach areas are measured in horizontal surfaces

b. The imaginary inclined plane which is directly above the approach area is called approach surface

c. Obstruction clearance surface and approach surface are same

d. All the above.

50. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m