***Sub Engineer***

1. One of the tacheometric constants is additive, the other constant, is

a. subtractive constant

b. multiplying constant

c. dividing constant

d. indicative constant.

2. Pick up the correct statement from the following :

a. the power of a lens is the reciprocal of its focal length

b. the unit of power of the lens is diopter

c. the power of two or more thin lenses in contact is the power of the combination of the lenses

d. all the above.

3. The bearing of AB is 190° and that of CB is 260° 30'. The included angle ABC, is

a. 80° 30'

b. 99° 30'

c. 70° 30'

d. none of these

4. The sensitiveness of a level tube decreases if

a. radius of curvature of its inner surface is increased

b. diameter of the tube is increased

c. length of the vapour bubble is increased

d. both viscosity and surface tension are increased.

5. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

6. Stainless steel contains

a. 18% of chromuim and 8% nickel

b. 8% of chromium and 18% of nickel

c. 12% of chromium and 36% of nickel

d. 36% of chromium and 12% of nickel.

7. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

8. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

9. The locus of the moment of inertia about inclined axes to the principal axis, is

a. straight line

b. parabola

c. circle

d. ellipse.

10. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

11. The reaction at support A of the beam shown in below figure, is

a. zero

b. 5 T

c. 10 T

d. 1 T

12. The region of the cross-section of a column in which compressive load may be applied without producing any tensile stress, is known as the core of the cross-section. In circular columns the radius of the core, is

a. one-third of the radius

b. one-quarter of the radius

c. one-fifth of the radius

d. one-sixth of the radius.

13. While applying the Bernoulli's equation  = total head, the work any section done on the flow system, if any

a. is added on the right side of the equation

b. is added on the left side of the equation

c. is ignored

d. none of these.

14. The best side slope for most economical trapezoidal section, is

a. 30°

b. 45°

c. 60°

d. None of these.

15. For a long pipe, the head loss

a. at the entrance is ignored

b. at the outlet is ignored

c. at the entrance and outlet both are ignored

d. due to friction is ignored.

16. The length of hydraulic jump is roughly

a. 2 to 3 times its height

b. 3 to 5 times its height

c. 5 to 7 times its height

d. None of these.

17. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

18. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

19. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

20. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

21. As per IS : 456, the reinforcement in a column should not be less than

a. 0.6% and not more than 6% of cross-see-tional area

b. 0.7% and not more than 7% of cross-sec-tional area

c. 0.8% and not more than 8% of cross-sectional area

d. none of these.

22. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

23. The zone in which transverse bending is likely to occur may be obtained by drawing a line from the faces of the column making an angle θ° with horizontal where θ° is

a. 30°

b. 45°

c. 60°

d. none of these.

24. The angle of internal friction of soil mass is the angle whose

a. tangent is equal to the rate of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

b. sine is equal to the ratio of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

c. cosine is equal to the ratio of the maximum resistance sliding on any internal inclined plane to the normal pressure acting on the plane

d. none of these.

25. Dado is usually provided in

a. bath rooms

b. living rooms

c. verandah

d. roofs.

26. The formWork including the props can be removed from beams, only after

a. 3 day

b. 7 days

c. 14 days

d. 21 days.

27. If the average compressive strength is 4000 kg/cm2 and standard deviation is 500, the co-efficient of variation is

a. 12.5%

b. 15%

c. 18.5 %

d. 20%

28. The grade of concrete not recommended by I.S. : 456, is

a. M 200

b. M 300

c. M 400

d. M 500

29. For plain chlorination of water, the quantity of chlorine used, is

a. 0.2 mg/litre

b. 0.3 mg/litre

c. 0.4 mg/litre

d. 0.5 mg/litre.

30. Distribution mains of any water supply, is normally designed for its average daily requirement

a. 100%

b. 150%

c. 200%

d. 225%.

31. The maximum diameter of sewers adopted in the designs is

a. 1.0 m

b. 2.0 m

c. 3.0 m

d. 4.0 m.

32. D is the diameter of a circular sewer and α is the side of a square section sewer. If both are hydraulically equivalent, the relationship which holds good, is

a. πD8/3 = 4 b8/3

b. πD3/8 = 4 b3/8

c. πD2/3 = 4 b2/3

d. πD3/2 = 4 b3/2

33. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

34. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

35. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

36. If the irrigation efficiency is 80%, conveyance losses are 20% and the actual depth of watering is 16 cm, the depth of water required at the canal outlet, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

37. The absolute minimum sight distance required for stopping a vehicle moving with a speed of 80 km ph, is

a. 120 m

b. 200 m

c. 640 m

d. none of these.

38. In retaining and breast walls, weep holes are provided at

a. 50 cm vertical height and 50 cm centre to centre horizontally

b. 100 cm vertical height and 100 cm centre to centre horizontally

c. 100 cm vertical height and 120 cm centre to centre horizontally

d. 120 cm vertical height and 100 cm centre to centre horizontally.

39. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

40. The maximum comfortable retardation applied to moving vehicles, is

a. 3.42 m/sec2

b. 4.42 m/sec2

c. 5.56 m/sec2

d. 7.80 m/sec2

41. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

42. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

43. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

44. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

45. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

46. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

47. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

48. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

49. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50

50. Pick up the correct statement from the following:

a. Approach areas are measured in horizontal surfaces

b. The imaginary inclined plane which is directly above the approach area is called approach surface

c. Obstruction clearance surface and approach surface are same

d. All the above.