***Sub Engineer***

1. The distances AC and BC are measured from two fixed points A and B whose distance AB is known. The point C is plotted by intersection. This method is generally adopted in

a. chain surveying

b. traverse method of surveys

c. triangulation

d. none of these.

2.  Hydrographic surveys deal with the mapping of

a. large water bodies

b. heavenly bodies

c. mountainous region

d. canal system

3. Number of subdivisions per metre length of a levelling staff is

a. 100

b. 200

c. 500

d. 1000

4. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

5. Pick up the correct statement from the following:

a. The plastic sheets are made by the calendering process in which the plastic material is allowed to pass between cylindrical rollers

b. The application of thermo-setting resins on sheets of paper, is called laminating process

c. The plastic articles made by placing raw material in the desired moulds, is known as moulding process

d. All the above.

6. In a rock calcium carbonate predominates. State whether it is :

a. Sillicious rock

b. Argillaceous rock

c. Calcareous rock

d. None of these.

7. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

8. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

9. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

10. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

11. In a three hinged arch, the third hinge is generally kept at

a. crown of the arch

b. midpoint of the crown and left support hinge

c. midpoint of the crown and right support hinge

d. none of these.

12. When a rectangular beam is loaded transversely, the maximum compressive stress develops on

a. bottom fibre

b. top fibre

c. neutral axis

d. every cross-section.

13. The height of water level in a tank above the centre of a circular hole 2.5 cm in diameter is 50 m. The velocity of water flowing through the hole, is

a. 31.1 m/sec

b. 31.2 m/sec

c. 31.3 m/sec

d. 31.4 m/sec.

14. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

15. The phenomenon occuring in an open channel when a rapidly flowing stream abruptly changes to a slowly flowing stream causing a distinct rise of liquid surface, is

a. water hammer

b. hydraulic jump

c. critical discharge

d. none of these.

16. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

17. The ratio of the volume of voids to the total volume of the given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

18. Water formed transported soil is

a. alluvial

b. marine

c. lacustrine

d. loess.

19. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

20. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

21. A very comfortable type of stairs is

a. straight

b. dog legged

c. geometrical

d. open newel.

22. The stresses developed in concrete and steel in reinforced concrete beam 25 cm width and 70 cm effective depth, are 62.5 kg/cm2 and 250 kg/cm2 respectively. If m = 15, the depth of its neutral axis is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

23. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

24. A flat slab is supported

a. on columns

b. on beams and columns

c. on columns monolithicaily built with slab

d. all the above

25. The additional piles which are driven to increase the capacity of supporting loads on vertical piles, are known

a. raking piles

b. eccentric piles

c. sinking piles

d. none of these.

26. The highest line of sloping roof, where two oposite slopes meet, is known as

a. rafter

b. ridge

c. crown

d. eave.

27. For quality control of Portland cement, the test essentially done is

a. soundness

b. tensile strength

c. consistency

d. all the above.

28. The cement whose strength is a little lower than the ordinary cement during the first three months but attains afterwards the same strength, is known as

a. low-heat Portland cement

b. rapid hardening Portland cement

c. Portland blast slag cement

d. Portland pozzolana cement

29. Pick up the incorrect statement from the following :

a. Higher value of pH means lower hydrogen ion concentration

b. Lower value of pH means higher hydrogen ion contraction

c. Lower value of pH gives alkaline solution

d. None of these.

30. Abyssinian tube well is a special type of

a. slotted type wells

b. cavity type wells

c. strainer type well

d. none of these.

31. Assertion (A) : Discharging the effluents from the oxidation ponds just up stream of lakes or reservoirs is undesirable. Reason (R) : The discharged algae get settled in the reservoirs and cause anaerobic decomposition and other water qualities.

a. Both A and R are true and R is the correct explanation of A

b. Both A and R are true but R is not a correct explanation of A

c. A is true but R is false

d. A is false but R is true.

32. The spacing of bars for perforations in coarse screens used for the treatment of sewage, is

a. 30 mm

b. 40 mm

c. 50 mm

d. 100 mm.

33. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

34. An outlet is said to be proportional if its flexibility, is

a. zero

b. less than one

c. more than one

d. one

35. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

36. F.S.L. of a canal at its head with respect to parent channel is kept

a. at the same level

b. 15 cm lower

c. 15 cm higher

d. none of these.

37. First operation during the detailed survey of a hill road, is

a. adjustment of alignment along with curves

b. derivation of longitudinal and cross-sections

c. fixation of Bench Marks

d. staking the ground.

38. The absolute minimum radius of horizontal curve for a design speed 60 km ph is

a. 131 m

b. 210 m

c. 360 m

d. none of these.

39. The head of public works department of any Indian state, is

a. Transport Minister

b. Chief Engineer

c. Superintending Engineer

d. Executive Engineer.

40. Newly constructed pavement with ordinary Portland cement attains its working strength after

a. 14 days

b. 21 days

c. 28 days

d. 42 days

41. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

42. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

43. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

44. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

45. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

48. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

49. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.

50. A gradient of + 0.08% is followed by a gradient of - 0.07%. If the permissible rate of change of grade is 0.003 per 30 metres, the length of the transition curve, is

a. 150 m

b. 140 m

c. 160 m

d. 175 m