***Sub Engineer***

1. Ranging is an operation of

a. reconnaissance

b. judging the distance

c. determination of slope

d. establishing intermediate points between terminals.

2. Accuracy of elevation of various points obtained from contour map is limited to

a.  of the contour interval

b.  th of the contour interval

c.  rd of the contour interval

d.  th of the contour interval.

3. For the construction of highway (or railway)

a. longitudinal sections are required

b. cross sections are required

c. both longitudinal and cross sections are required

d. none of these.

4. A bearing of a line is also known as

a. magnetic bearing

b. true bearing

c. azimuth

d. reduced bearing

5. The steel used for rails under heavy traffic and on sharp curves, is

a. Nickel steel

b. Chrome steel

c. Magnese steel

d. Vanadium steel.

6. Pick up correct statement from the following:

a. Fibre boards are used for thermal and acoustic control

b. Fibre boards are used for light weight standing members

c. Fibre boards are obtained by impregnating a resin product on fibres

d. all the above.

7. Pick up the correct statement from the following:

a. The phenol is either extracted from coal-tar or prepared from benzene

b. Phenol reacts with formaldehyde, to form phenol formaldehyde resin

c. The plastics prepared from phenol-formaldehyde are used for paints, varnishes, w.c. seats

d. All the above.

8. Pig iron made from heamatite ores free from sulphur, phosphorus and copper, is known as

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig

9. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

10. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

11. When a rectangular beam is loaded transversely, the maximum compressive stress develops on

a. bottom fibre

b. top fibre

c. neutral axis

d. every cross-section.

12. The slenderness ratio of a vertical column of square cross- section of 10 cm side and 500 cm long, is

a. 17.3

b. 173.2

c. 137.2

d. 13.72

13. To measure very low pressure, we use

a. barometers

b. piezometers

c. manometers

d. differential manometers.

14. The value of kinetic energy correction factor (a) for a laminar flow through a circular pipe, is

a. 1.0

b. l.5

c. 2.0

d. 2.5

15. If the pressure at the inlet of a pipe is 90 kg/cm2 and pressure drop over the pipe line is 10 kg/cm2, the efficiency of transmission, is

a. 66.6%

b. 77.7%

c. 55.5%

d. 88.8%.

16. In order to avoid capillary correction, the minimum diameter of a manometer used for measuring pressure, should be

a. 4 mm

b. 6 mm

c. 8 mm

d. 10 mm.

17. 'Loess' is silty clay formed by the action of

a. water

b. glacier

c. wind

d. gravitational force.

18. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

19. If the coefficients of volume change and compressibility of a soil sample are respectively 6.75 x 10-2 and 3 x 10-2, the void ratio of the soil sample, is

a. 1.15

b. 1.20

c. 1.25

d. 1.30

20. For a homogeneous earth dam 50 m high having 2 m free broad, a flow net was constructed and the results were : Number of potential drops = 2.4 Number of flow channels = 0.4. If coefficiency of permeability of the dam mateiral is 3 x 10-3 cm3/sec, the discharge per metre length of dam, is

a. 12 x 10-5 m3/sec

b. 24 x 10-3 m3/sec

c. 6 x 10-5 m3/sec

d. 24 x 10-5 m3/sec

21. An R.C.C. column of 30 cm diameter is reinforced with 6 bars 12 mm φ placed symmetrically along the circumference. If it carries a load of 40, 000 kg axially, the stress is

a. 49.9 kg/cm2

b. 100 kg/cm2

c. 250 kg/cm2

d. 175 kg/cm2

22. In a slab, the pitch of the main reinforcement should not exceed its effective depth

a. three times

b. four times

c. five times

d. two times.

23. The width of the flange of a L-beam, should be less than

a. one-sixth of the effective span

b. breadth of the rib + four times thickness of the slab

c. breadth of the rib + half clear distance between ribs

d. least of the above.

24. Spacing of stirrups in a rectangular beam, is

a. kept constant throughout the length

b. decreased towards the centre of the beam

c. increased at the ends

d. increased at the centre of the beam.

25. The bearing capacity of piles is determined by

a. dynamic formula

b. static formula

c. pile load tests

d. all the above.

26. Exposed portions of vertical surface at right angles to the door or window frame, are known as

a. jambs

b. lintels

c. reveals

d. soffits.

27. Water cement ratio is generally expressed in volume of water required per

a. 20 kg

b. 30 kg

c. 40 kg

d. 50 kg

28. For preparing a test-specimen, it is necessary

a. to mix coarse aggregates

b. to mix water to the cement, fine aggregates and coarse aggregates

c. to oil inner surface of the mould and the base plate, before placing concrete

d. all the above.

29. Pick up the incorrect statement from the following :

a. The flow in cavity type wells in spherical

b. In strainer type wells, area of flow depends upon the length of the strainer pipe

c. In cavity type wells, area of flow depends upon the size of the cavity

d. None of these.

30. Quality of water is said to be good if it is

a. colourless

b. free from pathogenic organism

c. tasteless

d. all the above.

31. Faculative bacterias survive in

a. the presence of oxygen

b. the absence of oxygen

c. both cases (a) and (b)

d. neither (a) nor (b).

32. Pick up the correct statement from the following :

a. anaerobic bacteria flourish in the absence of oxygen

b. aerobic bacteria flourish in the presence of oxygen

c. facultative bacteria flourish with or without oxygen

d. all the above.

33. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

34. A hydraulic structure is designed to withstand

a. seepage forces

b. hydraulic jump

c. hydraulic pressure

d. all the above.

35. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 H + d

b. B = 0.33 H + d

c. B = 0.44 H + d

d. B = 0.55 H + d

36. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

37. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

38. Super-elevation on roads in snow bound areas, should generally not exceed

a. 12%

b. 10%

c. 7%

d. 6%

39. The minimum vertical clearance of overhanging cliffs or any other structure above the surface of a hill road, should be

a. 4 m

b. 5 m

c. 6 m

d. 4.5 m

40. A single lane carriage way whenever changes to two-lane carriage way, is affected through a taper of

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 15 to 1 in 20

41. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

44. The expected out turn of brick work in cement mortar in foundation and plinth per mason per day, is

a. 1.00 m3

b. 1.25 m3

c. 1.50 m3

d. 1.75 m3

45. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

46. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

47. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

48. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

49. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.