***Sub Engineer***

1. In chain surveying field work is limited to

a. linear measurements only

b. angular measurements only

c. both linear and angular measurements

d. all the above.

2. A lemniscate curve will not be transitional throughout, if its deflection angle, is

a. 45°

b. 60°

c. 90°

d. 120°

3. It θ is the probable error of an observed bearing of a line of length l, the error over the whole length of the traverse of n lines of length l is

a. l n

b.  n

c. θ

d.  θ n

4. From any point on the surface with a given inclination

a. only one contour gradient is possible

b. two contour gradients are possible

c. indefinite contour gradients are possible

d. all the above.

5. Pick up the correct statement from the following:

a. During rusting, first ferrous bicarbonates are formed

b. On further oxidation ferrous bicarbonates get converted to ferric bicarbonates

c. Ultimately hydrated ferric oxide is formed during rusting and carbon dioxide gets liberated

d. All the above.

6. Good quality cement contains higher percentage of

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite

7. The compound of Portland cement which reacts immediately with water and also sets first is

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

8. The kiln which may work throughout the year, is

a. Clamp

b. Bull's kiln

c. Hoffman's kiln

d. none of these.

9. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

10. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

11. The reaction at support A of the beam shown in below figure, is

a. zero

b. 5 T

c. 10 T

d. 1 T

12. In a square beam loaded longitudinally, shear develops

a. on lower fibre along horizontal plane

b. on top fibre along vertical plane

c. equally on each fibre along horizontal plane

d. none of these.

13. The acceleration f required to accelerate a rectangular tank containing water horizontally so that the slope of its free surface is 45°, is

a. g

b. 2 g

c. 2.5 g

d. 3 g.

14. The discharg formula Q = Cd 2gH x A is used for rectangular

a. small orifices only

b. large orifices only

c. small and large orifices only

d. for all types of orifices.

15. Equation of continuity of fluids is applicable only if

a. flow is compressive

b. flow is one dimensional

c. velocity is uniform over the cross-sections

d. all the above.

16. Uniform flow is said to occur when

a. size and shape of the cross-section in a particular length remain constant

b. size and shape of the cross-section change along a length

c. frictional loss in the particular length of the channel will the more than the drop in its elevation

d. frictional loss in the particular length of the channel, will be less than the drop in elevation.

17. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

18. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

19. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

20. Chemical weathering of soil is caused due to

a. carbonation

b. hydration

c. leaching

d. all the above.

21. The shear reinforcement in R.C.C. is provided to resist

a. vertical shear

b. horizontal shear

c. diagonal compression

d. diagonal tension.

22. An R.C.C. beam of 6 m span is 30 cm wide and has a lever arm of 55 cm. If it carries a U.D.L. of 12 t per m and allowable shear stress is 5 kg/cm2, the beam

a. is safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined bars

d. needs revision of section

23. If diameter of a reinforcement bar is d, the anchorge value of the hook is

a. 8d

b. 12d

c. 16d

d. none of these.

24. An R.C.C. beam not provided with shear reinforcement may develop cracks in its bottom inclined roughly to the horizontal at

a. 35°

b. 45°

c. 55°

d. 60°

25. Safe bearing capacity of black cotton soil varies from

a. 2 to 3 t/m2

b. 5 to 7.5 t/m2

c. 8 to 10 t/m2

d. 10 to 12 t/m2.

26. The foundations are placed below ground level, to increase

a. strength

b. workability

c. stability of structure

d. all the above.

27. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

28. Water cement ratio is generally expressed in volume of water required per

a. 20 kg

b. 30 kg

c. 40 kg

d. 50 kg

29. The maximum pressure to which cast iron pipes may be subjected to, is

a. 3 kg/cm2

b. 5 kg/cm2

c. 7 kg/cm2

d. 10 kg/cm2.

30. The intake opening is generally covered by a screen to prevent entry of debris etc. and its level is kept

a. at the level of water of the source

b. at the bottom of water of the source

c. at about 2.5 m above the bottom

d. none of these.

31. The rate of accumulation of sludge per person per year, is

a. 15 litres

b. 20 litres

c. 25 litres

d. 30 litres.

32. Discrete or granular particles change their

a. size

b. shape

c. weight

d. none of these.

33. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

34. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

35. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

36. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

37. The road foundation for modern highways construction, was developed by

a. Telford

b. Macadam

c. Tresguet and Telford simultaneously

d. Telford and Macadam simultaneously.

38. Setting out of Lemniscate transition curves, is done with

a. perpendicular offsets

b. radial offsets

c. deflection angles

d. polar deflection angles.

39. While calculating the overtaking sight distance, the height of the object above road surface, is assumed

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

40. Maximum super-elevation on hill roads should not exceed

a. 7%

b. 8%

c. 10%

d. 15%

41. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

42. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

43. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

44. For the construction of buildings, the subheads of the estimate are

a. Brickwork, Stone work, Roofing

b. Brickwork Flooring, Wood work, Steel work

c. Plastering or pointing, finishing, water supply and sanitary work

d. All the above.

45. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

46. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

47. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

48. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

49. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.