***Sub Engineer***

1. Accuracy of 'fix' by two point problem, is

a. bad

b. good

c. not reliable

d. unique.

2. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

3. The 'fix' of a plane table from three known points, is good, if

a. middle station is nearest

b. middle station is farthest

c. either the right or left station is nearest

d. none of these.

4. An internal focussing type surveying telescope, may be focussed by the movement of

a. objective glass of the telescope

b. convex-lens in the telescope

c. concave lens in the telescope

d. plano-convex lens in the telescope.

5. Pick up the correct statement from the following:

a. The commercial plywoods are available upto 150 cm wide and upto 300 cm long

b. The plywoods possess uniform tensile strength in all directions

c. The plywoods are light in weight

d. All the above.

6. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

7. The steel used for the manufacture of rails, is

a. Bessemer steel

b. mild steel

c. cast steel

d. stainless steel.

8. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

9. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

10. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

11. When a rectangular beam is loaded longitudinally, shear develops on

a. bottom fibre

b. top fibre

c. middle fibre

d. every-horizontal plane.

12. For a stable frame structure, number of members required, is

a. twice the number of joints minus three

b. twice the number of joints minus two

c. twice the number of joints minus one

d. none of these.

13. The side slope of Cipolletti weir is generally kept

a. 1 to 4

b. 1 to 3

c. 1 to 2

d. 1 : 5

14. If cohesion between the molecules of a fluid is more than adhesion between the fluid and glass, the free level of fluid in a dipped glass tube will be

a. higher than the surface of liquid

b. same as the surface of liquid

c. lower than the surface of liquid

d. none of these.

15. Mercury is generally used in barometers because

a. the height of the barometer will be less

b. it is a best liquid

c. both (a) and (b) above

d. both (b) and (c) above.

16. The main assumption of Bernoulli's equation is :

a. The velocity of energy of liquid particle, across any cross-section of a pipe is uniform

b. No external force except the gravity acts on the liquid

c. There is no loss of energy of the liquid while flowing

d. All the above.

17. The equation τ = C + σ tan φ is given by

a. Rankine

b. Coulomb

c. Culaman

d. Mohr.

18. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

19. Hydrometer readings are corrected for:

a. meniscus correction

b. dispersing agent correction

c. meniscus and dispersing agent corrections

d. temperature, meniscus and dispersing agent corrections.

20. A fundamental equation of void ratio(e), specific gravity (G), water content (ω) and degree of saturation (Sr) is

a. e =

b. ω =

c. G =

d. Sr =

21. The advantage of reinforced concrete, is due to

a. fire-resisting and durability

b. economy because of less maintenance cost

c. moulding in any desired shape

d. All the above.

22. Steel beam theory is used for

a. design of simple steel beams

b. steel beams encased in concrete

c. doubly reinforced beams ignoring compressive stress in concrete

d. beams if shear exceeds 4 times allowable shear stress.

23. The percentage of minimum reinforcement of the gross sectional area in slabs, is

a. 0.12%

b. 0.15%

c. 0.18%

d. 0.20%

24. An R.C.C. column of 30 cm diameter is reinforced with 6 bars 12 mm φ placed symmetrically along the circumference. If it carries a load of 40, 000 kg axially, the stress is

a. 49.9 kg/cm2

b. 100 kg/cm2

c. 250 kg/cm2

d. 175 kg/cm2

25. A concrete structure is set on fire and the temperature raises to 1000°C. The strength of concrete as compared to original strength reduces to

a. 10%

b. 15%

c. 20%

d. 25%.

26. A roof which slopes in four directions, is called

a. shed roof

b. gable end roof

c. hipped roof

d. gambrel roof.

27. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

28. 1.S. Sieve Nos. 10 mm and 4.75 mm are geneally used for grading of

a. fine aggregates

b. neither (a) nor (b)

c. both (a) and (b)

d. none of these.

29. Run off is the quantity of water which flows

a. in sewer pipes

b. due to leakage in pipes

c. in rivers

d. none of these.

30. The duration of contact of chlorine with water before it is served to the first consumer, should be at least

a. 15 to 20 minutes

b. 20 to 30 minutes

c. 30 to 40 minutes

d. one hour.

31. For estimating the peak run off the rational formula Q = 0.0278 KpA was evolved by

a. Kinchling

b. Lloyd Davis

c. Frubling

d. all the above.

32. For sewer mains of 0.5 to 1 m diameter, the ratio of maximum daily sewage flow to the average daily sewage flow is assumed

a. 1.5

b. 2.0

c. 2.5

d. 3.0

33. The difference in level between the top of a bank and supply level in a canal, is called

a. berm

b. free board

c. height of bank

d. none of these.

34. The sinuosity of a meander is the ratio of

a. meander length and the width of meander

b. meander length and half width of the river

c. curved length and the straight distance

d. none of these.

35. Pick up the incorrect statement from the following

a. Side walls of a venturi head flume are splayed out from the end of the throat at 1 : 10 for a length of 4.5 m

b. Length of side walls should be such that the width of the flume is made equal to 2/3rd the bed width of the distributary

c. Once the width of the flume becomes 2/3rd of the width of the distributary, the splayed walls are increased to 1 in 3 to get full bed width

d. None of these.

36. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

37. The width of pavement in addition to a gravelled berm 1 m on either side for a two directional traffic recommended by Nagpur Conference of Chief Engineers, is

a. 5.7 m

b. 6.7 m

c. 7.7 m

d. 8.7 m

38. The radius of curvature provided along a transition curve, is

a. minimum at the beginning

b. same throughout its length

c. equal to the radius of circular curve

d. varying from infinity to the radius of circular curve.

39. A single lane carriage way whenever changes to two-lane carriage way, is affected through a taper of

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 15 to 1 in 20

40. Camber in pavements is provided by

a. straight line method

b. parabola method

c. straight at the edges and parabolic at the crown

d. all the above.

41. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

42. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

43. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

44. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

45. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

48. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

49. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.

50. The threshold markings are

a. 1 m clear space between adjacents

b. 45 m in length

c. placed symmetrically on either side of the runway centre line

d. all the above.