***Sub Engineer***

1. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

2. Pick up the correct statement from the following :

a. the tangent screw enables to give small movement under conditions of smooth and positive control

b. standing on the tripod is the levelling head or trib arch

c. the levelling screws are used to tilt the instrument so that its rotation axis is truly vertical

d. all the above.

3. It θ is the probable error of an observed bearing of a line of length l, the error over the whole length of the traverse of n lines of length l is

a. l n

b.  n

c. θ

d.  θ n

4. Diaphragm of a surveying telescope is held inside

a. objective

b. telescope tube at its mid point

c. telescope at the end nearer the eye-piece

d. telescope at its end nearer the objective.

5. If the ore impurities is

a. clay, lime stone is used as flux

b. lime stone, clay is used as flux

c. quartz, lime stone and argallaceous iron ores are used as flux

d. All the above.

6. Portland cement manufactured from pure white chalk and clay but free from iron-oxide, is known as

a. quick setting cement

b. rapid hardening cement

c. white cement

d. low heat Portland cement.

7. The commonly used thinner in oil paints, is

a. turpentine

b. both (a) and (b)

c. neither (a) nor (b)

d. none the these.

8. Laterite is found in

a. U.P.

b. Punjab

c. West Bengal

d. Kerala.

9. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

10. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3 M

d. depth d ∝

11. The weakest section of a diamond riveting, is the section which passes through

a. first row

b. second row

c. central raw

d. one rivet hole of end row.

12. A cantilever carrying a uniformly distributed load W over its full length is propped at its free end such that it is at the level of the fixed end. The bending moment will be zero at its free end also at

a. fixed point of the cantilever

b. 1/4th length from free end

c. 3/4th length from free end

d. half length from free end.

13. Pick up the correct statement from the following :

a. Dimensional homogeneity means the dimensions of each term in an equation on both sides are equal

b. Dimensionally homogeneous equations are independent of the system of units

c. In dimensionally homogeneous equation, the powers of fundamental dimensions on either side of the equation are identical

d. All the above.

14. If H is height of the liquid above the sill, the effect of end contractions, according to Francis formula, is

a. 0.1 H

b. 0.2 H

c. 0.3 H

d. 0.4 H

15. The value of kinetic energy correction factor (a) for a laminar flow through a circular pipe, is

a. 1.0

b. l.5

c. 2.0

d. 2.5

16. The intensity of pressure due to sudden closure of a valve of a pipe in which water flows with velocity v, is directly proportional to :

a. square root of the bulk modulus of elasticity of water

b. bulk modulus of elasticity of water

c. specific weight of water

d. none of these.

17. A suspended particle falls through a height H cm in water in t minutes. If the viscosity of water is η and specific gravity of the particle is G, the diameter of the particle is (where M is a constant factor)

a. 103M

b. 104M

c. 105M

d. 102M

18. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

19. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

20. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

21. An under-reinforced section means

a. Steel is provided at the under side only

b. Steel provided is insufficient

c. Steel provided on one face only

d. Steel will yield first.

22. In case the factor of safety against sliding is less than 1.5, a portion of slab is constructed downwards at the end of the heel slab, which is known as

a. a key

b. a cut-off wall

c. a rib

d. all the above.

23. A short column 20 cm x 20 cm in section is reinforced with 4 bars whose area of cross section is 20 sq. cm. If permissible compressive stresses in concrete and steel are 40 kg/cm2 and 300 kg/cm2, the Safe load on the column, should not exceed

a. 4120 kg

b. 41, 200 kg

c. 412, 000 kg

d. none of these.

24. The minimum thickness of the cover at the end of a reinforcing bar should not be less than twice the diameter of the bar subject to a minimum of

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm

25. The depth of excavation of foundations, is generally measured with a

a. ranging rod

b. steel tape

c. levelling staff

d. bonning rod.

26. The concrete slump recommended for foundations, is

a. 30 to 125 mm

b. 50 to 100 mm

c. 75 to 125 mm

d. none of these.

27. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

28. An aggregate is said to be flaky if its least dimension is less than

a. 2/5th of mean dimension

b. 3/5th of mean dimension

c. 4/5th of mean dimension

d. none of these.

29. Detention time for plain sedimentation tank usually ranges from

a. 4 to 8 hours

b. 6 to 10 hours

c. 8 to 12 hours

d. 12 to 16 hours.

30. Pick up the incorrect statement from the following :

a. The depth of water on the filter should be twice the depth of the filter sand

b. When the filter head is 0.75 times the depth of filter sand, the water obtained is purest

c. The cleaning of slow sand filters is done by back washing

d. All the above.

31. Fresh sewage is generally

a. alkaline

b. acidic

c. highly decomposed

d. a source of objectionable odour.

32. In sewage having fully oxidised organic matter, the nitrogen exists in the form of

a. nitrates

b. free ammonia

c. aluminoid nitrogen

d. none of these.

33. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

34. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

35. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

36. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

37. If the rate of change of the super-elevation along a curved portion of a 7 metre wide road is 1 in 150 and the maximum super-elevation allowed is 1 in 15, the maximum length of the transition curve to be provided at either end, is

a. 70 m

b. 75 m

c. 80 m

d. 90 m

38. If L is the length of the transition curves provided on either side of a circular curve of radius R, the maximum angle of deflection with tangent for the junctions of the transition curve and circular curve, is

a. L/2R

b. L/3R

c. L/4R

d. none of these.

39. The radius of curvature provided along a transition curve, is

a. minimum at the beginning

b. same throughout its length

c. equal to the radius of circular curve

d. varying from infinity to the radius of circular curve.

40. The best compromise between the increase of the length of a highway and reduction in its load carrying capacity, is the ruling gradient

a. 1 in 15

b. 1 in 20

c. 1 in 25

d. l in 30

41. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

42. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

43. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

44. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

45. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

46. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

47. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

48. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. Pick up the correct abbreviation from the following:

a. V.H.F, (very high frequency)

b. L.O.M. (low powered outer marker)

c. L.M.M. (low powered middle marker)

d. All the above