***Sub Engineer***

1. The direction of steepest slope on a contour, is

a. along the contour

b. at an angle of 45° to the contour

c. at right angles to the contour

d. none of these.

2. The orthographical projection of a traverse leg upon the reference meridian, is known as

a. departure of leg

b. latitude to the leg

c. co-ordinate of the leg

d. bearing of the leg.

3. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

4. In precision theodolite traverse if included angles are read twice and the mean reading accepted using both verniers having a least count of 30". Assuming the instrument to be in perfect adjustment, linear measurements correct to 6 mm per 30 metre tape duly corrected for temperature, slope and sag, the angular error of closure not to exceed (where n is the number of traverse legs)

a. 50" n

b. 30" n

c. 60" n.

5. Calcination of iron ores is done

a. to remove carbonic acid

b. by roasting in heaps

c. after dressing

d. all the above.

6. Which one of the following is used for preparing porcelain

a. feldspar

b. quartz

c. minerals

d. All of these.

7. Pick up the correct statement from the following:

a. Alexander Parkes, a Scottish chemist prepared a hard material by mixing camphor and alcohol with nitro cellulose and called it, as Parkesite

b. Dr. L. Bakeland, a Belgian scientist prepared a product known as Bakelite

c. Pollark, an Austrian scientist prepared a substance from urea and formaldehyde and called it Plastic

d. All the above.

8. Turpentine oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

9. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory

10. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

11. Rankine-Golden formula accounts for direct as well as buckling stress and is applicable to

a. long columns

b. short columns

c. intermediate columns

d. all the above.

12. Strain energy of any member may be defined as work done on it

a. to deform it

b. to resist elongation

c. to resist shortening

d. all the above.

13. An orifice is an opening in a vessel with

a. closed perimeter of regular shape through which water flows

b. the water level of the liquid on the up stream side is below the top of the orifice

c. partially full flow

d. prolonged sides having length of 2 to 3 diameters of the opening in thick wall.

14. The most efficient channel section, is

a. semi-circular

b. rectangular

c. triangular

d. half hexagon in the form of trapezoid.

15. Pick up the correct statement from the following :

a. In incompressible flow the density of a fluid remains constant

b. In compressible flow, the density of a fluid changes from point to point

c. In uniform flow, the velocity of a fluid does not change with respect to length of flow direction

d. All the above.

16. Bernoulli's equation assumes that

a. fluid is homogeneous

b. flow is steady

c. flow is along the stream line

d. all the above.

17. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

18. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

19. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

20. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

21. If the permissible compressive stress for a concrete in bending is C kg/m2, the modular ratio is

a. 2800/C

b. 2300/2C

c. 2800/3C

d. 2800/C2

22. In a combined footing if shear stress exceeds 5 kg/cm2, the nominal stirrups provided are:

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

23. Pick up the true statement from the following:

a. In the absence of beams, it is easier to install piping

b. In the absence of beams, it is easier to paint

c. A flat slab is capable to withstand concentrated loads

d. All the above.

24. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

25. The construction joints in buildings are provided after

a. 15 m

b. 20 m

c. 40 m

d. 60 m.

26. Arches in the form of masonry arcs struck from more than four centres, are called

a. two curved arches

b. gothic arches

c. ogee arches

d. drop gothic arches.

27. Bulking of sand is

a. mixing of different sizes of sand particles

b. mixing of lime with sand

c. maximum water with sand

d. swelling of sand when wetted.

28. Concrete gains strength due to

a. chemical reaction of cement with sand and coarse aggregates

b. evaporation of water from concrete

c. hydration of cement

d. All the above.

29. If discharge of a pump is 0.16 cumecs, the economic diameter of pipe, is

a. 0.488 m

b. 4.88 cm

c. 0.488 cm

d. 4.88 m

30. Derivation of Thiem's formula  is based on the assumption

a. the well is sunk through the full depth of the aquifer

b. the flow lines are radial and horizontal, and the flow is laminar

c. for hydraulic gradient in Darcy's equation, tangent value is used for sine value

d. all the above.

31. The disintegrating pump in which solid matter is broken up before passing out, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

32. For detecting the nitrates in sewage, the colour may be developed by adding

a. Pottassium permanganate

b. Sulphuric acid and napthamine

c. Phenol-di-sulphuric acid and pottassium hydroxide

d. None of these.

33. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

34. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

35. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

36. Useful soil moisture for plant growth, is

a. capillary water

b. gravity water

c. hygroscopic water

d. chemical water

37. For the movement of vehicles at an intersection of two roads, without any interference, the type of grade separator generally preferred to, is

a. delta

b. trumpet

c. diamond interchange

d. clover leaf.

38. In a braking test, a vehicle travelling at 36 km ph was stopped at a braking distance of 8.0 m. The average value of the vehicle's skid resistance (friction coefficient) is

a. 0.64

b. 6.25

c. 0.16

d. none of these

39. Side drains on both sides of a hill road, are essential when the road is

a. along the spur curves

b. along the re-entrant curves

c. in cutting

d. none of these.

40. If the rate of change of the super-elevation along a curved portion of a 7 metre wide road is 1 in 150 and the maximum super-elevation allowed is 1 in 15, the maximum length of the transition curve to be provided at either end, is

a. 70 m

b. 75 m

c. 80 m

d. 90 m

41. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

42. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

43. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

44. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

45. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

46. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

47. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

48. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)

50. Pick up the correct statement from the following:

a. Approach areas are measured in horizontal surfaces

b. The imaginary inclined plane which is directly above the approach area is called approach surface

c. Obstruction clearance surface and approach surface are same

d. All the above.