***Sub Engineer***

1. If the angular measurements of a traverse are more precise than its linear measurements, balancing of the traverse, is done by

a. Bowditch's rule

b. Transit rule

c. Empirical rule

d. all of the above.

2. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

3. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

4. The slope correction may be ignored if

a. the slope of the ground is less than 3°

b. to slope of the ground is say 1 in 19

c. both (a) and (b)

d. neither (a) nor (b)

5. Sand stone is

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. volcanic rock.

6. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

7. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

8. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

9. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

10. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

11. For a beam having fixed ends, the unknown element of the reactions, is

a. horizontal components at either end

b. vertical components at either end

c. horizontal component at one end and vertical component at the other

d. horizontal and vertical components at both the ends.

12. The minimum number of rivets for the connection of a gusset plate, is

a. 1

b. 2

c. 3

d. 4

13. The intensity of pressure due to sudden closure of a valve of a pipe in which water flows with velocity v, is directly proportional to :

a. square root of the bulk modulus of elasticity of water

b. bulk modulus of elasticity of water

c. specific weight of water

d. none of these.

14. Centre of buoyancy is

a. centroid of the floating body

b. centroid of the fluid displaced

c. centre of pressure of the displaced liquid

d. none of these.

15. The line joining the points to which the liquid rises in vertical piezometer tubes fitted at different cross-sections of a conduit, is known as

a. piezometric line

b. pressure grade line

c. hydraulic grade line

d. all the above.

16. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

17. A structure is erected on an impervious clay whose thickness is 12 m. Drainage is possible both at upper and lower surfaces. Coefficient of consolidation is 0.015 cm2 per minute. For attaining 50% consolidation with a time factor 0.20, the number of days required

a. 3233

b. 3123

c. 33331

d. 3313

18. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

19. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

20. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

21. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

22. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

23. Columns may be made of plain concrete if their unsupported lengths do not exceed their least lateral dimension

a. three times

b. four times

c. five times

d. six times.

24. Steel beam theory is used for

a. design of simple steel beams

b. steel beams encased in concrete

c. doubly reinforced beams ignoring compressive stress in concrete

d. beams if shear exceeds 4 times allowable shear stress.

25. Pick up the incorrect statement from the following :

a. In king post truss, one vertical post is used

b. In a queen post truss, one vertical post is used

c. In a queen post truss, two vertical posts are used

d. None of these.

26. The concrete slump recommended for foundations, is

a. 30 to 125 mm

b. 50 to 100 mm

c. 75 to 125 mm

d. none of these.

27. The cement becomes useless if its absorbed moisture content exceeds

a. 2%

b. 3%

c. 4%

d. 5%

28. Joints in concrete structures, are provided

a. to reduce the tensile stresses likely to be developed due to evaporation of water

b. to minimise the change in the dimensions of the slab

c. to minimise the necessary cracking

d. all the above.

29. Normal values of overflow rate for plain sedimentation tanks in litres/hr/m2, generally range between

a. 100 to 250

b. 250 to 500

c. 500 to 750

d. 750 to 1000

30. In rapid sand filters the ratio of length and diameter of the lateral, should not be greater than

a. 10

b. 15

c. 20

d. 25

31. Pick up the correct statement from the following :

a. The screenings are disposed off either by burning or by burial or by dumping

b. The process of burning the screenings, is called incineration

c. The process of burning the screenings, is known as compositing

d. All the above.

32. Discrete or granular particles change their

a. size

b. shape

c. weight

d. none of these.

33. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

34. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

35. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

36. For the conditions enumerated to provide a crossing at C1 You will probably provide

a. an aqueduct

b. a super-passage

c. a syphon aqueduct

d. none of these.

37. The usual width of side drains along Highways in hilly region, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 100 cm

38. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

39. Pick up the correct statement from the following:

a. Various geometric design features and generally guided by ruling design speed

b. The design speed for a given highway should preferably be uniform

c. Abrupt change in the design speed should not be permitted

d. all the above.

40. The tangent length of a simple circular curve of radius R deflecting through θ°, is

a. R tan θ/2

b. R sin θ

c. R sin θ/2

d. R cot θ/2

41. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

42. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

43. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

44. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

45. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

46. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

47. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

48. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

49. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.

50. The strength of winds is measured with the help of

a. Benfort scale

b. Wind indicator

c. Barometres

d. None of these.