***Sub Engineer***

1. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

2. Pick up the correct statement from the following :

a. mistakes arise from inattention, inexperience or carelessness

b. systematic errors persist and have regular effects in the survey performances

c. accidental errors occur inspite of every precaution is taken

d. all the above.

3. Diaphragm of a surveying telescope is held inside

a. objective

b. telescope tube at its mid point

c. telescope at the end nearer the eye-piece

d. telescope at its end nearer the objective.

4. Chain surveying is well adopted for

a. small areas in open ground

b. small areas with crowded details

c. large areas with simple details

d. large areas with difficult details.

5. To retard the initial setting time of cement, the compound responsible, is

a. Tricalcium silicate

b. Gypsum

c. Di-calcium silicate

d. Tri calcium aluminate.

6. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

7. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

8. Duco is one of the patent forms of

a. plastic paints

b. bituminous paints

c. aluminium paints

d. cellulose paints.

9. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

10. The ratio of moments of inertia of a triangular section about its base and about a centroidal axis parallel to its base, is

a. 1.5

b. 2.0

c. 2.5

d. 3.0

11. Beams of uniform strength are preferred to those of uniform section because these are economical for

a. large spans

b. heavy weights

c. light weights

d. short spans.

12. The number of points of contraflexure in a simple supported beam carrying uniformly distributed load, is

a. 1

b. 2

c. 3

d. 0

13. From a nozzle exposed to atmosphere, the liquid jet traverses

a. a straight line

b. a circular path

c. an elliptical path

d. a parabolic path.

14. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

15. The flow in open channel is laminar if the Reynold number is

a. less than 500

b. more than 500

c. 1000

d. none of these.

16. If v1 and v2 are the velocities of flow before and after sudden enlargement in a pipe, the head loss given by Carnot and Borda equation, is

a. (v22 - v12)/2g

b. (v1 - v2)2/2g

c. (v2 - v1)2/2g

d. (v1 + v2)2/2g.

17. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

18. Pick up the incorrect statement from the following:

a. Compaction has no effect on the structure of a soil

b. Permeability decreases with increase in the dry density of a compacted soil

c. A wet side compacted soil is more compressible than a dry side compacted soil

d. Dry side compaction soils swell more when given access to moisture

19. The seepage exit gradient in a soil is the ratio of

a. flow line to slope

b. head upstream to that at downstream

c. head loss to the length of the seepage

d. none of these.

20. Pick up the correct statement from the following:

a. The void space between the soil grains, is filled partly with air and partly with water

b. In perfectly saturated soil, the voids are completely filled with water

c. In dry soil, the voids are completely filled with air

d. all the above.

21. The zone in which transverse bending is likely to occur may be obtained by drawing a line from the faces of the column making an angle θ° with horizontal where θ° is

a. 30°

b. 45°

c. 60°

d. none of these.

22. The maximum ratio of span to depth of a slab simply supported and spanning in one direction, is

a. 25

b. 30

c. 20

d. 15

23. If a rectangular prestressed beam of an effective span of 5 meters and carrying a total load 3840 kg/m, is designed by the load balancing method, the central dip of the parabolic tendon should be

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

24. A T-beam behaves as a rectangular beam of a width equal to its flange if its neutral axis

a. remains within the flange

b. remains below the slab

c. coincides the geometrical centre of the beam

d. none of these.

25. Pick up the incorrect statement from the following:

a. The section of the retaining wall should be so proportioned that it may not overturn by the lateral pressure

b. The retaining wall should be safe against sliding

c. The foundation of the retaining wall should not be stressed beyond safe bearing capacity ; due to its weight and the force resulting from the earth pressure

d. to drain off water from the earth retained, weep holes are provided near the top of the retaining wall.

26. Cast iron piles

a. are suitable for works under sea water

b. resist shocks or vibrations

c. are suitable for use as batter piles

d. are useful for heavy vertical loads.

27. Pick up the correct statement from the following:

a. Silica in excess, causes the cement to set slowly

b. Alumina in excess, reduces the strength of the cement

c. Magnesium oxide in excess, remains in free state and makes the cement unsound

d. All the above.

28. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

29. The process of passing water through beds of granular materials, is called

a. screening

b. sedimentation

c. filtration

d. none of these.

30. When the reduced level of the water source is higher than the reduced level of the consumer's place, water is generally supplied

a. by pumping system

b. by gravitational system

c. both (a) and (b)

d. all the above.

31. The grit chambers of sewage treatment plants, are generally cleaned after

a. 5 days

b. 7 days

c. 12 days

d. 14 days.

32. If the flame of a miner's safety lamp in a manhole extinguishes within 5 minutes, the sewer certainly contains

a. Carbon dioxide

b. Methane

c. Oxygen

d. None of these.

33. The field capacity of a soil is 25%, its permanent wilting point is 15% and specific dry unity weight is 1.5. If the depth of root zone of a crop, is 80 cm, the storage capacity of the soil, is

a. 10 cm

b. 12 cm

c. 14 cm

d. 16 cm

34. Side slopes generally kept for canals in loam soil, are :

a. 1 : 1 in cutting and  : 1 in filling

b.  : 1 in cutting and 2 : 1 in filling

c. Neither (a) nor (b)

d. Both (a) and (b)

35. The ratio of the rate of change of discharge of an outlet to the rate of change in level of water surface in a distributary at its normal depth, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

36. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

37. If the elevations along a road increase, the slope of the road along the longitudinal direction, is known as

a. gradient

b. grade

c. positive grade

d. negative grade.

38. The distance travelled by a moving vehicle during perception and brake reaction times, is known as

a. sight distance

b. stopping distance

c. lag distance

d. none of these.

39. If d is the thickness of a concrete pavement, the equivalent radius b of resisting section for an interior loading, is

a. b = 0.6 + d2 - 0.675 d

b. b = 1.6 + d2 + 0.675 d

c. b = 1.6 + d2 - 3.675 d

d. b = 1.6 + d2 - 0.675 d

40. If R is the radius of a main circular curve, Δ is the angle of deflection and α is the polar deflection angle of any point, its radial distance is

a. 3 R sin Δ/3 sin 2α

b. 3 R sin Δ/2 sin 3α

c. 3 R sin Δ/3 sin α/2

d. 3 R sin 2Δ sin α

41. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

42. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

43. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

44. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

45. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

46. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

47. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

48. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

49. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.

50. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.