***Sub Engineer***

1. One of the tacheometric constants is additive, the other constant, is

a. subtractive constant

b. multiplying constant

c. dividing constant

d. indicative constant.

2. An ideal vertical curve to join two gradients, is

a. parabolic

b. elliptical

c. hyperbolic

d. none of these.

3. Contour lines of different elevations can unite to form one line, only in the case of

a. a vertical cliff

b. a saddle

c. a water shed line

d. a hill top.

4. If a tacheometer is fitted with an anal-latic lens

a. additive constant is 100, multiplying constant is zero

b. multiplying constant is 100, additive constant is zero

c. both multiplying and additive constants are 100

d. both multiplying and additive constants are 50.

5. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

6. Asbestos

a. is fire-proof and acid proof

b. has sp. gravity equal to 3.10

c. is smooth like glass and silk

d. All the above.

7. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

8. Asbestos cement

a. is brittle

b. warps due to changes in humidity

c. strength is lowered when saturated by water

d. all the above.

9. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

10. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

11. If the shear force along a section of a beam is zero, the bending moment at the section is

a. maximum

b. minimum

c. average of maximum-minimum

d. none of these.

12. If the depth of a simply supported beam carrying an isolated load at its centre, is doubled, the deflection of the beam at the centre will be changed by a factor of

a. 1/2

b. 8

c. 1/8

d. 4

13. If cohesion between the molecules of a fluid is more than adhesion between the fluid and glass, the free level of fluid in a dipped glass tube will be

a. higher than the surface of liquid

b. same as the surface of liquid

c. lower than the surface of liquid

d. none of these.

14. Pick up the correct statement from the following :

a. For tranquil flow h > 2g or v < v22g

b. For critical flow h=v22g

c. For torrential flow h > 2g  or v < gh

d. None of these.

15. The specific weight of water is 1000 kg/m3

a. at normal pressure of 760 mm

b. at 4°C temperature

c. at mean sea level

d. all the above.

16. When water flows over a rectangular suppressed weir, the negative pressure created beneath the nappe

a. increases the discharge

b. decreases the discharge

c. does not effect the discharge

d. none of these.

17. The Westergaard analysis is used for

a. sandy soils

b. cohesive soils

c. stratified soils

d. clayey soils.

18. The ratio of the volume of voids to the volume of soil solids in a given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

19. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

20. The length/diameter ratio of cylindrical specimens used in triaxial test, is generally

a. 1.5

b. 2

c. 2.5

d. 3

21. Minimum spacing between horizontal parallel reinforcement of different sizes, should not be less than

a. one diameter of thicker bar

b. sum of the diameters of ihinner and thicker bars

c. twice the diameter of thinner bar

d. none of these.

22. A pre-stressed concrete member

a. is made of concrete

b. is made of reinforced concrete

c. is stressed after casting

d. possesses internal stresses.

23. According to I.S.: 456, 1978 the thickness of reinforced concrete footing on piles at its edges, is kept less than

a. 30 cm

b. 40 cm

c. 50 cm

d. 75 cm

24. If the average bending stress is 6 kg/cm2 for M 150 grade concrete, the length of embedment of a bar of diameter d according to I.S. 456 specifications, is

a. 38 d

b. 48 d

c. 58 d

d. 95 d

25. The type of stone masonry in which stones of same height are laid in layers, is called

a. random rubble masonry

b. course rubble masonry

c. uncoursed rubble masonry

d. ashlar masonry.

26. In soft clay of low bearing capacity, the type of steel pile generally used, is

a. screw pile

b. disc pile

c. pipe pile

d. raking pile.

27. Saw dust can be rendered chemically inert by boiling it in water containing

a. ferrous sulphate

b. potassium chloride

c. ammonia

d. nitric acid

28. The operation of removing humps and hollows of uniform concrete surface, is known as

a. screeding

b. trowelling

c. finishing

d. none of these.

29. Derivation of Thiem's formula  is based on the assumption

a. the well is sunk through the full depth of the aquifer

b. the flow lines are radial and horizontal, and the flow is laminar

c. for hydraulic gradient in Darcy's equation, tangent value is used for sine value

d. all the above.

30. The ensure proper growth of children's teeth, the quantity of flouride used in water mains, is

a. 1 mg/litre

b. 2 mg/litre

c. 3 mg/litre

d. 4 mg/litre

31. Drop manholes at the junctions of sewer lines, are provided if

a. invert level of a branch sewer is more than 60 cm that of the main sewer

b. sewer line runs along a main road

c. ordinary manhole cannot be built

d. two sewer lines intersect.

32. The bottom of the sewage inlet chamber of septic tanks, is provided an outward slope

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 25

33. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

34. Attracting groynes are built

a. perpendicular to the bank

b. inclined down stream

c. inclined up stream

d. none of these.

35. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

36. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

37. The minimum value of camber provided for thin bituminous surface hill roads, is

a. 2.2%

b. 2.5%

c. 3.0%

d. 3.5%

38. If the designed speed on a circular curve of radius 1400 m is 80 km/hour, no superelevation is provided, if the camber, is

a. 4%

b. 3%

c. 2%

d. 1.7%

39. The extra widening of pavements recommended by Indian Roads Congress for hill roads having radius 150 metres, is

a. 1.0 m

b. 0.5 m

c. 0.0 m

d. 2.0 m

40. Thickness of a pavement may be reduced considerably by

a. compaction of soil

b. stabilisation of soil

c. drainage of soil

d. combination of all the above.

41. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

42. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

43. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

44. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

45. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

46. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

47. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

48. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

49. Pick up the correct statement from the following:

a. F.I.C. stands for flight information centre

b. Radius of control area is 160 km

c. Radius of control zone is 40 km

d. All the above.

50. The distance between main gears is 10 m, and the distance of nose gear from centre of main gears is 30 m. If the angle of turning is 60°, the distance of centre of rotation from the nearer main gear, is

a. 12.30 m

b. 11.30 m

c. 10.30 m

d. 9.30 m