***Sub Engineer***

1. The chord of a curve less than peg interval, is known as

a. small chord

b. sub-chord

c. normal chord

d. short chord.

2. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

3. For high sensivity of the bubble tube

a. a liquid of low surface tension is used

b. the bubble space should be long

c. the bubble tube should not be too narrow

d. all the above.

4. Cross-staff is used for

a. setting out right angles

b. measuring contour gradient

c. taking levels

d. measuring distances

5. Pick up the correct statement from the following:

a. During rusting, first ferrous bicarbonates are formed

b. On further oxidation ferrous bicarbonates get converted to ferric bicarbonates

c. Ultimately hydrated ferric oxide is formed during rusting and carbon dioxide gets liberated

d. All the above.

6. Pick up the correct statement from the following:

a. The articles prepared from clay which is burnt at low temperature and cooled down slowly, are called earthen-ware

b. The articles prepared from refractory clays which as mixed with stone and crushed pottery, are called stone ware

c. The articles prepared from fine earthen ware which is white, thin and semi-transparent, are called procelain

d. All the above.

7. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

8. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

9. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

10. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

11. The maximum compressive stress at the top of a beam is 1600 kg/cm2 and the corresponding tensile stress at its bottom is 400 kg/cm2 . If the depth of the beam is 10 cm, the neutral axis from the top, is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

12. The slenderness ratio of a vertical column of square cross- section of 10 cm side and 500 cm long, is

a. 17.3

b. 173.2

c. 137.2

d. 13.72

13. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

14. The discharge over a Cipolletti weir of length 2.185 m when the head over the weir is 1 m, is

a. 2.0 m3

b. 2.5 m3

c. 3.0 m3

d. 4.0 m3.

15. In a fluid flow a particle may posses

a. kinetic energy

b. pressure energy

c. initial energy

d. all the above.

16. In Chezy's formula V = C mi

a. V is the mean velocity of flow

b. m is the hydraulic mean depth

c. i is the loss of head per unit length of pipe

d. All the above.

17. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

18. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

19. A clay subjected to pressure in excess to its present over-burden, is said to be

a. pre-compressed

b. pre-consolidated

c. over-consolidated

d. all the above.

20. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

21. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

22. In a simply supported slab, alternate bars are curtailed at

a. 1/5th of the span

b. 1/6th of the span

c. 1/7th of the span

d. none of these.

23. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

24. If the diameter of the main reinforcement in a slab is 16 mm, the concrete cover to main bars is

a. 13 mm

b. 14 mm

c. 15 mm

d. 16 mm

25. For each storey of a building, the depth of exploration should be

a. 1 metre

b. 2 metres

c. 3 metres

d. 4 metres.

26. While investigating the site, a thick layer of fairly firm clay over a deep layer of soft clay is encountered. In such a situation, the following type of foundation is useful :

a. pile formation

b. raft foundation

c. grillage foundation

d. none of these.

27. The strength and quality of concrete, depend upon:

a. surface area of aggregates

b. shape of aggregates

c. surface texture of aggregates

d. all the above.

28. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

29. Normal values of overflow rate for plain sedimentation tank using coagulants, is

a. 750 to 1000 litres/hr/m2

b. 1000 to 1250 litres/hr/m2

c. 1250 to 1500 litres/hr/m2

d. 1500 litres/hr/m2.

30. The population growth curve is

a. S-shaped curve

b. parabolic curve

c. circular curve

d. straight line

31. The gas evolved in sewers is

a. Carbondioxide

b. Hydrogen sulphide

c. Methane

d. All of these.

32. Pick up the correct statement from the following :

a. The water supply pipes carry pure water free from solid particles

b. The water supply pipes get clogged if flow velocity is less than self cleansing velocity

c. The sewers may be carried up and down the hills and valleys

d. The sewer pipes are generally laid along level hills

33. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

34. Pick up the correct statement from the following:

a. Duty of canal water is usually calculated at the head discharge of a channel

b. Duty of canal water at the head of a channel and at the outlet of a distributary, differ considerably due to progressive losses of water

c. Duty of canal water at the outlet of a distributary, is generally known as outlet factor or outlet discharge factor

d. All the above.

35. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

36. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

37. The minimum width of the pavement of a National Highway should be

a. 5.7 m

b. 6.7 m

c. 7.7 m

d. 8.0 m

38. To ensure that bullock carts may not overturn on curves, the maximum value of super-elevation, recommended by I.R.C., is

a. 1 in 12

b. 1 in 15

c. 1 in 20

d. 1 in 25

39. In water bound macadam roads, binding material, is

a. sand

b. stone dust

c. cement

d. brick dust.

40. Thickness of a pavement may be reduced considerably by

a. compaction of soil

b. stabilisation of soil

c. drainage of soil

d. combination of all the above.

41. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

42. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

43. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

44. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

45. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

46. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

47. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

48. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

49. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.

50. Pick up the correct statement from the following:

a. Minimum turning radius of aircrafts decides the size of the apron and the radius of the curves at taxi-ends

b. Take off and landing distances for an aircraft, determine the minimum runway length

c. The length of the normal haul of the air craft decides the frequency of operation

d. All the above.