***Sub Engineer***

1. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

2. Pick up the correct statement from the following :

a. the lines of sight while observing back sight and fore  sight lie in the same horizontal plane

b. the   staff readings are measurements made vertically downwards from a horizontal plane

c. the horizontal plane with reference to which staff readings are taken, coincides with the level surface through the telescope axis

d. all the above.

3. In tacheometrical observations, vertical staff holding is generally preferred to normal staffing, due to

a. ease of reduction of observations

b. facility of holding

c. minimum effect of careless holding on the result

d. none of these.

4. An ideal transition curve is

a. cubic parabola

b. cubic spiral

c. clothoid spiral

d. true spiral.

5. Mastic asphalt is normally used for

a. sound insulation

b. water proofing

c. fire proofing

d. none to these.

6. Elastomers can extend upto

a. five times their original dimensions

b. seven times their original dimensions

c. ten times their original dimensions

d. three times their original dimensions.

7. Pick up the correct statement from the following:

a. Plastics have generally low melting point

b. The coefficient of thermal expansion of plastics is about three times than that of steel

c. The acoustical boards prepared by impregnating fibre-glass with phenolic resins has absorption coefficient of about 0.67

d. All the above.

8. The melting point of silica is :

a. 1570°C

b. 1630°C

c. 1730°C

d. 1850°C.

9. A steel bar 20 mm in diameter simply-supported at its ends over a total span of 40 cm carries a load at its centre. If the maximum stress induced in the bar is limited to N/mm2, the bending strain energy stored in the bar, is

a. 411 N mm

b. 511 N mm

c. 611 Nmm

d. 711 N mm

10. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

11. A cantilever carries is uniformly distributed load W over its whole length and a force W acts at its free end upward. The net deflection of the free end will be

a. zero

b.  upward

c.  downward

d. none of these.

12. The moment diagram for a cantilever whose free end is subjected to a bending moment, will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

13. The Empirical formula  for discharge over large rectangular weirs, is known as

a. Francis formula

b. Bazin formula

c. Rehbook formula

d. Kutter's formula.

14. The ratio of the maximum height to which a jet inclined through θ rises to its total horizontal span is

a.  sin θ

b.  cos θ

c.  tan θ

d.  cot θ.

15. Falling drops of water become spheres due to

a. adhesion

b. cohesion

c. surface tension

d. viscosity.

16. The main assumption of Bernoulli's equation is :

a. The velocity of energy of liquid particle, across any cross-section of a pipe is uniform

b. No external force except the gravity acts on the liquid

c. There is no loss of energy of the liquid while flowing

d. All the above.

17. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

18. A plane inclined at an angle φ to the horizontal at which the soil is expected to stay in the absence of any lateral support, is known as

a. natural slope line

b. repose line

c. the φ line

d. all the above.

19. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

20. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

21. A reinforced concrete cantilever beam is 3.6 m long, 25 cm wide and has its lever arm 40 cm. It carries a load of 1200 kg at its free end and vertical stirrups can carry 1800 kg. Assuming concrete to carry one-third of the diagonal tension and ignoring the weight of the beam, the number of shear stirrups required, is

a. 35

b. 40

c. 45

d. 50

22. The pitch of the main bars in a simply supported slab, should not exceed its effective depth by

a. four times

b. five times

c. six times

d. seven times.

23. In favourable circumstances a 15 cm concrete cube after 28 days, attains a maximum crushing strength

a. 200 kg/cm2

b. 300 kg/cm2

c. 400 kg/cm2

d. 500 kg/cm2

24. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

25. Cast iron piles

a. are suitable for works under sea water

b. resist shocks or vibrations

c. are suitable for use as batter piles

d. are useful for heavy vertical loads.

26. Queen closer may be placed

a. in stretcher course

b. in header course next to first brick

c. in stretcher course next to first brick

d. in any position.

27. Segregation is responsible for

a. porous layers in concrete

b. surface scaling in concrete

c. sand streaks in concrete

d. all the above.

28. Expansion joints are provided if the length of concrete structures exceeds

a. 15 m

b. 15 m

c. 35 m

d. 45 m

29. The maximum permitted loss of head in a rapid sand filter, is

a. 2 m

b. 3 m

c. 4 m

d. 5 m.

30. The prescribed hardness limit of potable water ranges between

a. 75 to 115 P.P.M.

b. 100 to 150 P.P.M.

c. 150 to 200 P.P.M.

d. none of these.

31. For design of sewers, percentage of sewage discharge is assumed

a. 65 to 70%

b. 70 to 75%

c. 75 to 80%

d. 85%.

32. To test chemical oxygen demand (C.O.D.) of sewage, organic matter is oxidised by pottassium dichromate in the presence of

a. Hydrochloric acid

b. Sulphuric acid

c. Nitric acid

d. Citric acid.

33. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

34. The width of a meander belt is the transverse distance between

a. apex point of one curve and apex point of the reserve curve

b. apex point and the crossing

c. two banks of meandering river

d. none of these.

35. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

36. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

37. Design of horizontal and vertical alignments, super-elevation, sight distance and grades, is worst affected by

a. width of the vehicle

b. length of the vehicle

c. height of the vehicle

d. speed of the vehicle

38. In ideal pavement is constructed with

a. hard soil

b. Portland cement concrete

c. tar

d. none of these.

39. A gradient along which the vehicle does not require any tractive effort to maintain a specified speed, is known as

a. ruling gradient

b. pushing gradient

c. floating gradient

d. minimum gradient

40. From the point of tangency before an intersection, the route markers are fixed at a distance of

a. 20 m to 35 m

b. 40 m to 50 m

c. 50 m to 75 m

d. 100 m to 150

41. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

42. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

43. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

44. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

45. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

46. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

47. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

48. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

49. The length of runway is increased per 300 m rise above M.S.L.

a. 4%

b. 5%

c. 6%

d. 7%

50. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.