***Sub Engineer***

1. In a telescope the object glass of focal length 14 cm, is located at 20 cm from the diaphragm. The focussing lens is midway between them when a staff 16.50 m away is focussed. The focal length of the focussing lens, is

a. 5.24 cm

b. 6.24 cm

c. 7.24 cm

d. 8.24 cm

2. Surveys which are carried out to depict mountains, rivers, water bodies, wooded areas and other cultural details, are known as

a. city surveys

b. topographical surveys

c. guide map surveys

d. plane surveys.

3. An ideal vertical curve to join two gradients, is

a. parabolic

b. elliptical

c. hyperbolic

d. none of these.

4. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

5. The compound of Portland cement which contributes to the strength after two to three years is

a. Tricalcium silicate

b. Di-calcium silicate

c. Tricalcium aluminate

d. Tetracalcium alumino ferrite.

6. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

7. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

8. The weight of a good quality brick when immersed in water for a period of 16 hours should not exceed the weight of dry brick

a. 20%

b. 15%

c. 10%

d. None of these.

9. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

10. The force in BC of the truss shown in the given figure, is

a. 3.0t tension

b. t tension

c. t compression

d. None of these.

11. For structural analysis of forces, the method refers to

a. moment-area-theorem

b. three-moment equation

c. Maxwell's reciprocal theorem

d. none of these.

12. If the stress produced by a prismatic bar is equal to the working stress, the area of the cross-section of the prismatic bar, becomes

a. zero

b. infinite

c. maximum

d. minimum.

13. Pick up the correct statement from the following :

a. Discharge over a triangular notch is proportional to H5/2

b. Discharge over a rectangular notch is proportional to H3/2

c. Both (a) and (b)

d. Neither (a) nor (b).

14. Back water curve is caused if

a. friction head loss is more than the bed slope

b. pressure is due to weir in the channel

c. there is an increase in width of the channel

d. none of these.

15. For steady flow in open channels, which one of the following does not change :

a. depth of flow

b. velocity of flow

c. rate of flow

d. All of these.

16. The specific weight of water is 1000 kg/m3

a. at normal pressure of 760 mm

b. at 4°C temperature

c. at mean sea level

d. all the above.

17. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

18. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

19. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

20. The Westergaard analysis is used for

a. sandy soils

b. cohesive soils

c. stratified soils

d. clayey soils.

21. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

22. The anchorage value of a hook is assumed sixteen times the diameter of the bar if the angle of the bend, is

a. 40°

b. 45°

c. 60°

d. all the above.

23. The floor slab of a building is supported on reinforced cement floor beams. The ratio of the end and intermediate spans is kept

a. 0.8

b. 0.9

c. 0.6

d. none of these.

24. The width of the flange of a T-beam, which may be considered to act effectively with the rib depends upon

a. overall thickness of the rib

b. centre to centre distance between T-beams

c. span of the T-beam

d. all the above.

25. The 9 cm x 9 cm side of a brick as seen in the wall face, is generally known as

a. face

b. front

c. header

d. side.

26. Black cotton soil is unsuitable for foundations because its

a. bearing capacity is low

b. permeability is uncertain

c. particles are cohesive

d. property to undergo a volumetric change due to variation of moisture content.

27. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

28. Hydration of cement is due to chemical action of water with

a. Tricalcium silicate and dicalcium silicate

b. Dicalcium silicate and tricalcium aluminate

c. Tricalcium aluminate and tricalcium alumino ferrite

d. All the above.

29. Continuous flow of water can be expected from

a. gravity springs

b. surface springs

c. artesian springs

d. none of these.

30. In distribution pipes, air valves are provided at

a. lower points

b. junction points

c. higher points

d. any where.

31. Drop manholes at the junctions of sewer lines, are provided if

a. invert level of a branch sewer is more than 60 cm that of the main sewer

b. sewer line runs along a main road

c. ordinary manhole cannot be built

d. two sewer lines intersect.

32. The following is the physical characteristic of sewage

a. colour

b. odour

c. temperature

d. all the above.

33. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 H + d

b. B = 0.33 H + d

c. B = 0.44 H + d

d. B = 0.55 H + d

34. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

35. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

36. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

37. Indian Road Congress (I.R.C.) was founded and constituted with its head quarters at New Delhi, in

a. 1927

b. 1930

c. 1934

d. 1942

38. For a vehicle moving with a speed of 80 km per hour, the brake reaction time, in ordinary cases, is

a. 1.5 sec

b. 2.0 sec

c. 2.5 sec

d. 3.0 sec

39. If R is the radius of a main circular curve, Δ is the angle of deflection and α is the polar deflection angle of any point, its radial distance is

a. 3 R sin Δ/3 sin 2α

b. 3 R sin Δ/2 sin 3α

c. 3 R sin Δ/3 sin α/2

d. 3 R sin 2Δ sin α

40. Deviation of the alignment of a trace cut may be permitted in areas involving

a. land slides

b. sand dunes

c. dens

d. none of these.

41. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

42. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

43. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

44. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

45. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

46. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

47. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

48. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

49. The threshold markings are

a. 1 m clear space between adjacents

b. 45 m in length

c. placed symmetrically on either side of the runway centre line

d. all the above.

50. The maximum length and pavement strength of the runway is that of

a. A 1

b. B 2

c. B 3

d. G 7