***Sub Engineer***

1. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

2. The staff intercept will be

a. greater farther off the staff is held

b. smaller, farther off the staff is held

c. smaller, nearer the staff is held

d. same, wherever the staff is held.

3. Rankine's deflection angle in minutes is obtained by multiplying the length of the chord by

a. degree of the curve

b. square of the degree of the curve

c. inverse of the degree of the curve

d. none of these.

4. The chord of a curve less than peg interval, is known as

a. small chord

b. sub-chord

c. normal chord

d. short chord.

5. The hardest rock is

a. marble

b. diamond

c. talc

d. quartz.

6. The PVC doors and windows are preferred as they are

a. rot proof

b. termile proof

c. water proof

d. all of these.

7. Durability of building stone is affected by its

a. texture

b. resistance to atmosphere

c. location in structure

d. all the above.

8. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

9.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

10. The equivalent length of a column of length L, having both the ends hinged, is

a. 2L

b. L

c.

d.

11. In a loaded beam, the point of con-traflexture occurs at a section where

a. bending moment is zero or changes sign

b. bending moment is maximum

c. shearing force is maximum

d. shearing force is minimum.

12. While testing a cast iron beam (2.5 cm x 2.5 cm) in section and a metre long simply supported at the ends failed when a 100 kg weight is applied at the centre. The maximum stress induced is :

a. 960 kg/cm2

b. 980 kg/cm2

c. 1000 kg/cm2

d. 1200 kg/cm2.

13. Euler's equation for the motion of liquids assumes that

a. fluid is viscous

b. fluid is homogeneous and incompressible

c. velocity of flow is non-uniform over the section

d. flow is unsteady along the stream line.

14. In C.G.S. system the units of kinematic viscosity, is

a. stoke

b. poise

c. Newton

d. none of these.

15. The flow in which each liquid particle has a definite path and the paths of adjacent particles do not cross each other, is called

a. stream line flow

b. uniform flow

c. steady flow

d. turbulent flow.

16. Capillary rise of water is

a. directly proportional to surface tension

b. inversely proportional to water density

c. inversely proportional to diameter of the tube

d. All of these.

17. If there is no impervious boundary at the bottom of a hydraulic structure, stream lines tend to follow :

a. a straight line

b. a parabola

c. a semi-ellipse

d. a semi-circle.

18. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

19. If σ0' and σ' represent initial and increased pressure ; e0 and e void ratios corresponding to initial and increased pressure ; and C0 the compression index (dimensionless), then, the virgin compression curve as expressed by Terzaghi empirical formula is

a.  e = e0 + Cc log10 σ'/σ0'

b. e = e0 - Cc log10 σ'/σ0'

c. e0 = e - Cc log10 σ'/σ0'

d. e0 = e + Cc log10 σ0'/σ'

20. The consolidation time for soils

a. decreases with increasing permeability

b. increases rapidly with increasing size of soil mass

c. is independent of the magnitude of the stress change.

d. All the above.

21. The thickness of base slab of a retaining wall generally provided, is

a. one-third of the width of the stem at the bottom

b. one fourth of the width of the steam at the bottom

c. width of the stem at the bottom

d. twice the width of the steam at the bottom.

22. If permissible working stresses in steel and concrete are respectively 1400 kg/cm2 and 80 kg/cm2 and modular ratio is 18, in a beam reinforced in tension side and of width 30 cm and having effective depth 46 cm, the lever arms of the section, is

a. 37 cm

b. 38 cm

c. 39 cm

d. 40 cm

23. The width of the flange of a T-beam should be less than

a. one-third of the effective span of the T-beam

b. distance between the centres of T-beam

c. breadth of the rib plus twelve times the thickness of the slab

d. least of the above.

24. The stresses developed in concrete and steel in reinforced concrete beam 25 cm width and 70 cm effective depth, are 62.5 kg/cm2 and 250 kg/cm2 respectively. If m = 15, the depth of its neutral axis is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

25. In the method of tube boring of soil investigation, the following is essential :

a. a flap valve at the bottom of tube is provided to extract the soil sample

b. the tube is raised and lowered by 4 thick rope moving over a pulley suspended on a tripod stand

c. the tube is dropped to fall under gravity in side a metalic casing pipe which is driven as the depth of excavation proceeds.

d. all the above.

26. In high mountaneous region, the type of roof generally recommended for buildings, is

a. shed type

b. gable type

c. gambrel type

d. mansord type.

27. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

28. While designing an air entrained concrete

a. proportion of aggregates is reduced

b. an allowance for the entrained air is made

c. strength of the concrete, is reduced

d. all the above.

29. A slow sand filter is cleaned if its filter head is higher than

a. 20 cm to 40 cm

b. 40 cm to 70 cm

c. 70 cm to 120 cm

d. 120 cm to 150 cm.

30. High pH value of water does not produce

a. sediment deposits

b. turberculation

c. corrosion

d. both (c) and (d).

31. For trunk and out-fall, the type of sewers generally used, is

a. circular shaped

b. horse shoe shaped

c. parabolic shaped

d. semi-elliptical shaped.

32. In circular sewers if depth of flow is 0.2 times the full depth, the nominal gradient,

a. is only provided

b. is doubled

c. is trebled

d. is not enough.

33. Regime conditions in a channel may occur if

a. discharge is constant

b. channel flows uniformly in incoherent alluvium as that transported in suspension

c. silt grade and silt charge are constant

d. all the above.

34. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

35. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

36. In a Sarda type fall, the rectangular crest, may be used for discharge upto

a. 10 cumecs

b. 14 cumecs

c. 20 cumecs

d. 25 cumecs.

37. Design of horizontal and vertical alignments, super-elevation, sight distance and grades, is worst affected by

a. width of the vehicle

b. length of the vehicle

c. height of the vehicle

d. speed of the vehicle

38. Customers prefer parking of their vehicles at

a. 85° to aisles

b. 80° to aisles

c. 75° to aisles

d. 60° to aisles

39. Minimum thickness of the base of a flexible pavement, is

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

40. The head light of vehicles should be such that its lower beam illuminates objects at

a. 20 m

b. 30 m

c. 40 m

d. 50 m

41. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

42. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

43. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

44. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

45. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

46. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

47. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

48. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

49. The bearing of the runway at threshold is 290°, the runway number is

a. N 70° W

b. 290°

c. 29°

d. W 20° N

50. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m