***Sub Engineer***

1. Magnetic bearing of a survey line at any place

a. remains constant

b. changes systematically

c. varies differently in different months of the year

d. is always greater than true bearing.

2. The bearing of C from A is N 30° E and from B, 50 metres east of A, is N 60° W. The departure of C from A is

a. 50 m

b. 50 3 m

c. 25 3 m

d. 25 m

3. During levelling if back sight is more than foresight

a. The forward staff is at lower point

b. The back staff is at lower point

c. The difference in level, cannot be ascertained.

d. none of these.

4. For locating a distant object visible from two transit stations, the method usually preferred to, is

a. Angles and distances from transit stations

b. Angles from two transit stations

c. distance from two transit stations

d. Angle from one transit station and distance from the other.

5. The percentage of alumina and silica in good fire clay vary respectively is

a. 25, 75

b. 30, 70

c. 35, 65

d. All of these.

6. Rapid hardening cement attains early strength due to

a. larger proportion of lime grounded finer than normal cement

b. lesser proportion of lime grounded coarser than normal cement

c. lesser proportion of lime grounded finer than normal cement

d. larger proportien of lime grounded coarser than normal cement

7. Bitumen emulsion is

a. a liquid containing bitumen in suspension

b. a paint

c. used as anti-corrosive paint

d. all the above.

8. Softer variety of steel may be obtained by

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

9. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

10.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. Every material obeys the Hooke's law within its

a. elastic limit

b. plastic point

c. limit of proportionality

d. none of these.

12. For keeping the stress wholly compressive the load may be applied on a circular column anywhere within a concentric circle of diameter

a. d/3

b. d/4

c. d/8

d. d/10

13. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

14. The flow is called rotational if its velocity normal to the plane of area is equal to

a. angular velocity vector

b. twice the angular velocity vector

c. thrice the angular velocity vector

d. none of these.

15. The ratio of the maximum height to which a jet inclined through θ rises to its total horizontal span is

a.  sin θ

b.  cos θ

c.  tan θ

d.  cot θ.

16. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

17. Pick up the correct statement applicable to plate load test

a. At the centre of the test pit, a hole is dug out whose size is kept equal to the size of the test plate

b. Bottom level of the hole dug at the centre of the test pit, is kept at the level of the actual formation

c. Ratio of the depth of the hole to the width of the test plate is the same as the ratio of the depth of the actual formation to the width of the formation

d. All the above.

18. The coefficient of curvature for a well graded soil, must be between

a. 0.5 to 1.0

b. 1.0 to 3.0

c. 3.0 to 4.0

d. 4.0 to 5.0

19. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

20. The factor which affects the compaction, is

a. compacting content

b. method of compaction

c. type of soil

d. All the above.

21. If the diameter of the main reinforcement in a slab is 16 mm, the concrete cover to main bars is

a. 13 mm

b. 14 mm

c. 15 mm

d. 16 mm

22. In a cantilever retaining wall without a heel slab

a. thickness of the stem is kept same throughout

b. base slab is made 10 cm thicker than the stem

c. width of the base slab is kept 0.7 time the total height of the wall

d. all the above.

23. The design of heel slab of a retaining wall, is based on the maximum bending moment due to:

a. weight of the soil above it,

b. load of the surcharge, if any

c. upward soil reaction

d. all the above.

24. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

25. In flat roof of reinforced cement concrete, the recommended angle of slope, is

a. zero

b. a few degrees

c. 10°

d. 200°.

26. To stagger vertical joints in successive courses of a wall, a piece of brick is generally used at the end of the course, which is known as

a. bat

b. header

c. stretcher

d. closer.

27. 1.S.I. has specified the full strength of concrete after

a. 14 days

b. 21 days

c. 28 days

d. 35 days

28. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

29. Carbonates in water produce

a. temporary hardness

b. permanent hardness

c. acidity

d. alkanity.

30. The total domestic consumption in a city water supply, is assumed

a. 20%

b. 30%

c. 40%

d. 60%.

31. Pick up the correct statement from the following :

a. Indore method of composting involves decomposition under aerobic conditions

b. Bangalore method of composting involves decomposing under anaerobic conditions

c. Fully stabilised refuse by the Bangalore method of compositing is a powdery mass called humus

d. all of these.

32. The grit and silt of the grit chambers, may not be used for

a. raising low lying areas by dumping

b. concreting

c. both (a) and (b)

d. neither (a) nor (b).

33. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

34. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

35. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

36. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

37. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

38. Width of vehicles affects the width of

a. lanes

b. shoulders

c. parking spaces

d. all the above.

39. Interior thickness of concrete road slab for design wheel load 6300 kg and permissible flexural stress 21 kg/cm2, is

a. 25.5 cm

b. 34.0 cm

c. 42.5 cm

d. 50.0 cm

40. At a road junction, 7 cross conflict points are severe if

a. both are one-way roads

b. both are two-way roads

c. one is two-way road and other is one-way road

d. none of these.

41. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

42. Anti-siphonage pipe is connected to

a. Main soil pipe

b. Bottom of P trap W.C.

c. Top of P trap W.C.

d. Side of water closet.

43. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

44. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

45. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

46. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

47. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

48. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

49. The threshold markings are

a. 1 m clear space between adjacents

b. 45 m in length

c. placed symmetrically on either side of the runway centre line

d. all the above.

50. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.