***Sub Engineer***

1. In tacheometrical observations, vertical staff holding is generally preferred to normal staffing, due to

a. ease of reduction of observations

b. facility of holding

c. minimum effect of careless holding on the result

d. none of these.

2. The bearings of two traverse legs AB and BC are N52° 45' E and N34° 30' E respectively. The deflection angle is

a. 18° 15' N

b. 18° 15' W

c. 18° 15' R

d. 18° 15' L

3. In a closed traverse, sum of south latitudes exceeds the sum of north latitudes and the sum of east departures exceeds the sum of west departures, then, the closing line will lie in

a. north-west quadrant

b. north east quadrant

c. south-east quadrant

d. south-west quadrant.

4. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

5. Pick up the correct statement from the following:

a. The bulking of sand is taken into account while volumetric proportioning of the aggregates

b. The dry sand and the sand completely flooded with water, have practically the same volume

c. The expansion and contraction joints are provided if concrete structures exceed 12 m in length

d. All the above.

6. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

7. Portland pozzolana cement possesses

a. lower heat of hydration

b. lower shrinkage on drying

c. water tightness

d. all the above.

8. Refractory bricks resist

a. high temperature

b. chemical action

c. dampness

d. all the above.

9. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

10. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

11. While testing a cast iron beam (2.5 cm x 2.5 cm) in section and a metre long simply supported at the ends failed when a 100 kg weight is applied at the centre. The maximum stress induced is :

a. 960 kg/cm2

b. 980 kg/cm2

c. 1000 kg/cm2

d. 1200 kg/cm2.

12. If the width b and depth d of a beam simply supported with a central load are interchanged, the deflection at the centre of the beam will be changed in the ratio of

a. d/b

b. (d/b)2

c. (b/d)2

d. (b/d)3.

13. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

14. An ideal fluid is

a. incompressible

b. compressible

c. compressible and non-viscous

d. slightly affected by surface torque.

15. The magnitude of water hammer in a pipe depends upon

a. length of the pipe line

b. elastic properties of the pipe material

c. elastic properties of the following liquid

d. all the above.

16. For the most economical trapezoidal open channel,

a. half of the top width must be equal to one of the sloping sides

b. the hydraulic mean depth must be equal to half the depth of flow

c. the semicircle drawn with top width as diameter must touch the three sides of the channel

d. All of these.

17. The specific gravity of sands, is approximately

a. 2.0

b. 2.2

c. 2.4

d. 2.6

18. The soil moisture driven off by heat, is called

a. free water

b. hydroscopic water

c. gravity water

d. none of these.

19. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

20. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

21. The reinforced concrete beam which has width 25 cm, lever arm 40 cm, shear force 6t/cm2, safe shear stress 5 kg/cm2 and B.M. 24 mt,

a. is safe in shear

b. is unsafe in shear

c. is over safe in shear

d. needs redesigning.

22. The width of the flange of a T-beam should be less than

a. one-third of the effective span of the T-beam

b. distance between the centres of T-beam

c. breadth of the rib plus twelve times the thickness of the slab

d. least of the above.

23. As the percentage of steel increases

a. depth of neutral axis increases

b. lever arm increases

c. lever arm decreases

d. none of these.

24. In a singly reinforced beam, if the permissible stress in concrete reaches earlier than that in steel, the beam section is called

a. under-reinforced section

b. over reinforced section

c. economic section

d. critical section.

25. Pick up the correct statement from the following :

a. D.P.C. should be continuous

b. D.P.C.   should  be  of good  impervious material

c. D.P.C. may be horizontal or vertical

d. All the above.

26. The depth of excavation of foundations, is generally measured with a

a. ranging rod

b. steel tape

c. levelling staff

d. bonning rod.

27. The bulk density of aggregates, is generally expressed as

a. kg/cubic metre

b. kg/litre

c. g/cm3

d. none of these.

28. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

29. Shales are

a. permeable

b. porous and permeable

c. porous but not permeable

d. neither porous nor permeable.

30. Corrosion of well pipes may not be reduced by

a. reducing the flow velocity

b. using thicker pipes

c. using screens having larger area of openings

d. none of these.

31. The formula which accepts the value of rugosity coefficient n = 0.012 to be used in Manning's formula, is given by

a. Bazin

b. Crimp and Bruge

c. William-Hazen

d. Kutter.

32. Removal of oil and grease from sewage, is known

a. screening

b. skimming

c. filtration

d. none of these.

33. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

34. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

35. In gravity canals, F.S.L. is

a. always at the ground level

b. always below the ground level

c. generally 4 to 5 metres above the ground level

d. only a few cm above the ground level.

36. For designing the venturi head,

a. width of the throat should not be less than the width obtained from the formula B = Q / 1.2d3/2, where d is the difference in levels of F.S.L. and sill

b. minimum throat width should not be less than 60 cm

c. length of the throat should be 2.5 times the depth measured to F.S.L. in the main canal

d. all the above.

37. The convexity provided to the carriageway between the crown and edge of the pavement, is known as

a. super-elevation

b. camber

c. height of the pavement

d. None of these.

38. Reference pillars fixed on the centre line of a proposed road, provide the following information:

a. horizontal distance of road from the centre line

b. reduced level at the top of pillar

c. formation level of the road

d. all the above.

39. Reinforcement in cement concrete slab of road pavements, is placed

a. transversely

b. longitudinally and transversely

c. in the form of welded wire mesh

d. none of these.

40. Increase in traffic volume, due to increase in transport vehicles, is known as

a. normal traffic growth

b. generated traffic growth

c. current traffic

d. none of these.

41. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

42. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

43. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

44. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

45. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

46. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

47. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

48. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

49. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.

50. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.