***Sub Engineer***

1. The minimum range for sliding the focusing lens in the internal focusing telescope for focusing at all distances beyond 4 m is

a. 5 mm

b. 10 mm

c. 15 mm

d. 20 mm.

2. While measuring a chain line between two stations A and B intervened by a raised ground

a. vision gets obstructed

b. chaining gets obstructed

c. both vision and chaining get obstructed

d. all the above.

3. If h1 and h2 are the differences in level between ground and the formation levels, m is the slope of the sloping sides. D is the distance between the cross sections then, prismoidal correction for a level section is

a. D/3m(h1 - h2)

b. D/6m(h1 - h2)2

c. D/6m(h1 - h2)3

d. D/6m(h1 + h2)2

4. If the radius of a simple curve is R, the length of the chord for calculating offsets by the method of chords produced, should not exceed.

a. R/10

b. R/15

c. R/20

d. R/25.

5. Age of a tree may be ascertained by

a. radius of its stem

b. circumference of its stem

c. number of branches

d. number of annual rings.

6. Dolomite is a lime stone which contains carbonate of magnesia upto

a. 20%

b. 25%

c. 35%

d. 45%.

7. The method of addition polymerization is used for obtaining:

a. polypropylene

b. polyvinylchloride

c. polystyrene

d. All of these.

8. Basalt is

a. sedimentary rock

b. metamorphic rock

c. extrusive igneous rock

d. intrusive igneous rock.

9. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

10. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

11. A beam of length L is pinned at both ends and is subjected to a concentrated bending couple of moment M at its centre. The maximum bending moment in the beam is

a. M

b. M/2

c. M/3

d. ML/2

12. The radius of gyration of a rectangular section is not proportional to

a. square root of the moment of inertia

b. square root of the inverse of the area

c. square root of the moment of inertia divided by area of the section

d. none of these.

13. The equation  for the viscosity, is suggested

a. Newton

b. Chezy

c. Bezin

d. Helmholtz.

14. The value of momentum correction factor (β) for a laminar flow through a circular pipe, is

a. 2/3

b. 3/4

c. 4/3

d. 3/2

15. In a short cylindercial external mouthpiece, the venacontracta occurs at a distance from the outlet of orifice equal to

a. diameter of the orifice

b. one-fourth the diameter of the orifice

c. one-third the diameter of the orifice

d. two-third the diameter of the orifice

16. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

17. Pick up the correct statement from the following:

a. The porosity of a soil is defined as the ratio of the volume of voids to the gross volume of the soil

b. The bulk density of a soil is defined as the unit weight of the soil

c. The dry density of a soil is defined as weight of solids to the total volume of the soil

d. All the above.

18. The minimum depth of building foundations on

a. sandy soils is 80 cm to 100 cm

b. clay soils is 90 cm to 160 cm

c. rocky soils is 5 cm to 50 cm

d. all the above.

19. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

20. The shear strength of a soil

a. is proportional to the cohesion of the soil

b. is generally known as the strength of the soil

c. is proportional to the tangent of the angle of internal friction

d. all the above.

21. The minimum cube strength of concrete used for a prestressed member, is

a. 150 kg/cm2

b. 250 kg/cm2

c. 350 kg/cm2

d. 400 kg/cm2

22. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

23. The maximum diameter of a bar used in a ribbed slab, is

a. 6 mm

b. 20 mm

c. 22 mm

d. 24 mm

24. Bottom bars under the columns are extended into the interior of the footing slab to a distance greater than

a. 42 diameters from the centre of the column

b. 42 diameters from the inner edge of the column

c. 42 diameters from the outer edge of the column

d. 24 diameter from the centre of the column

25. A stair should not have pitch more than

a. 30°

b. 40°

c. 50°

d. 60°.

26. The maximum bearing capacity of soil is that of

a. loose fine sandy soil

b. dry coarse sandy soil

c. hard rocks

d. soft clay soil.

27. According to IS : 382-1963, a good aggregate should be

a. chemically inert

b. sufficiently strong

c. hard and durable

d. all the above.

28. Pick up the incorrect statement from the following:

a. With passage of time, the strength of cement increases

b. With passage of time, the strength of cement decreases

c. After a period of 24 months, the strength of cement reduces to 50%

d. The concrete made with storage deteriorated cement, gains strength with time

29. If Q is discharge is cubicmetres per sec and D is the economical diameter of the pipe. According to Lea

a. D = 0.67 to 0.87 Q

b. D = 0.77 to 0.97 Q

c. D = 0.97 to 1.22 Q

d. D = 1.22 to 1.33 Q.

30. One degree of hardness of water means a content of salts of

a. 12.25 mg/litre

b. 14.25 mg/litre

c. 16.25 mg/litre.

d. 20 mg/litre.

31. For a continuous flow type of sedimentation tanks

a. length of the tank is normally kept 4 to 5 times the width

b. maximum horizontal flow velocity is limited to 0.3 m/minute

c. capacity of the tank for 24 hours, is kept equal to maximum daily flow of sewage

d. all the above.

32. It is customary to design a sewer for D.W.F. on the basis of

a. average demand

b. twice the average demand

c. thrice the average demand

d. four times the average demand.

33. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

34. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

35. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

36. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

37. As per recommendations of I.R.C., traffic volume study is carried out for rural roads for 7 days continuously during

a. harvesing

b. lean season

c. harvesting and lean season

d. none of these.

38. In water bound macadam roads, binding material, is

a. sand

b. stone dust

c. cement

d. brick dust.

39. To prevent compressive stresses in a rigid concrete pavement, the joint prvoided, is

a. expansion joint

b. contraction joint

c. hinged joint

d. all the above.

40. Border Roads Organisation for hilly regions, was formed in

a. 1954

b. 1958

c. 1960

d. 1962

41. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

42. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

43. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

44. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

45. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

46. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

47. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

48. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

49. Pick up the incorrect statement from the following:

a. In single engine aeroplanes, the engine is provided in the nose of the aircraft

b. In double engine aeroplanes, one engine on either wing is placed symmetrically

c. In three engine aeroplanes, two engines are placed on both wings and one engine is placed in the tail

d. None of these.

50. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m