***Sub Engineer***

1. True meridian of different places

a. converge from the south pole to the north pole

b. converge from the north pole to the south pole

c. converge from the equator to the poles

d. run parallel to each other.

2. If the rate of gain of radial acceleration is 0.3 m per sec3 and full centrifugal ratio is developed. On the curve the ratio of the length of the transition curve of same radius on road and railway, is

a. 2.828

b. 3.828

c. 1.828

d. 0.828.

3. In a precision traverse, included angles are measured by setting the vernier

a. to read zero exactly on back station

b. to read 5° exactly on back station

c. some where near zero and reading both verniers on back station

d. all the above.

4. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

5. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

6. The size of modular bricks, is

a. 10 x 10 x 9 cm

b. 19 x 9 x 9 cm

c. 22.5 x 10 x 8.5 cm

d. 22.5 x 8.0 x 9 cm.

7. A prime coat is given to steel work with

a. an oxide of iron paint

b. a mixture of white lead and lead paint

c. a special paint

d. cement paint.

8. Quartzite, a metamorphic stone is

a. brittle

b. crystalline

c. compact

d. All the above.

9. In the truss, the force in the member AC is

a. 6.25 t compressive

b. 8.75 t tentile

c. t tensile

d. t compressive.

10. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

11. A 8 metre long simply supported rectangular beam which carries a distributed load 45 kg/m. experiences a maximum fibre stress 160 kg/cm2. If the moment of inertia of the beam is 640 cm4, the overall depth of the beam is

a. 10 cm

b. 12 cm

c. 15 cm

d. 16 cm

12. The areas of cross-section of a square beam and a circular beam subjected to equal bending moments, are same.

a. square beam is more economical

b. both the beams are equally strong

c. both the beams are equally economical

d. none of these.

13. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

14. A pipe of 0.1 m2 cross sectional area suddenly enlarges to 0.3 m2 cross-sectional area. If the discharge of the pipe is 0.3 m3 /sec, the head loss is

a. 2/g m of water

b. g/2 m of water

c. 1g m of water

d. g m of water.

15. Fluids change the volume under external presssure due to

a. plasticity

b. viscosity

c. compressibility

d. none of these.

16. While applying the Bernoulli's equation  = total head, the work any section done on the flow system, if any

a. is added on the right side of the equation

b. is added on the left side of the equation

c. is ignored

d. none of these.

17. The critical exist gradient of seepage water in soils, increases with

a. a decrease in specific gravity

b. a decrease in void ratio

c. both (a) and (c)

d. none of these.

18. Which one of the following statements is true ?

a. Clays are more porous than sands

b. Pressure of organic matter in a soil decreases the bearing capacity of the soil

c. Aluminous cement is used for foundations in soils with chemical deposits

d. All the above.

19. For determing the specific gravity of soil solids, using a pycnometer of 500 cc., the following data is available : Weight of dry empty pycnometer = 125 g Weight of dried soil and pycnometer = 500 g Weight of dried soil and distilled = 850 g water filled in pycnometer up to top The specific gravity of soil soilds, is

a. 2.0

b. 2.25

c. 2.50

d. 2.75

20. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

21. The maximum area of tension reinforcement in beams shall not exceed

a. 0.15%

b. 1.5%

c. 4%

d. 1%

22. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

23. The maximum shear stress (qmax) in a rectangular beam is

a. 1.50 times the average

b. 1.75 times the average

c. 2.0 times the average

d. 2.5 times the average.

24. According to load factor method, the permissible load W on a short column reinforced with longitudinal bars and lateral stirrups, is

a. Stress in concrete x area of concrete

b. Stress in steel x area of steel

c. Stress in concrete x area of concrete + Stress in steel x area of steel

d. None of these.

25. The under surface of an arch, is called

a. soffit

b. intrados

c. haunch

d. back.

26. Slate

a. is a metamorphic rock

b. splits into thin sheets along its bedding planes

c. has a smooth surface and contains alumina and silica

d. possesses good water absorption capacity

27. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

28. Curing of pavements, floors, roofs and slabs, is done by

a. ponding method

b. covering surface with bags

c. sprinkling water method

d. shading concrete method.

29. Pick up the correct statement from the following :

a. Water with hardness more than 200 ppm is considered hard

b. Water needed for laundaries should contain hardness less than 75 ppm

c. To provide taste during drinking, water should possess hardness between 75 to 120

d. All the above.

30. 'Shrouding' is essentially provided in

a. strainer type wells

b. cavity type wells

c. slotted type well

d. all the above.

31. The rate of accumulation of sludge in septic tanks is recommended as

a. 30 litres/person/year

b. 25 litres/person/year

c. 30 litres/person/month

d. 25 litres/person/month.

32. Which one of the following resists hydrogen sulphide corrosion

a. R.C.C.

b. Glazed stone wares

c. Asbestos cement

d. Glazed ware.

33. Regime conditions in a channel may occur if

a. discharge is constant

b. channel flows uniformly in incoherent alluvium as that transported in suspension

c. silt grade and silt charge are constant

d. all the above.

34. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

35. When a canal is carried over a natural drainage, the structure provided, is known as

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

36. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

37. If L is the length of the transition curves provided on either side of a circular curve of radius R, the maximum angle of deflection with tangent for the junctions of the transition curve and circular curve, is

a. L/2R

b. L/3R

c. L/4R

d. none of these.

38. The minimum ratio of the radii of two circular curves of a compound curve, is kept

a. 1.25

b. 1.5

c. 1.75

d. 2.0

39. For the movement of vehicles at an intersection of two roads, without any interference, the type of grade separator generally preferred to, is

a. delta

b. trumpet

c. diamond interchange

d. clover leaf.

40. Traffic surveys are carried out

a. to know the type of traffic

b. to determine the facilities to traffic regulations

c. to design proper drainage system

d. all the above.

41. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

42. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

43. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

44. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

45. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

46. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

47. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

48. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

49. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.

50. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.