***Sub Engineer***

1. If h is the difference in level between end points separated by l, then the slope correction is  . The second term may be neglected if the value of h in a 20 m distance is less than

a.  m

b. 1 m

c. 2 m

d. 3 m

2. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

3. Pick up the method of surveying in which field observations and plotting proceed simultaneously from the following

a. chain surveying

b. compass surveying

c. plan table surveying

d. tacheometric surveying.

4. The latitude of a traverse leg is obtained by multiplying its length by

a. tangent of its reduced bearing

b. sign of its reduced bearing

c. cosine of its reduced bearing

d. cosecant of its reduced bearing.

5. Polymerization helps to improve the property of

a. strength

b. rigidity

c. elasticity

d. all of these.

6. Pick up the hypabyssal rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

7. For melting one tonne of cast iron

a. 700 m3 air is required

b. 20 kg limestone is required

c. one quintal coke is required

d. all the above.

8. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

9. To determine the force in BD of the truss shown in the given figure a section is passed through BD, CD and CE, and the moments are taken about

a. A joint

b. B joint

c. C joint

d. D joint.

10. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

11. The width b and depth d of a beam cut from a wooden cylindrical log of 100 cm diameter for maximum strength are :

a. b = 57.73 cm d = 81.65 cm

b. b = 81.65 cm d = 57.73 cm

c. b = 50.00 cm d = 50.00 cm

d. b = 40.00 cm d = 80.00 cm

12. In a square beam loaded longitudinally, shear develops

a. on lower fibre along horizontal plane

b. on top fibre along vertical plane

c. equally on each fibre along horizontal plane

d. none of these.

13. Discharge with velocity of approach, over a rectangular weir is

a.  Cd x L x 2g [(H + ha)3/2 + ha3/2]

b.  Cd x L x 2g [(H + ha)3/2 - ha3/2]

c.  Cd x L x 2g [(H - ha)3/2 + ha3/2]

d.  Cd x L x 2g [(H - ha)3/2 - ha3/2]

14. Uniform flow is said to occur when

a. size and shape of the cross-section in a particular length remain constant

b. size and shape of the cross-section change along a length

c. frictional loss in the particular length of the channel will the more than the drop in its elevation

d. frictional loss in the particular length of the channel, will be less than the drop in elevation.

15. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

16. In a centrifugual pump casing, the flow of water leaving the impeller, is

a. rectilinear flow

b. radial flow

c. free vortex motion

d. forced vertex.

17. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

18. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

19. If water content of a soil is 40%, G is 2.70 and void ratio is 1.35, the degree of saturation is

a. 75%

b. 80%

c. 85%

d. 90%

20. Under-reamed piles are generally

a. driven piles

b. bored piles

c. precast piles

d. all the above.

21. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

22. If T and R are tread and rise respectively of a stair, then

a. 2R + T = 60

b. R + 2T = 60

c. 2R + T = 30

d. R + 2T= 30

23. A T-beam behaves as a rectangular beam of a width equal to its flange if its neutral axis

a. remains within the flange

b. remains below the slab

c. coincides the geometrical centre of the beam

d. none of these.

24. Design of a two way slab simply supported on edges and having no provision to prevent the corners from lifting, is made by

a. Marcus formula

b. Rankine Grashoff formula

c. Grashoff formula

d. Rankine-Marcus formula.

25. The foundation which consists of a thick reinforced cement slab covering whole area to support heavy concentrated structural loads, is known as

a. combined footing

b. strap footing

c. raft footing

d. none of these.

26. Pile foundation is generally provided if soil is

a. compressible

b. water logged

c. made up

d. all the above.

27. Percentage of pozzolanic material containing clay upto 80% used for the manufacture of pozzolana cement, is

a. 30%

b. 40%

c. 50%

d. 60%

28. The entrained air in concrete

a. increases workability

b. decreases workability

c. decreases resistance to weathering

d. increases strength

29. Water to the consumers may be supplied from

a. infiltration galleries connected to sump well

b. infiltration well dug out on the banks of rivers

c. ranney wells sunk to the water level

d. all the above.

30. Rapid gravity filters can remove bacterial impurities up to a maximum of

a. 60%

b. 70%

c. 80%

d. 90%.

31. If the diameter of sewer is 225 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

32. Which one of the following part of human body withstands minimum radiation

a. thyroid

b. kidneys

c. eyes

d. ovaries/testis.

33. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

34. If straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the area of its cross-section, is

a. R(θ + cot θ)

b. R2(θ + tan θ)

c. R2(θ + cot θ)

d. R(θ + tan θ)

35. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

36. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

37. On most smooth hard surfaced roads, rolling resistance to moving vehicles, ranges from

a. 7 kg to 9 kg/tonne

b. 9 kg to 11 kg/tonne

c. 11 kg to 13 kg/tonne

d. none of these.

38. According to the recommendations of Nagpur Conference, the width formation of an ideal National Highway in hard rock cutting, is

a. 7.9 m

b. 6.9 m

c. 6.5 m

d. 7.5 m

39. For maximum strength and durability minimum percentage of cement, by weight is

a. 20%

b. 25%

c. 30%

d. 33%

40. The width of pavement in addition to a gravelled berm 1 m on either side for a two directional traffic recommended by Nagpur Conference of Chief Engineers, is

a. 5.7 m

b. 6.7 m

c. 7.7 m

d. 8.7 m

41. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

42. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

43. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

44. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

45. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

46. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

47. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

48. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

49. For Class A Air port the difference of reduced levels of higher and lower edges of the conical surface, is

a. 25 m

b. 50 m

c. 75 m

d. 100 m

50. For the taxiways, the following statement is true

a. The maximum longitudinal grade is 3%

b. The permissible rate of change of grade is 1%

c. The permissible transverse grade is 1.5%

d. All the above.