***Sub Engineer***

1. Pick up the correct statement from the following:

a. the theodolite in which telescope can be rotated in vertical plane is called a transit

b. when the vertical circle is to the left of the telescope during observation, it is called to be in left face

c. when the vertical circle is to the right of the telescope during observation, it is called to be in right face

d. all the above.

2. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

3. If + 0.8% grade meets - 0.7% grade and the rate of change of grade for 30 m distance is 0.05, the length of the vertical curve will be

a. 700 m

b. 800 m

c. 900 m

d. 1000 m

4. Which one of the following mistakes/errors may be cumulative + or - :

a. bad ranging

b. bad straightening

c. erroneous length of chain

d. sag.

5. In a rock calcium carbonate predominates. State whether it is :

a. Sillicious rock

b. Argillaceous rock

c. Calcareous rock

d. None of these.

6. In stone masonry, if stones are so placed that their layers are parallel to the direction of load, they

a. split easily

b. are affected by moisture

c. both (a) and (b)

d. none of these.

7. Asbestos is

a. an incombustible fire proof material

b. an organic substance

c. a bad insulator for sound and heat

d. all the above.

8. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

9. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

10. If a concrete column 200 x 200 mm in cross-section is reinforced with four steel bars of 1200 mm2 total cross-sectional area. Calculate the safe load for the column if permissible stress in concrete is 5 N/mm2 and Es is 15 Ec

a. 274 MN

b. 284 MN

c. 294 MN

d. None of these.

11. When a rectangular beam is loaded transversely, the maximum compressive stress develops on

a. bottom fibre

b. top fibre

c. neutral axis

d. every cross-section.

12. A three-hinged arch is said to be :

a. statically determinate structure

b. statically indeterminate structure

c. a bent beam

d. none of these.

13. The Empirical formula  for discharge over large rectangular weirs, is known as

a. Francis formula

b. Bazin formula

c. Rehbook formula

d. Kutter's formula.

14. The side slope of Cipolletti weir is generally kept

a. 1 to 4

b. 1 to 3

c. 1 to 2

d. 1 : 5

15. A pitot tube is used to measure

a. pressure

b. difference in pressure

c. velocity of flow

d. none of these.

16. A rectangular channel 6 m wide and 3 m deep and having a bed slope as 1 in 2000 is running full. If Chezy's constant C = 54.8, pick up the correct specification of the channel from the following :

a. hydraulic mean depth = 1.5 m

b. Velocity of flow = 1.5 m/sec

c. Rate of flow = 27 m3/sec

d. All the above.

17. If drainage is permitted throughout the test, during the application of both normal, and shear stresses so that full consolidation occurs and no excess pore pressure is set up at any stage of the test, is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

18. The lateral earth pressure on a retaining wall

a. proportional to the depth of the soil

b. proportional to the square of the depth of the soil

c. proportional to the internal friction of the soil

d. none of these

19. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit

20. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

21. A pre-stressed concrete member is preferred because

a. its dimensions are not decided from the diagonal tensile stress

b. large size of long beams carrying large shear force need not be adopted

c. removal of cracks in the members due to shrinkage

d. all the above.

22. In a slab, the pitch of the main reinforcement should not exceed its effective depth

a. three times

b. four times

c. five times

d. two times.

23. By over-reinforcing a beam, the moment of resistance can be increased not more than

a. 10%

b. 15%

c. 20%

d. 25%

24. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

25. The pile provided with one or more bulles in its vertical shaft, is generally known as

a. under-ream pile

b. friction pile

c. bearing pile

d. sheet pile.

26. A projecting piece usually provided to support a truss, is

a. cornice

b. coping

c. frieze

d. lintal.

27. The standard sand now a days used in India, is obtained from

a. Jullundur (Punjab)

b. Hyderabad (Andhra Pradesh)

c. Ennore (Madras)

d. Cuttuck (Orissa).

28. Log Angles machine is used to test the aggregate for

a. impact value

b. abrasion resistance

c. water absorption

d. none of these.

29. Derivation of Thiem's formula  is based on the assumption

a. the well is sunk through the full depth of the aquifer

b. the flow lines are radial and horizontal, and the flow is laminar

c. for hydraulic gradient in Darcy's equation, tangent value is used for sine value

d. all the above.

30. Rapid gravity filters can remove bacterial impurities up to a maximum of

a. 60%

b. 70%

c. 80%

d. 90%.

31. The algae dies out in the zone of

a. degradation

b. active decomposition

c. recovery

d. cleaner water.

32. The minimum diameter of sewer to be adopted is

a. 10 cm

b. 12.5 cm

c. 15 cm

d. 25 cm.

33. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

34. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

35. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

36. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

37. Shoulders for high traffic volume roads, should

a. be stable throughout the year to be used by vehicles in the case of emergency

b. support and protect the ends of carriage ways

c. not allow entrance of water to sub-grade

d. all the above.

38. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

39. Which one of the following transportation systems is not used for the conveyance of passengers in India?

a. roads

b. shipping

c. air lines

d. pipe lines.

40. The absolute minimum sight distance required for stopping a vehicle moving with a speed of 80 km ph, is

a. 120 m

b. 200 m

c. 640 m

d. none of these.

41. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

42. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

43. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

44. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

45. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

46. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

47. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

48. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

49. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.

50. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m