***Sub Engineer***

1. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

2. For locating a distant object visible from two transit stations, the method usually preferred to, is

a. Angles and distances from transit stations

b. Angles from two transit stations

c. distance from two transit stations

d. Angle from one transit station and distance from the other.

3. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

4. Number of subdivisions per metre length of a levelling staff is

a. 100

b. 200

c. 500

d. 1000

5. The cast iron when heated to red heat with powdered red haemetite in an oven for increasing its toughness, is converted to

a. grey cast iron

b. white cast iron

c. mottled cast iron

d. toughed cast iron.

6. German silver is an alloy of

a. silver, gold and lead

b. copper, nickel and zinc

c. copper, brass and zinc

d. brass, silver and zinc.

7. Jhumb bricks are

a. under burnt

b. over burnt

c. kutcha

d. none of these.

8. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

9. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

10. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

11. Pick up the correct statement from the following :

a. The rate of change of bending moment is equal to rate of shear force

b. The rate of change of shear force is equal to rate of loading

c. neither (a) nor (b)

d. both (a) and (b).

12. The maximum bending moment due to a moving load on a simply supported beam, occurs

a. at the supports

b. under the load

c. anywhere on the beam

d. none of these.

13. In a venturimeter, the divergent cone is kept

a. shorter than convergent cone

b. equal to covergent cone

c. longer than convergent cone

d. none of these.

14. Pick up the incorrect statement from to following regarding triangular notch :

a. For measuring low discharge, it gives more accurate result

b. Only one reading (i.e. H) is required for computation of discharge

c. Ventilation is necessary

d. None of these.

15. If total head available at the inlet of pipe and f1 is the loss of head due to friction in the pipe, the maximum efficiency of transmission of power (ηmax) is

a. 1/2

b. 2/3

c. 3/4

d. 4/5

16. In flowing liquids pitot tubes are used measure

a. discharge

b. pressure

c. velocity

d. depth.

17. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

18. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

19. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

20. The ratio of the volume of voids to the volume of soil solids in a given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

21. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

22. The diameter of the column head support a flat slab, is generally kept

a. 0.25 times the span length

b. 0.25 times the diameter of the column

c. 4.0 cm larger than the diameter of the column

d. 5.0 cm larger than the diameter of the column

23. The live load to be considered for an accessible roof, is

a. Nil

b. 75 kg/m3

c. 150 kg/m2

d. 200 kg/cm2

24. A pre-stressed concrete member is preferred because

a. its dimensions are not decided from the diagonal tensile stress

b. large size of long beams carrying large shear force need not be adopted

c. removal of cracks in the members due to shrinkage

d. all the above.

25. Couple roof is used for spans

a. 3.5 m or less

b. 3.5 m but less than 5 m

c. 5 m but less than 6.5 m

d. 6.5 m but less than 8 m.

26. The arrangement of supporting an existing structure by providing supports underneath, is known as

a. shoring

b. underpinning

c. jacking

d. piling

27. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

28. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

29. Pick up the incorrect statement from the following regarding fire hydrants

a. Fire hydrants are fitted in water mains at 100 m to 150 m apart at fire

b. The minimum water pressure hydrants, is kept 1.5 kg/cm2

c. The water at pressure 1 to 1.5 kg/cm2 is made available for 4 to 5 hours for constant use

d. None of these.

30. Turbidity of raw water is a measure of

a. suspended solids

b. acidity of water

c. B.O.D.

d. none of these.

31. The sewage treatment units in which anaerobic decomposition of organic matter is used, are called

a. imhoff tanks

b. trickling filters

c. sludge sedimentation tanks

d. none of these.

32. Bio-chemical oxygen demand (BOD) for the first 20 days in generally referred to

a. initial demand

b. first stage demand

c. carbonaceous demand

d. all of these.

33. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

34. A deficit of sediments in flowing water may cause a river

a. meandering type

b. aggrading type

c. degrading type

d. sub-critical type.

35. The width of a dowla is generally kept between 30 to 60 cm and its height above the road level should invariably be more than

a. 10 cm

b. 20 cm

c. 30 cm

d. 40 cm

36. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

37. The ideal shape of a transition curve, is

a. clothoid

b. cubic spiral

c. cubic parabola

d. lamniscate

38. If present A.D.T. is 5000 vehicles and annual increase is 10%, the average future flow after 5 years will be

a. 6050 vehicles

b. 7050 vehicles

c. 8050 vehicles

d. 9050 vehicles

39. The top height of a route marker above crown level is

a. 1.50 m

b. 1.75 m

c. 2.00 m

d. 2.25 m

40. The weaving length of a roadway is the distance

a. between the channelising islands

b. equal to half circumference

c. equal to total width of adjoining radial roads

d. equal to diameter of rotary.

41. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

42. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

43. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

44. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

45. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

46. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

47. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

48. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

49. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.

50. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10