***Sub Engineer***

1. Probable systematic error in precise levelling as recommended by International Geodetic Association should not exceed (where k is in kilometers.)

a. ± 0.1 k mm

b. ± 0.2 k mm

c. ± 0.1 k

d. 0.2 k mm.

2. The bearing of lines OA and OB are 16° 10' and 332° 18', the value of the included angle BOA is

a. 316° 10'

b. 158° 28'

c. 348° 08'

d. 43°  52'

3. In a lemniscate curve the ratio of the angle between the tangent at the end of the polar ray and the straight, and the angle between the polar ray and the straight, is

a. 3

b. 4

c. 5

d. 3/2

4. The boundary of water of a still lake, represents

a. level surface

b. horizontal surface

c. contour line

d. a concave surface.

5. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

6. Glazing of clay products, is done

a. to improve their appearance

b. to protect them from atmospheric effect

c. to protect them from corrosive action

d. all the above.

7. Stainless steel contains

a. 18% of chromuim and 8% nickel

b. 8% of chromium and 18% of nickel

c. 12% of chromium and 36% of nickel

d. 36% of chromium and 12% of nickel.

8. Pick up the correct characteristic of Pyroxene from the following:

a. It converts to chlorine by hydration

b. Its density is 2.3 to 3.6 g/cm2

c. Its hardness is 5 to 6

d. All the above.

9. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

10. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

11. When a rectangular beam is loaded transversely, the maximum compressive stress develops on

a. bottom fibre

b. top fibre

c. neutral axis

d. every cross-section.

12. A diagram which shows the variations of the axial load for all sections of the span of a beam, is called

a. shear force diagram

b. thrust diagram

c. stress diagram

d. none of these.

13. The phenomenon occuring in an open channel when a rapidly flowing stream abruptly changes to a slowly flowing stream causing a distinct rise of liquid surface, is

a. water hammer

b. hydraulic jump

c. critical discharge

d. none of these.

14. In flowing liquids pitot tubes are used measure

a. discharge

b. pressure

c. velocity

d. depth.

15. The momentum correction factor (β) for the viscous flow through a circular pipe is

a. 1.33

b. 1.50

c. 1.66

d. 2.00

16. Power transmitted through a pipe is maximum when friction head loss, is

a. one-half of the total head supplied

b. one-third of the total head supplied

c. one-fouth of the total head supplied

d. equal to the total head supplied.

17. Depending upon the properties of a material, the failure envelope may

a. be either straight or curved

b. pass through the origin of stress

c. intersect the shear stress axis

d. all the above.

18. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

19. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

20. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

21. If W is weight of a retaining wall and P is the horizontal earth pressure, the factor of safety against sliding, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

22. As per IS : 456, the reinforcement in a column should not be less than

a. 0.6% and not more than 6% of cross-see-tional area

b. 0.7% and not more than 7% of cross-sec-tional area

c. 0.8% and not more than 8% of cross-sectional area

d. none of these.

23. Bottom bars under the columns are extended into the interior of the footing slab to a distance greater than

a. 42 diameters from the centre of the column

b. 42 diameters from the inner edge of the column

c. 42 diameters from the outer edge of the column

d. 24 diameter from the centre of the column

24. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

25. To obtain good bonding in brick masonry

a. first class bricks are used

b. vertical joints in alternate courses are kept in plumb line

c. bats are used where necessary

d. all the above.

26. The depth of an arch is the distance between

a. ground level and springing line

b. crown and springing line

c. crown and ground level

d. intrados and extrados.

27. For batching 1:3:6 concrete mix by volume, the ingredients required per bag of 50 kg cement, are:

a. 70 kg of sand and 140 litres of aggregates

b. 105 litres of sand and 140 litres of aggregates

c. 105 litres of sand and 210 litres of aggregates

d. none of these.

28. Pick up the correct statement from the following:

a. Angular and rough aggregates reduce the workability of the concrete

b. Large size aggregates increase the workability due to lesser surface area

c. The slump of the concrete mix decreases due to an increase in temperature

d. All the above.

29. The maximum permitted loss of head in a rapid sand filter, is

a. 2 m

b. 3 m

c. 4 m

d. 5 m.

30. The dilution ratio at which the odour is hardly detectable is generally called threshold odour number and for public supplies it should not exceed

a. 3

b. 5

c. 7

d. 9

31. If the diameter of a sewer is 100 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

32. Drop manholes at the junctions of sewer lines, are provided if

a. invert level of a branch sewer is more than 60 cm that of the main sewer

b. sewer line runs along a main road

c. ordinary manhole cannot be built

d. two sewer lines intersect.

33. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

34. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

35. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

36. Cross regulators in main canals are provided

a. to regulate water supply in the distributaries

b. to increase water head upstream when a main canal is running with low supplies

c. to overflow excessive flow water

d. none of these.

37. Before providing super-elevation on roads, the portion of the carriageway between the crown and the outer edge is made

a. to have a reduced fall

b. horizontal

c. to have slope of the camber on the other half of the carriageway

d. none of these.

38. Pick up the correct statement from the following:

a. During reconnaissance, the general route of the alignment is selected

b. After reconnaissance, a trace is cut for the alignment

c. Last stage is the detailed surveys for desired geometries'of the highway

d. All the above.

39. The ability of a driver to stop the vehicle moving with the designed speed, depends upon:

a. brake reaction time

b. efficiency of the brakes

c. frictional resistance between road surface and vehicle

d. all the above.

40. To prevent movement of moisture from subgrade to road pavement at the same level as that of water-table, thickness of a cut off layer of coarse sand, is

a. 15 cm

b. 20 cm

c. 30 cm

d. 45 cm

41. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

42. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

43. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

44. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

45. Final technical authority of a project lies with

a. Assistant Engineer

b. Executive Engineer

c. Superintending Engineer

d. Chief Engineer.

46. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

47. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

48. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

49. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.

50. Pick up the correct abbreviation from the following:

a. V.H.F, (very high frequency)

b. L.O.M. (low powered outer marker)

c. L.M.M. (low powered middle marker)

d. All the above