***Sub Engineer***

1. Pick up the correct statement from the following :

a. The directions of plumb lines suspended at different points in a survey are not strictly parallel

b. In surveys of small extent, the effect of curvature may be ignored and the level surface of the earth is assumed as horizontal

c. In surveys of large extent, the effect of curvature of the earth must be considered

d. All the above.

2. Keeping the instrument height as 1.5 m, length of staff 4 m, the slope of the ground as 1 in 10, the sight on the down-slope, must be less than

a. 30 m

b. 25 m

c. 20 m

d. 15 m

3. Metric chains are generally available in

a. 10 m and 20 m length

b. 15 m and 20 m length

c. 20 m and 30 m length

d. 25 m and 100 m length

4. In reciprocal levelling, the error which is not completely eliminated, is due to

a. earth's curvature

b. non-adjustment of line of collimation

c. refraction

d. non-adjustment of the bubble tube.

5. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

6. Pick up the compound responsible for early strength of cement from the following:

a. Tetra-calcium alumino-ferrite

b. Tricalcium silicate

c. Tricalcium aluminate

d. Dicalcium silicate.

7. For one cubic metre of brick masonry, number of bricks required, is

a. 425

b. 450

c. 500

d. 550.

8. Plaster of Paris is obtained by calcining

a. gypsum

b. kankar

c. lime stone

d. none of these.

9. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

10. A close coil helical spring when subjected to a moment M having its axis along the axis of the helix

a. it is subjected to pure bending

b. its mean diameter will decrease

c. its number of coils will increase

d. all the above.

11. The ratio of the tensile stress developed in the wall of a boiler in the circumferential direction to the tensile stress in the axial direction, is

a. 4

b. 3

c. 2

d. 1

12. A member is balanced at its end by two inclined members carrying equal forces. For equilibrium the angle between the inclined bars must be

a. 45°

b. 60°

c. 90°

d. 120°

13. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

14. Fluids change the volume under external presssure due to

a. plasticity

b. viscosity

c. compressibility

d. none of these.

15. The imaginary line drawn such that the tangents at its all points indicate the direction of the velocity of the fluid particles at each point, is called

a. path line

b. stream line

c. potential line

d. streak line.

16. A triangular notch is preferred to a rectangular notch because

a. its formula is simple to remember

b. it gives more accurate results for low discharge

c. it measures a wide range of flows accurately

d. all the above.

17. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

18. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

19. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

20. The seepage exit gradient in a soil is the ratio of

a. flow line to slope

b. head upstream to that at downstream

c. head loss to the length of the seepage

d. none of these.

21. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

22. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

23. If the effective length of a 32 cm diameter R.C.C. column is 4.40 m, its slenderness ratio, is

a. 45

b. 50

c. 55

d. 60

24. Design of R.C.C. simply supported beams carrying U.D.L. is based on the resultant B.M. at

a. supports

b. mid span

c. every section

d. quarter span.

25. Which one of the following factors is considered for the orientation of buildings :

a. the direction of the prevailing winds in the area

b. the exposure of the walls and roof of the buildings to the rays of sun

c. the extent up to which the sunrays penetrate with the verandah.

d. all the above.

26. A solid core of rock is formed in side the cylinder in the case of

a. auger boring

b. percussion drilling

c. diamond drilling

d. wash boring.

27. Pick up the correct statement from the following:

a. Concrete using sand stones, cracks due to excessive shrinkage

b. Very hard and close grained crystallined lime stones are suitable aggregates but provide low strength

c. Broken bricks produce a concrete having good fire resisting qualities

d. All the above.

28. The compaction of concrete, improves

a. density

b. strength

c. durability

d. all the above.

29. Biochemical Oxygen Demand (B.O.D.) of safe drinking water must be

a. nil

b. 5

c. 10

d. 15

30. If discharge of a pump is 0.16 cumecs, the economic diameter of pipe, is

a. 0.488 m

b. 4.88 cm

c. 0.488 cm

d. 4.88 m

31. During putrifaction process of sewage the gas given off, is

a. hydrogen

b. ammonia

c. methane

d. all the above.

32. For design purposes, the normal rate of infiltration of ground water into the sewer, is

a. 1000 litres/km/cm

b. 1500 litres/km/cm

c. 2000 litres/km/cm

d. 2500 litres/km/cm.

33. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

34. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

35. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

36. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

37. The gradients of trace cuts for hilly roads, are kept

a. equal to ruling gradient

b. 10 tO 20 per cent easier than ruling gradients

c. 10 to 20 per cent steeper than ruling gradients

d. none of these.

38. In case of a multi-lane road, overtaking is generally permitted

a. from right

b. from left

c. from both sides right and left

d. not at all.

39. Intermediate catch water drains are provided only, if

a. catchment area of the watershed above road is large

b. intensity of rainfall is heavy

c. single catch water drain is inadequate

d. all the above.

40. Gradient resistance of moving vehicles along down slopes, is

a. + 7 kg/tonne

b. + 9 kg/tonne

c. - 9 kg/tonne

d. - 7 kg/tonne

41. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

42. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

43. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

44. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

45. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

46. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

47. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

48. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

49. The maximum length and pavement strength of the runway is that of

a. A 1

b. B 2

c. B 3

d. G 7

50. The bearing of the runway at threshold is 290°, the runway number is

a. N 70° W

b. 290°

c. 29°

d. W 20° N