***Sub Engineer***

1. If the smallest division of a vernier is longer than the smallest division of its primary scale, the vernier is known as

a. direct vernier

b. double vernier

c. retrograde vernier

d. simple vernier.

2. Orientation of a plane table by solving two point problem is only adopted when

a. saving of time is a main factor

b. better accuracy is a main factor

c. given points are inaccessible

d. none of these.

3. Cross hairs in surveying telescopes, are fitted

a. in the objective glass

b. at the centre of the telescope

c. at the optical centre of the eye piece

d. in front of the eye piece.

4. The ratio of the length of long chord and the tangent length of a circular curve of radius R deflecting through angle Δ, is

a. cos

b. tan

c. 2 sin

d. 2 cos .

5. Pick up the correct statement from the following:

a. Quick lime contains calcium oxide

b. Slaked lime may be obtained from quick lime

c. Slaked lime is obtained by adding water to quick lime

d. All the above.

6. If P is the percentage of water required for normal consistency, water to be added for determination of initial setting time, is

a. 0.75 P

b. 0.80 P

c. 0.85 P

d. 0.90 P.

7. Mastic asphalt is generally used for

a. damp proof course

b. water proof layer

c. partition walls

d. both (a) and (b).

8. Pick up the correct statement from the following:

a. The lime stone resists fire upto about 800°C and at higher temperature it splits into CaO and CO2

b. The sand stone with silicates resist a fire in a better way

c. The argillaceous stone though poor in strength can resist fire quite weak

d. All the above.

9. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

10. The ratio of shear stress and shear strain of an elastic material, is

a. Shear Modulus

b. Young's Modulus

c. Modulus of Elasticity

d. both (a) and (b).

11. When two plates butt together and are riveted with two cover plates with two rows of rivets, the joint is known as

a. lap joint

b. butt join

c. single riveted single cover butt joint

d. double riveted double cover butt joint.

12. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

13. The flow in open channel is said to be critical if the Froude number is :

a. less than 1.0

b. equal to 1.0

c. greater than 1.0

d. None of these.

14. Equation of continuity of flow is based on the principle of conservation of

a. mass

b. momentum

c. force

d. none of these.

15. The centre of pressure of a vertical plane immersed in a liquid is at

a. centre of higher edge

b. centre of lower edge

c. centroid of the area

d. none of these.

16. The ratio of inertia force of a flowing fluid and the viscous force of the liquid is called :

a. Renold's number

b. Froude's number

c. Euler's number

d. Weber's number.

17. The angle of internal friction of clays, is usually

a. 0° to 5°

b. 5° to 20°

c. 20° to 30°

d. 30° to 45°

18. For a homogeneous earth dam 50 m high having 2 m free broad, a flow net was constructed and the results were : Number of potential drops = 2.4 Number of flow channels = 0.4. If coefficiency of permeability of the dam mateiral is 3 x 10-3 cm3/sec, the discharge per metre length of dam, is

a. 12 x 10-5 m3/sec

b. 24 x 10-3 m3/sec

c. 6 x 10-5 m3/sec

d. 24 x 10-5 m3/sec

19. A pycnometer is used to determine

a. voids ratio

b. dry density

c. water content

d. density index.

20. The intensity of active earth pressure at a depth of 10 metres in dry cohesionless sand with an angle of internal friction of 30° and with a weight of 1.8 t/m3, is

a. 5 t/m2

b. 6 t/m2

c. 7 t/m2

d. 8 t/m2

21. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

22. On piles, the drop must be at least

a. 100 cm

b. 120 cm

c. 140 cm

d. 150 cm

23. For a continuous floor slab supported on beams, the ratio of end span length and intermediate span length, is

a. 0.6

b. 0.7

c. 0.8

d. 0.9

24. The stresses developed in concrete and steel in reinforced concrete beam 25 cm width and 70 cm effective depth, are 62.5 kg/cm2 and 250 kg/cm2 respectively. If m = 15, the depth of its neutral axis is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

25. The angular steps used for changing direction of the stairs, are called

a. angular steps

b. winders

c. radial steps

d. circular steps.

26. Pick up the incorrect statement from the following :

a. a long vertical load transferring concrete structure is called a concrete pile

b. in pile which transfers the load to the soil by the friction between the pile and the surrounding soil is called friction pile.

c. the pile which transfers the load to a hard rock bed at certain depth is called load bearing

d. none of the these.

27. The 28 days cube strength of mass concrete using aggregates of maximum size 5 cm for gravity dams should be

a. between 350 to 600 kg/cm2

b. between 150 to 500 kg/cm2

c. below 200 kg/cm2

d. none of these.

28. Pick up the correct statement from the following:

a. Silica in excess, causes the cement to set slowly

b. Alumina in excess, reduces the strength of the cement

c. Magnesium oxide in excess, remains in free state and makes the cement unsound

d. All the above.

29. Pick up the correct statement from the following :

a. Rain water which percolates through the ground, is free from suspended materials

b. Under ground water may dissolve minerals and salts present in the earth's layers

c. Small quantities of iron, calcium and magnesium dissolved in water, may be useful for human health

d. All the above.

30. Efficiency of removing bacterias from raw water by a slow sand filter, is

a. 80% to 81%

b. 85% to 86%

c. 90% to 97%

d. 98% to 99%.

31. The discharge per unit plan area of a sedimentation tank, is generally called

a. over flow rate

b. surface loading

c. over flow velocity

d. all the above.

32. The suitable cross-section of sewers to carry combined flow, is

a. circular

b. egg shaped

c. rectangular

d. horse shoe shaped.

33. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

34. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

35. The field capacity of a soil is 25%, its permanent wilting point is 15% and specific dry unity weight is 1.5. If the depth of root zone of a crop, is 80 cm, the storage capacity of the soil, is

a. 10 cm

b. 12 cm

c. 14 cm

d. 16 cm

36. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

37. Increase in traffic volume, due to increase in transport vehicles, is known as

a. normal traffic growth

b. generated traffic growth

c. current traffic

d. none of these.

38. The minimum vertical clearance of overhanging cliffs or any other structure above the surface of a hill road, should be

a. 4 m

b. 5 m

c. 6 m

d. 4.5 m

39. For a properly designed vehicle, the resistance generally ignored, is

a. rolling resistance

b. grade resistance

c. axle resistance

d. none of these.

40. The width of the right of way, is influenced by

a. shoulders or berms

b. classification of road

c. availability of funds

d. all the above.

41. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

42. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

43. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

44. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

45. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

46. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

47. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

48. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

49. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.