***Sub Engineer***

1. If R is the radius of the main curve, θ the angle of deflection, S the shift and L the length of the transition curve, then, total tangent length of the curve, is

a. (R + S) tan θ/2 - L/2

b. (R + S) tan θ/2 + L/2

c. (R - S) tan θ/2 + L/2

d. (R - S) cos θ/2 + L/2

2. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

3.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

4. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

5. Pick up the correct statement from the following:

a. The edges of a finished wrought iron, are rough due to red shortage

b. Pig iron (charcoal) is manufactured from magnetic ore (Fe3O4)

c. For the manufacture of wrought iron, non-sulphurous fuel is not necessary

d. All the above.

6. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

7. In paints, the pigment is responsible for

a. colour

b. smoothness

c. glassy face

d. none of these.

8. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

9. Pick up the correct statement from the following:

a. Mcg = (M M2 + r2) where letters carry their usual meanings

b. Tcp = m2 + T2 where letters carry their usual meanings

c. The torque which when acting alone would produce maximum shear stress equal to the maximum shear stress caused by the combined bending and torsion, is called equivalent torque

d. All the above.

10. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

11. The principal stresses at a point are 100, 100 and-200 kgf/cm2, the octo hedral shear stress at the point is :

a. 100 2 kg/cm2

b. 200 2 kg/cm2

c. 300 2 kg/cm2

d. 400 2 kg/cm2

12. The ratio of the moment of inertia of a circular plate and that of a square plate for equal depth, is

a. equal to one

b. more than one

c. equal to 3π/16

d. none of these.

13. The equation  for the viscosity, is suggested

a. Newton

b. Chezy

c. Bezin

d. Helmholtz.

14. An error of 1% in measuring the head of water over the crest of a rectangular weir, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 1.75%

d. 2.25%

15. For a long pipe, the head loss

a. at the entrance is ignored

b. at the outlet is ignored

c. at the entrance and outlet both are ignored

d. due to friction is ignored.

16. A stepped notch is a combination of

a. rectangular notches of different sizes

b. triangular notches of different sizes

c. rectangular and triangular notches

d. all the above.

17. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

18. When a cohesionless soil attains quick condition, it looses

a. shear strength

b. bearing capacity

c. both (a) and (b)

d. neither (a) nor (6).

19. A direct shear test possesses the following disadvantage:

a. A relatively thin thickness of sample permits quick drainage

b. A relatively thin thickness of sample permits quick dissipation of pore pressure developed during the test

c. As the test progresses the area under shear, gradually changes

d. none of these.

20. If the coefficient of the active pressure Ka is 1/3, the coefficient of passive pressure Kp, is

a. 2/3

b. 1

c. 3/2

d. 3

21. The transverse reinforcements provided at right angles to the main reinforcement

a. distribute the load

b. resist the temperature stresses

c. resist the shrinkage stress

d. all the above.

22. The length of the straight portion of a bar beyond the end of the hook, should be at least

a. thrice the diameter

b. four times the diameter

c. five times the diameter

d. seven times the diameter.

23. Enlarged head of a supporting column of a flat slabs is technically .known as

a. top of the column

b. capital

c. drop panel

d. none of these.

24. If P kg/m2 is the upward pressure on the slab of a plain concrete footing whose projection on either side of the wall is a cm, the depth of foundation D is given by

a. D = 0.00775 aP

b. D = 0.0775 aP

c. D = 0.07775 aP

d. D = 0.775 Pa

25. The service area in a building means the area occupied by

a. stairs

b. toilets

c. light and shafts

d. all the above.

26. The triangular portion between any two adjacent arches and the tangent to their crowns, is

a. haunch

b. spandril

c. soffit

d. rise.

27. Proper proportioning of concrete, ensures

a. desired durability

b. water tightness of the structure

c. resistance to water

d. all the above.

28. For a concrete mix 1:3:6 and water cement ratio 0.6 both by weight, the quantity of water required per bag, is

a. 12 kg

b. 14 kg

c. 16 kg

d. 20 kg

29. Increase in population of a rapidly growing city, may be estimated by

a. arithmetical mean method

b. geometrical method

c. incremental increase method

d. graphical comparison method.

30. Pick up the incorrect statement from the following. The underground sources of water, is from

a. springs

b. infiltration wells

c. storage reservoirs

d. none of these.

31. The laying of sewers is done with

a. theodolite

b. level

c. clinometer

d. plane table.

32. Rate of flow of sewage is generally assumed

a. more than the rate of water supply

b. equal to the rate of water supply

c. less than the rate of water supply

d. at 150 litres per capita.

33. Disposal of extra excavated earth of canals, is utilised to provide a spoil bank on

a. left side

b. right side

c. both sides

d. all the above.

34. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

35. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

36. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

37. Any gradient on a road is said to be an exceptional gradient, if it is

a. more than ruling gradient

b. less than average gradient

c. more than floating gradient

d. less than minimum gradient or more than maximum gradient.

38. The maximum safe speed on roads, depends on the

a. type of road surface

b. type of curves

c. sight distance

d. all the above.

39. A district road with a bituminous pavement has a horizontal curve of 1000 m for a design speed of 75 km ph. The super-elevation is

a. 1 in 40

b. 1 in 50

c. 1 in 60

d. 1 in 70

40. If degree of a road curve is defined by assuming the standard length of an arc as 30 metres, the radius of 1° curve is equal

a. 1719 m

b. 1146 m

c. 1046 m

d. 1619 m

41. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

42. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

43. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

44. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

45. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

46. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

47. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

48. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

49. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.