***Sub Engineer***

1. Greater accuracy in linear measurements, is obtained by

a. tacheometry

b. direct chaining

c. direct taping

d. all the above.

2. Pick up the incorrect statement from the following :

a. while measuring a distance with a tape of length 100.005 m, the distance to be increasing by 0.005 m for each tape length

b. an increase in temperature causes a tape to increase in length and the measured distance is too large

c. the straight distance between end points of a suspended tape is reduced by an amount called the sag correction

d. a 100 m tape of cross section 10 mm x 0.25 mm stretches about 10 mm under 5 kg pull.

3. The 10 mm markings on a levelling staff placed at 20 m are separated by

a.  radian

b.  radian

c.  radian

d.  radian.

4. The chaining on sloping ground is

a. easier along the falling gradient

b. easier along the up gradient

c. equally convenient along falling as well as up gradient

d. all the above.

5. Pick up the correct statement from the following:

a. The phenol is either extracted from coal-tar or prepared from benzene

b. Phenol reacts with formaldehyde, to form phenol formaldehyde resin

c. The plastics prepared from phenol-formaldehyde are used for paints, varnishes, w.c. seats

d. All the above.

6. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

7. For lime concrete,

a. flexural strength at 90 days is 0.2 N/mm2

b. compressive strength at 90 days is 1.5 N/mm2

c. compressive strength at 26 days is 1.2 N/mm2

d. all the above.

8. Lime putty

a. is made by adding lime to water

b. can be used only upto three days

c. may be obtained from drying lime water mix passing through IS sieve No. 300

d. all of above.

9. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

10. For calculating the permissible stress  is the emprical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

11. Pick up the incorrect statement

a. A welded joint develops strength of its parent metal

b. Welded joints provide rigidity

c. Welded joints have better finish

d. Welding takes more time than riveting.

12. The range within which a load can be applied on a rectangular column, to avoid any tensile stress, is

a. one-third of the base

b. one-fourth of the base

c. one-fifth of the base

d. one sixth of the base on either side of centroid.

13. The momentum correction factor (β) for the viscous flow through a circular pipe is

a. 1.33

b. 1.50

c. 1.66

d. 2.00

14. To measure very low pressure, we use

a. barometers

b. piezometers

c. manometers

d. differential manometers.

15. The flow is called rotational if its velocity normal to the plane of area is equal to

a. angular velocity vector

b. twice the angular velocity vector

c. thrice the angular velocity vector

d. none of these.

16. For a most economical rectangular channel, the width of the channel must be

a. equal to depth of flow

b. twice the depth of flow

c. half the depth of flow

d. None of these.

17. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

18. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

19. The lateral earth pressure on a retaining wall

a. proportional to the depth of the soil

b. proportional to the square of the depth of the soil

c. proportional to the internal friction of the soil

d. none of these.

20. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

21. The diameter of main bars in R.C.C. columns, shall not be less than

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

22. An R.C.C. column of 30 cm diameter is reinforced with 6 bars 12 mm φ placed symmetrically along the circumference. If it carries a load of 40, 000 kg axially, the stress is

a. 49.9 kg/cm2

b. 100 kg/cm2

c. 250 kg/cm2

d. 175 kg/cm2

23. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

24. By over-reinforcing a beam, the moment of resistance can be increased not more than

a. 10%

b. 15%

c. 20%

d. 25%

25. The skirting/dado in a bath roof should be upto

a. ceiling

b. 15 cm above floor level

c. 200 cm

d. level of the tap.

26. Pick up the incorrect statement from the following:

a. The section of the retaining wall should be so proportioned that it may not overturn by the lateral pressure

b. The retaining wall should be safe against sliding

c. The foundation of the retaining wall should not be stressed beyond safe bearing capacity ; due to its weight and the force resulting from the earth pressure

d. to drain off water from the earth retained, weep holes are provided near the top of the retaining wall.

27. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

28. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

29. In a reservoir the average volume of sediment deposition is 0.15 million cubic per year. If the dead storage and total capacity of the reservoir are 8 million cubic metres and 36 million cubic metres respectively,

a. the reservoir will theoretically be silted in 240 years

b. the reservoir will start reducing after 60 years

c. both (a) and (b)

d. neither (a) nor (b).

30. Distribution of wash water is provided in

a. sedimentation tank

b. slow sand filter

c. rapid gravity filter

d. all the above.

31. The first stage of neutral process of sludge digestion, is

a. acid fermentation

b. acid regression

c. alkaline fermentation

d. none of these.

32. During preliminary treatment of a sewage

a. Oil and grease are removed from skimming tanks

b. Floating materials are removed by screening

c. Girt and sand are removed by girt chambers

d. All the above.

33. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

34. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

35. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

36. According to G.W. Pickles the effect of confining the flood water of a river between levee, is to increase

a. water surface elevation during floods

b. maximum discharge

c. velocity and scouring action

d. surface slope of streams above the leveed portion.

37. In a right angle bend of a road provided with a transition throughout, the maximum polar angle will be

a. 15°

b. 20°

c. 30°

d. 45°

38. If the number of lanes on the carriageway of a road is more than two, the total width of lane ways is equal to 3.0 m

a. + 0.70 m

b. + 0.80 xn

c. + 0.90 m

d. + 1.50 m

39. Camber in pavements is provided by

a. straight line method

b. parabola method

c. straight at the edges and parabolic at the crown

d. all the above.

40. If cross slope of a country is 10% to 25%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

41. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

42. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

43. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

44. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

45. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

46. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

47. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

48. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

49. The length of runway is increased per 300 m rise above M.S.L.

a. 4%

b. 5%

c. 6%

d. 7%

50. For Class A Air port the difference of reduced levels of higher and lower edges of the conical surface, is

a. 25 m

b. 50 m

c. 75 m

d. 100 m