***Sub Engineer***

1. The slope correction for a 3° slope for a length of 100 m, is

a. -0.11 m

b. -0.12 m

c. -1.87 m

d. -0.137 m

2. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

3. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

4. An ideal vertical curve to join two gradients, is

a. parabolic

b. elliptical

c. hyperbolic

d. none of these.

5. Pick up the correct statement from the following:

a. Strength and stiffness of corrugated sheets are considerably increased

b. Corrugated sheets are generally gal-venised to protect iron from corrosion by rust

c. Corrugated sheets are generally used on slanting roofs

d. all the above.

6. The commonly used raw material in the manufacture of cement, is

a. slate

b. sand stone

c. lime stone

d. basalt.

7. Asbestos cement

a. is brittle

b. warps due to changes in humidity

c. strength is lowered when saturated by water

d. all the above.

8. A 1st class brick immersed in water for 24 hours, should not absorb water (by weight) more than

a. 15%

b. 20%

c. 25%

d. 5%.

9. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

10. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

11. A member which is subjected to reversible tensile or compressive stress may fail at a stress lower than the ultimate stress of the material. This property of metal, is called

a. plasticity of the metal

b. elasticity of the metal

c. fatigue of the metal

d. workability of the metal.

12. A three hinged arch is loaded with an isolated load 1000 kg at a horizontal distance of 2.5 m from the crown, 1 m above the level of hinges at the supports 10 metres apart. The horizontal thrust is

a. 1250 kg

b. 125 kg

c. 750 kg

d. 2500 kg

13. Fluids change the volume under external presssure due to

a. plasticity

b. viscosity

c. compressibility

d. none of these.

14. Pick up the correct statement from the following :

a. Dimensional homogeneity means the dimensions of each term in an equation on both sides are equal

b. Dimensionally homogeneous equations are independent of the system of units

c. In dimensionally homogeneous equation, the powers of fundamental dimensions on either side of the equation are identical

d. All the above.

15. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

16. Poise is the unit of

a. viscosity

b. velocity gradient

c. mass density

d. kinematic viscous.

17. A soil sample has passing 0.075 mm sieve = 60% liquid limit = 65% and plastic limit = 40%. The group index of the soil, is

a. 5

b. 20

c. 40

d. none of these.

18. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

19. The coefficient of compressibility of soil, is the ratio of

a. stress to strain

b. strain to stress

c. stress to settlement

d. rate of loading to that of settlement.

20. A coarse-grained soil has a voids ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

21. The width of the flange of a T-beam, which may be considered to act effectively with the rib depends upon

a. overall thickness of the rib

b. centre to centre distance between T-beams

c. span of the T-beam

d. all the above.

22. The weight of a foundation is assumed as

a. 5% of wall weight

b. 7% of wall weight

c. 10% of wall weight

d. 12% of wall weight

23. If H is the overall height of a retaining wall retaining a surcharge, the width of the base slab usually provided, is

a. 0.4 H

b. 0.5 H

c. 0.6 H

d. 0.7 H

24. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

25. The brick laid with its length parallel to the face of a wall, is a known as

a. header

b. stretcher

c. closer

d. none of these.

26. Slate

a. is a metamorphic rock

b. splits into thin sheets along its bedding planes

c. has a smooth surface and contains alumina and silica

d. possesses good water absorption capacity

27. The shuttering of a hall measuring 4 m x 5 m, can be removed after

a. 7 days

b. 10 days

c. 14 days

d. 21 days

28. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

29. Service connections to consumers houses, are generally provided with

a. hume pipes

b. galvanised iron pipes

c. P.V.C. pipes

d. none of these.

30. The least thickness of class B cast iron (spun) pipe, is

a. 7.2 mm

b. 7.9 mm

c. 8.6 mm

d. 10 mm.

31. In normal conditions, the period for sludge digestion, is

a. 20 days

b. 30 days

c. 60 days

d. 90 days.

32. If γw is the unit weight of water, r the hydraulic mean depth of the sewer and S the bed slope of the sewer, then the tractive force exerted by flowing water, is

a. γw . r . S

b. γw r1/2 . S

c. γw r S

d. γw r2/3 S

33. The most suitable location of a canal head work, is

a. boulders stage of the river

b. delta stage of the river

c. rock stage of the river

d. trough stage of the river.

34. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

35. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

36. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

37. To prevent movement of moisture from subgrade to road pavement at the same level as that of water-table, thickness of a cut off layer of coarse sand, is

a. 15 cm

b. 20 cm

c. 30 cm

d. 45 cm

38. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

39. The length of the side of warning sign boards of roads is

a. 30 cm

b. 40 cm

c. 45 cm

d. 50 cm

40. For a properly designed vehicle, the resistance generally ignored, is

a. rolling resistance

b. grade resistance

c. axle resistance

d. none of these.

41. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

42. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

43. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

44. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

45. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

46. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

47. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

48. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. For Class A Air port the difference of reduced levels of higher and lower edges of the conical surface, is

a. 25 m

b. 50 m

c. 75 m

d. 100 m

50. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.