***Sub Engineer***

1. The slope correction may be ignored if

a. the slope of the ground is less than 3°

b. the slope of the ground is say 1 in 19

c. both (a) and (b)

d. neither (a) nor (b)

2. In case of reduction of levels by the height of instrument method,

a. ∑ B.S. - ∑ F.S. = difference in R.L.S of the first station and last station

b. ∑ (R.L. + I + F.S.) - first R.L = ∑ (H.I. + No. of R.L.s.)

c. both (a) and (b) above

d. neither (a) nor (b).

3. For orientation of a plane table with three points A, B and C, Bessel's drill is

a. Align b through a and draw a ray towards c, align a through b and draw a ray towards c, finally align c through the point of intersection of the previously drawn rays

b. Align c through a and draw a ray towards b, align a through c and draw a ray towards b, finally align b through the point of intersection of the previously drawn rays

c. Align c through b and draw a ray towards a, align b through c and draw a ray towards a, finally align a, through the point of intersection of the previously, drawn rays

d. In the first two steps any two of the points may be used and a ray drawn towards the third point, which is sighted through the point of intersection of previously drawn rays in the final step.

4. The correction to be applied to each 30 metre chain length along θ° slope, is

a. 30 (sec  θ - 1) m

b. 30 (sin  θ - 1) m

c. 30 (cos  θ - 1) m

d. 30 (tan  θ - 1) m

5. Pick up the correct statement from the following:

a. The average crushing strength of hand moulded bricks is 6000 t/m2

b. The average tensile strength of hand moulded brick is 200 t/m2

c. The average shearing strength of hand moulded brick is 600 t/m2

d. All the above.

6. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

7. Pick up the correct statement from the following:

a. In basic Bessemer process, the steel heats the converter

b. In open-hearth process, the furnace heats the steel

c. In Siemens process, the impurities of pig iron are oxidised by the oxygen of the ore

d. all the above.

8. Pig iron is manufactured from the ores by

a. dressing

b. calcination and roasting

c. smelting

d. all the above.

9.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

10. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory

11. Strain energy of a member may be equated to

a. average resistance x displacement

b. ½ x stress x strain x area of its cross-section

c.  ½ x stress x strain x volume of the member

d. ½ x (stress)2 x volume of the member + Young's modulus E.

12. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

13. The value of kinetic energy correction factor (a) for a laminar flow through a circular pipe, is

a. 1.0

b. l.5

c. 2.0

d. 2.5

14. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

15. An error of 1% in measuring the head of water over the crest of a triangular notch, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 2.0%.

d. 2.5%

16. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

17. For a homogeneous earth dam 50 m high having 2 m free broad, a flow net was constructed and the results were : Number of potential drops = 2.4 Number of flow channels = 0.4. If coefficiency of permeability of the dam mateiral is 3 x 10-3 cm3/sec, the discharge per metre length of dam, is

a. 12 x 10-5 m3/sec

b. 24 x 10-3 m3/sec

c. 6 x 10-5 m3/sec

d. 24 x 10-5 m3/sec

18. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

19. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

20. 'Loess' is silty clay formed by the action of

a. water

b. glacier

c. wind

d. gravitational force.

21. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

22. The horizontal portion of a step in a stairs case, is known as

a. rise

b. flight

c. winder

d. tread.

23. If the maximum shear stress at the end of a simply supported R.C.C. beam of 6 m effective span is 10 kg/cm2, the share stirrups are provided for a distance x from either end where x is

a. 50 cm

b. 100 cm

c. 150 cm

d. 200 cm

24. According to I.S. : 456, slabs which span in two directions with corners held down, are assumed to be divided in each direction into middle strips and edge strips such that the width of the middle strip, is

a. two-third of the width of the slab

b. three-fourth of the width of the slab

c. four-fifth of the width of the slab

d. three-fifth of the width of the slab.

25. The vertical side member of a shutter frame, is known

a. style

b. reveal

c. mullion

d. post.

26. A wooden block fixed on back side of a door frame on its post, is known as

a. cleat

b. stop

c. horn

d. none of these.

27. An ideal ware house, is provided

a. water proof roof

b. few windows which remain generally closed

c. 15 cm thick concrete floor laid on a dry course of soling

d. all the above.

28. Pick up the incorrect statement from the following:

a. The axis of the mould is perpendicular to the end faces

b. The internal surface of the mould is kept clean and free from set cement

c. The mould is in the form of a frustum of hexagonal pyramid

d. None of these.

29. If Q is discharge is cubicmetres per sec and D is the economical diameter of the pipe. According to Lea

a. D = 0.67 to 0.87 Q

b. D = 0.77 to 0.97 Q

c. D = 0.97 to 1.22 Q

d. D = 1.22 to 1.33 Q.

30. Pick up the incorrect statement from the following :

a. Higher value of pH means lower hydrogen ion concentration

b. Lower value of pH means higher hydrogen ion contraction

c. Lower value of pH gives alkaline solution

d. None of these.

31. The pH value of fresh sewage is usually

a. equal to 7

b. more than 7

c. less than 7

d. equal to zero.

32. The sewer pipes

a. carry sewage as gravity conduits

b. are designed for generating self-cleansing velocities at different discharge

c. should resist the wear and tear caused due to abrasion

d. all the above.

33. If water table is comparatively high, the irrigation canal becomes useless, due to

a. water logging of the cultivated areas

b. uncertain water demand

c. spread of malaria

d. all the above.

34. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

35. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

36. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

37. The full width of land acquired before finalising a highway, alignment is known

a. width of formation

b. right of way

c. carriage way

d. roadway.

38. For a poorly graded sub-grade soil, thickness of sub-base, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

39. At intersection of roads, the traffic volume study is carried out to ascertain the number of vehicles

a. moving along straights

b. turning left

c. turning right

d. all the above.

40. The minimum vertical clearance of overhanging cliffs or any other structure above the surface of a hill road, should be

a. 4 m

b. 5 m

c. 6 m

d. 4.5 m

41. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

42. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

43. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

44. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

45. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

46. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

47. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

48. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

49. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.

50. A gradient of + 0.08% is followed by a gradient of - 0.07%. If the permissible rate of change of grade is 0.003 per 30 metres, the length of the transition curve, is

a. 150 m

b. 140 m

c. 160 m

d. 175 m