***Sub Engineer***

1. The representation of general topography of a very flat terrain is possible only

a. by drawing contours at large interval

b. by drawing contours at small interval

c. by giving spot levels at large interval

d. by giving spot levels to salient features at close interval.

2. Short offsets are measured with

a. an ordinary chain

b. an invar tape

c. a metallic tape

d. a steel tape.

3. The apparent error on reversal is

a. equal to the actual error

b. twice the actual error

c. thrice the actual error

d. none of these.

4. When the bubble of the level tube of a level, remains central

a. line of sight is horizontal

b. axis of the telescope is horizontal

c. line of collimation is horizontal

d. geometrical axis of the telescope is horizontal.

5. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

6. In a rock calcium carbonate predominates. State whether it is :

a. Sillicious rock

b. Argillaceous rock

c. Calcareous rock

d. None of these.

7. French polish is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

8. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

9. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

10. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

11. A three hinged parabolic arch hinged at the crown and springings, has a horizontal span of 4.8 m and a central rise of 1 m. It carries a uniformly distributed load of 0.75 tonne per metre over half left hand span. The horizontal thrust at the support will be

a. 1.08 tonnes

b. 1.8 tonnes

c. 0.8 tonnes

d. none of these.

12. The areas of cross-section of a square beam and a circular beam subjected to equal bending moments, are same.

a. square beam is more economical

b. both the beams are equally strong

c. both the beams are equally economical

d. none of these.

13. In an inclined position, a venturimeter records

a. more reading

b. less reading

c. same reading

d. none of these.

14. Hydraulic ram is a device

a. for lifting water without an electric motor

b. for accelerating water flow

c. for lifting heavy loads

d. none of these.

15. In steady flow, which one of the following changes with time

a. velocity

b. pressure

c. density

d. none of these.

16. A pitot tube is used to measure

a. pressure

b. difference in pressure

c. velocity of flow

d. none of these.

17. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

18. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

19. The neutral stress in a soil mass is

a. force per neutral area

b. force per effective area

c. stess taken up by the pore water

d. stress taken up by solid particles.

20. The method of the slices is applicable to

a. homogenous soils

b. stratified soils

c. saturated soils

d. non-uniform slopes

21. In a slab, the pitch of the main reinforcement should not exceed its effective depth

a. three times

b. four times

c. five times

d. two times.

22. For M 150 grade concrete (1:2:4) the moment of resistance factor is

a. 8.50

b. 7.50

c. 5.80

d. none of these.

23. A flat slab is supported

a. on columns

b. on beams and columns

c. on columns monolithicaily built with slab

d. all the above

24. If the width of the foundation for two equal columns is restricted, the shape of the footing generally adopted, is

a. square

b. rectangular

c. trapezoidal

d. triangular.

25. Pick up the correct statement from the following :

a. the sand is kept moist at the time of placing and tamping

b. the top one metre of the pile is filled up with cement concrete to provide a cap for the filled up sand

c. sand piles are generally used under column loads

d. all of the above.

26. Pick up the incorrect statement from the following:

a. In dog-legged stairs, no space between its flights is provided

b. In open newel stair, a rectangular well is provided

c. In geometric stair, a curved shaped well between forward and backward flights, is provided

d. In geometrical stair, two quarter space landing is provided.

27. Joints in concrete structures, are provided

a. to reduce the tensile stresses likely to be developed due to evaporation of water

b. to minimise the change in the dimensions of the slab

c. to minimise the necessary cracking

d. all the above.

28. The final operation of finishing floors, is known as

a. floating

b. trowelling

c. finishing

d. all the above.

29. By adding 1 ml of orthotolidine solution to 100 ml chlorinated water taken after contact period, the residual chlorine makes the colour of solution

a. yellowish

b. greenish

c. bluish

d. reddish

30. An area is declared drought affected if its mean rainfall is less than

a. 60%

b. 75%

c. 80%

d. 85%.

31. The detention period for plain sedimentation water tanks, is usually

a. 4 to 8 hours

b. 8 to 16 hours

c. 16 to 24 hours

d. 24 to 36 hours.

32. The self-cleansing velocity of water flowing through pipe lines, is

a. 2 metres/sec

b. 1 metre/sec

c. 0.5 metre/sec

d. 0.25 metre/sec.

33. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

34. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

35. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

36. In a Sarda type fall, the rectangular crest, may be used for discharge upto

a. 10 cumecs

b. 14 cumecs

c. 20 cumecs

d. 25 cumecs.

37. Width of the shoulders of carriage way is generally kept

a. 125 cm

b. 150 cm

c. 200 cm

d. 250 cm

38. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

39. The traffic manoeuvre means

a. diverging

b. merging

c. crossing

d. all the above.

40. Pick up the correct statement from the following:

a. Safety fences are provided on either side of a roadway if embankments are in excess of 6 metres

b. Safety fences are provided on outside of the curves of radii less than 750 m if the embankments are between 3 metres and 6 metres

c. Guard stones are provided at 2.5 metres intervals if embankments are between 1.6 metres to 3 metres

d. All the above.

41. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

42. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

43. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

44. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

45. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

46. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

47. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

48. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

49. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.

50. The threshold markings are

a. 1 m clear space between adjacents

b. 45 m in length

c. placed symmetrically on either side of the runway centre line

d. all the above.